# # $ K + CPU Window

The CPU Window displays registers and allows registers and signals to be modified.

Screen Description

The registers are shown with their values.  Double click on the value field to edit.

Menu Selections

Options

Reset	Reset the processorresettingtheprocessor.

Reset CPU Only	Reset the processor.  Do not update windows.

Signals	Enable or disable signalsenablinganddisablingsignals.

Windows	Move between windowsmovingbetweenwindows

Help Index	MICEpack/SLD table of contents.

Help with Help	Using the help system.

Help with CPU	CPU Window table of contents.

Exit	Close the CPU Window

Using the CPU Window

Warning on Changing CPU Registerswarningonchanging

Search Keywords

shell, cpu

# # $ K + Warning on Changing CPU Registers

If the CS:IP or the SS:SP is changed, the source and stack context may be invalid.

The CS:IP is changed by the Source Window'ssource Run/Go From Cursor, the CPU Windowcpu, and the register Shell commandregistershell.

The SS:SP and status register are changed from the CPU Windowcpu or by the register Shell commandregistershell.

See also

Resetting the Processorresettingtheprocessor

# $ # + K Enabling and Disabling Signals

Enable a signal to connect the signal from the emulator probe head to the user's target.  A check mark indicates an enabled signal.

Disable a signal to connect the signal to a source in the probe head and disconnect it from the target.

Use the CPU Window'scpu Options/Signals or the signal Shell commandsignalshell to enable and disable signals.

When the emulator starts up, it tries to communicate with the target processor.  If the target processor has no clock, the emulator switches the clock source to the internal clock in the probe head and tries again to communicate with the target processor.

# $ # + K Resetting the Processor

## To reset the processor, choose one of the following:

-	The CPU window'scpu Options/Reset menu option.

-	The Source window'ssource Go/Reset and Go/Reset And Go menu option.

-	The Toolbar'stoolbar Configure/Reset menu option.

-	The reset Shell commandresetshell.

## A reset causes the following actions:

-	The processor reset pin is asserted.

-	The CS:IP and SS:SP values are read from memory.

-	All open windows are updated.

## The consequences of a reset are:

-	The stack is set to an undefined state so the Stack Window contents are invalid.

-	The Source windowsource points at the start of code which may be startup code without an associated source file.

## If the reset command fails:

-	Reset the processor only by one of the following:

-	Choose the CPU window's Options menu/Reset CPU Only command.

OR

-	Choose the Toolbar window's Configure menu/Reset CPU Only command.

OR

-	Enter the resetShell cpuonly Shell command.

-	Initialize the CS:IP and the SS:SP to valid values using the CPU Windowcpu or the register Shell commandregistershell.

-	Reset the processor and update the windows by choosing one of the following:

-	CPU window'scpu Options/Reset.

OR

-	Source window's Go/Reset.

OR

-	Toolbar's Configure/Reset.

OR

-	Enter the reset Shell commandresetshell.

# # $ K Register Shell Command

Display and change CPU registers.

The CPU Windowcpu is equivalent to this command.

Syntax

register [<register name> [value]]...

Parameters

register name	The name of a register.

value	The new value for the register.  Be careful changing certain registerswarningonchanging.

Examples

Display all registers.

> register

See also

Shell Syntax Conventionsshellsyntaxconventions

# # $ K Reset Shell Command

Reset the processorresettingtheprocessor.

The CPU Window'scpu Options/Reset is equivalent to this command.

Syntax

reset [cpuonly]

Parameters

cpuonly	Reset the processor and do not update windows.  Normally, all windows are updated on a reset.  If the CS:IP or the SS:SP memory contents are invalid at reset, the processor can hang again while updating these windows.  Use cpuonly in this case and then, set valid CS:IP and SS:SP values.  Issue reset again to update windows.

Examples

Reset the processor and update the windows.

reset

See also

Resetting the Processorresettingtheprocessor

Shell Syntax Conventionsshellsyntaxconventions

# # $ K Signal Shell Command

Enable and disable signalsenablinganddisablingsignals

The CPU Window'scpu Options/Signals is equivalent to this command.

Syntax

signal [[ <signal name> [enable | disable]] | [all enable | all disable]]

Parameters

all enable	Enable all signals.

all disable	Disable all signals.

signal name	The signal name from the following list:
Intel 80C186/188 XL/EA:  INT0, INT1, INT2, INT3, NMI, DRQ0, DRQ1, RESET, HOLD, PEREQ, ERROR.
                        
Intel 80C186/188 EB:  INT0, INT1, INT2, INT3, INT4, NMI, RESET, HOLD, PEREQ, ERROR.

Intel 80C186/188 EC:  INT, NMI, DRQ, RESET, HOLD, PEREQ, ERROR.

enable	Enable the signal.

disable	Disable the signal.  The processor does not see the signal.

Examples

Display current signal values.

> signal

// CLK  DISABLE

// RESET  ENABLE

Disable the reset line and display the new value.

> signal reset disable; signal reset

// RESET  DISABLE

// RESET  DISABLE

See also

Shell Syntax Conventionsshellsyntaxconventions

# cpu

$ CPU Window

K cpu;registers;signal;reset;

+ POWERVIEWS:0040

# warningonchanging

$ Warning on Changing CPU Registers

K cpu;register

+ USINGTHECPU:0010

$ Enabling and Disabling Signals

# enablinganddisablingsignals

+ usingthecpu:0020

K signal;enabling signals;disabling signals

$ Resetting the Processor

# resettingtheprocessor

+ usingthesource:0120

K reset;chip;initialization;cpu

# registershell

$ Register Shell Command

K cpu;registers;shell, cpu;

# resetshell

$ Reset Shell Command

K cpu;reset;shell, cpu;

# signalshell

$ Signal Shell Command

K cpu;signal;reset;clk;shell, cpu;