# # $ K ToolBar Window

The ToolBar contains buttons to access the most commonly used windows and commands.

Button Bar

Click on one of the buttons to access the feature:

Map	Set up memory mapmappingmemory.

Load	Load code and symbols.

Trigger	Set up trigger.trigger.

Source	Show source codesource.

Stack	Display program stackstack.

CPU	Show CPU registers.cpu.

Mem	Display memory regionmemory.

Periph	Show Peripheral Registersperiph.

Go	Start emulationemulationterm.

Halt	Stop emulation.

Show	Show trace collectedtrace.

Shell	Command lineshell.

Menu Selections

File

Exit MICEpack/SLD	Exit MICEpack/SLD applicationexitingpowerviews.

Configure

Map	Set up memory mapmappingmemory.

Run Access	Enable or disable run accessrunaccessterm.

Symbolic Disassembly	Enable or disable symbols in disassembly, which is displayed in the Sourcesource and Memorymemory windows.

Reset	Resetresettool and reinitialize the target processor.

Reset CPU Only	Reset only the processorresetcpuonly and does not update the windows.

Layout

Save Settings Now	Save the current coordinates for the MICEpack/SLD windows and icons, to be restored the next time you invoke the tool.

Save Settings on Exit	Save the window and icon coordinates upon exiting MICEpack/SLD.

Windows	Move between windowsmovingbetweenwindows.

Help	On-line helponlinehelp

Using the ToolBar Window

Reading and Writing Memory while Emulating using Run Accessrunaccessterm

Minimizing MICEpack/SLD from the ToolBarminimizingpowerviewsfrom

# # $ K # K Minimizing MICEpack/SLD from the ToolBar

Minimizing the ToolBar window minimizes all of the MICEpack/SLD windows and icons into a single icon.  Maximizing the ToolBar icon will restore the ToolBar and access to the windows and other icons.

See also

Moving Between Windowsmovingbetweenwindows

# $ # K Moving Between Windows

To move the cursor to a MICEpack/SLD window:

-	Click on the window

-or-

-	Choose it from the Windows menu

-or-

-	Press F6.

To move to a different application's Window:

-	Click on it or press Alt-Tab.

# $ # K RAM

Random access memory (RAM) is memory that is readable and writable.

# $ # K ROM

Read only memory (ROM) is readable, but not writable memory.

# $ # K Mapping Memory

You can map memory to overlay RAMramterm to replace bad or non-existent target memory (RAM or ROMromterm).  An access to target memory is intercepted by overlay memory.  Signals are active in the target, but data is read or written to the overlay memory.

-OR-

You can map memory to the target.

The Copy Shell commandcopyshell or the Memory Window's Edit/Copy... menu item moves memory between target and overlay areas.

Map features are:

-	Options for 512 KB or 1 MB of RAM overlay.

-	Set region on any 2 KB boundary.

-	You can set memory type as:

-	Overlay or Target

-	You can set access rights as one of the following:

-	RAM allows read and write accesses.

-	Prevent writes to memory (ROM) and break emulation after a write is detected.

-	NONE breaks emulation after a read or write to memory is detected.

-	ROM or NONE, does not write to overlay memory.  The memory can not be corrupted.  However, MICEpack cannot prevent writes to target memory and a write does corrupt memory.

-	OFF memory is mapped to the target.

To map memory:

-	Use the map dialog boxmapdialogbox by clicking on the Map buttonmappingmemory in the ToolBar.

-	Use the map Shell commandmapshell.

See also

Memory Copy Shell Commandcopyshell

# $ # K Map Dialog Box

The Map dialog box displays the currently mapped regions of memory:

Start Addr	The start address of the region, always on a block size boundary (2k)

End Addr	The end address of the region

Length	The length of the region, in bytes

Type	Overlay or Target

Access Rights	Allow any access (RAM), prevent writes (ROM) with break, or break if any access (NONE), or memory is mapped to the target (OFF)

The following buttons are available at the bottom of the Map box:

Add	Map a new region.

Edit	Change the mapping of the region highlighted in the displayed list.  (Available when at least one region is mapped.)

Delete	Delete the mapping of the region highlighted in the displayed list.  (Available when at least one region is mapped.)

Save	Save the current mapping scheme to a file.

Restore	Retrieve a mapping scheme from a previously saved file.

Close	Close the Map dialog box.

Help	Open a MICEpack/SLD Help window.

To add or edit a mapped region, click on the Add or Edit button and fill-in the dialog box.

See also

Mapping Memorymappingmemory

Map Shell Commandmapshell

# # $ K Map Shell Command

Set memory map for target and overlay memory.  You can also choose the ToolBar Mapmapdialogbox button.

Syntax

map [clear | <address base> [<end address>] [target] [ram | rombrk | none | off]]

Parameters

clear	sets all memory to target by disabling all map blocks to overlay.

address base	address to start an overlay memory range.  The address is rounded down to the nearest boundary block equal to amount of memory mapped (2k).

end address	if no end address is specified, 2k will be mapped.  The end address is rounded up to the top of the 2k region containing the end address.

target	map memory range to the target.

ram	allow reads and writes to region (default).  This allows unrestricted access to the region.

rombrk	allow reads, prevent writes, and break emulation if a write to the region is detected.

none	disallow reads, prevent writes, and break emulation if any access to the region is detected.

off	Set the block of memory specified by the start address to target.

Examples

Clear existing map.

>map clear

// All memory mapped to target

Map 4k region starting at address 0x1000 (rounded down).

>map 0x10010

// Mapped block starting at address 0x010000 0x010fff RAM

Display current map.

>map

// Mapped block starting at address 0x010000 0x010fff RAM

Map the 128k block to target ROM area and break the processor when writing to this area.

>map 0xfe0000 0xffffff target rombrk

// Map block starting at address 0x10000 0x010fff RAM

// Map block starting at address 0xfe000 0xffefff ROM

See also

Mapping Memorymappingmemory

Map Dialog Boxmapdialogbox

Shell Syntax Conventionsshellsyntaxconventions

# $ # K On-line Help

To access on-line help:

-	Press F1.  This gives context sensitive help for the current window or dialog box.

-	Choose the Help menu item and pick a topic of interest.

# $ # K Symbol Entry Syntax

Symbols are specified as:

[#<module name>] [#<function name>] [#<variable name>] [#<line number>]

A fully-qualified symbol contains the module name, function name, and variable name.  Line number specifies a line number.

If you do not specify a fully-qualified name MICEpack/SLD attempts to:

-	match the symbol in the current function

-	match the symbol in the current module

-	match the symbol to a global

Many symbol specifications are valid, including:

#<line number>

#<module name>

#<function name>

#<variable name>

#<module name>#<function name>

#<module name>#<variable name>

#<function name>#<variable name>

#<module name>#<function name>#<variable name>

# # $ K Version Shell Command

Display firmware and software version information.

Syntax

version

Parameters

none

Examples

Display version information.

>version

See also

Reporting Problems to Microtekreportingproblemsto

# $ # K Swapfile

A portion of the hard disk is set aside to swap memory images when more memory is needed than actually exists.  To find the amount of swapfile space available:

-	Choose Program Manager's Main program group.

-	Choose the Control Panel.

-	Choose 386 Enhanced.

-	Choose Virtual Memory... button.

There are three settings:

-	Drive points to a hard disk partition (e.g. C:).

-	Size is the number of bytes in the swapfile (e.g. 6,000 KB is 6 Megabytes).

-	Type is permanent or temporary (e.g., type is Permanent if using BIOS).

MICEpack/SLD requires a minimum of 4 MB of permanent swapfile.

# $ # + K Exiting MICEpack/SLD

The ToolBar Window'stoolbar File/Exit closes all windows and terminates the MICEpack/SLD application.

Each individual window has an exit menu option.  This exit closes the window, but does not exit MICEpack/SLD.

# # $ K Peripheral Window

The Peripheral window enables the viewing and modifying of internal peripheral registers.

Screen Description

View the internal registers in compressed or expanded format.  The columns are (from left to right):

-	(-) or (+) indicates expanded or compressed, respectively.  Click to change format.

-	Register address.  Double-click anywhere on the line to edit.

-	Register value.

-	Register mnemonic.

-	Register Description.

Menu Selections

File

Exit	Close the Peripheral Window.

Edit

Registereditperiph...	Edit a peripheral register.

Go To Peripheralperiphgotoperiphdialog...	Scroll to a specified peripheral.

Go To Addressperiphgotoaddressdialog...	Scroll to a specified peripheral address.

Go To Registerperiphgotoregisterdialog...	Scroll to a specified register.

View

Expand All	Expandexformperiph the register display.

Compress All	Compresscomformperiph the register display.

Refresh Display	Refreshupdateperiph the register display.

Windows	Move between windowsmovingbetweenwindows.

Help	On-line helponlinehelp.

Using the Peripheral Window

Editing a Peripheral Registereditperiph

Scrolling through the Peripheral Windowscrollperiph

Search Keywords

shell, peripheral

# # $ K Edit Peripheral Register Dialog Box

Depending on the peripheral you are editing, MICEpack/SLD will display one of the following dialog boxes:

-	Edit Peripheral Register Dialog Boxeditperiphregdialog (for most peripherals).

-	Edit Peripheral Data Register Dialog Boxeditperiphregbufferdialog

# # $ K Expanded Format Peripheral Window

Shows the individual bit fields of a peripheral register, as well as the information shown in compressed formatcomformperiph.

# # $ K Compressed Format Peripheral Window

In this format, the columns in the Peripheral windowperiph are (from left to right):

-	Register address.

-	Register mnemonic

-	Register value (in red).

-	Register description.

# # $ K Update Peripheral Registers During Emulation

You can manually update the Peripheralperiph window if you suspect that the register values have changed but that change is not reflected in the window.

1.	If you want to be able to manually update memory during emulation then, from the Toolbartoolbar, choose Options/Enable Run Access before starting emulation.

2.	Choose View/Refresh Display.

# # $ K Scrolling the Peripheral Window

## If the Emulator is Halted:

-	Scrolling or paging updates the registers.

## To Scroll During Emulation:

1.	Toggle on Options/Run Accessrunaccessterm from the Toolbar before emulating.

2.	Scrolling or paging updates the registers.

# # $ K Receive and Transmit Buffers

Receive and transmit buffers are displayed in the Peripheralperiph window in their entirety.  Buffers are displayed in byte format.

-	Double click on a buffer line to display an edit dialog box that allows you to edit specific bytes in hex.

# # $ K Reset

Choose Configure/Reset from the Toolbartoolbar to do the following:

-	The processor reset pin is asserted.

-	The CS:IP and stack pointer values are read from memory.

-	All windows are updated with the following results:

-	The Stack display is invalid because the stack is undefined.

-	The Source window points to the start of code.

See also

Resetting the Processorresettingtheprocessor

Reset CPU onlyresetcpuonly

# # $ K Reset CPU only

Resets only the processor and does not update the windows.  This command should be used only if the Reset command fails to reset the processor.

See also

Resetting the Processorresettingtheprocessor

Resetresettool

# # $ K Pregister Shell Command

Set, get, and dump peripheral register values.

Syntax

pregister [<register name>[[<byte#>]]] [-V | <values>]

Parameters

If no <register name> is entered, all peripheral registers are displayed.

If <register name> is specified, that register value is displayed.

[<byte#>]	Specifies a byte to access for writing to a buffer based on zero offset.

-V	Displays peripheral registers in expandedexformperiph (verbose) format.

If both a <register name> and a valid numeric <value> are specified, the value is written to the specified register.

Register names conform to the Intel mnemonics.

Examples

Display CS:IP.

>pregister CS:IP

See also

Shell Syntax Conventionsshellsyntaxconventions

# # $ K Peripheral Go To Address Dialog Box

The window displays:

-	The peripheral at a valid hexadecimal address (prefixed by 0x).

To reposition the Peripheralperiph window display:

-	Enter an address.

See also

Go To Peripheral Dialog Boxperiphgotoperiphdialog

Go To Register Dialog Boxperiphgotoregisterdialog

# # $ K Go To Peripheral Dialog Box

1.	Click the down-arrow on the drop-down box to display the list of peripheral groups.

2.	Enter a peripheral group for repositioning the Peripheralperiph window display.

See also

Peripheral Go To Address Dialog Boxperiphgotoaddressdialog

Go To Register Dialog Boxperiphgotoregisterdialog

# # $ K Go To Register Dialog Box

1.	Click the down-arrow on the drop-down box to display the list of peripheral groups.

2.	Enter a register name for repositioning the Peripheralperiph window display.

See also

Peripheral Go To Address Dialog Boxperiphgotoaddressdialog

Go To Peripheral Dialog Boxperiphgotoperiphdialog

# # $ K Edit Peripheral Data Register Dialog Box

The dialog box displays:

-	A listbox that shows the data in all memory locations that comprise the buffer.

-	An edit field for the selected memory location.

To edit the buffers:

1.	Highlight the memory location you want to change

2.	Type the new value.

3.	When finished with all changes, write all locations of the buffer by choosing OK.

4.	Choose Cancel to exit the dialog box without making changes.

See also

Edit Peripheral Register Dialog Boxeditperiphregdialog

Receive and Transmit Buffersbufferperiph

# # $ K Edit Peripheral Register Dialog Box

The dialog box displays:

The register name in the title.

A list box with register fields.

The field to the right of the Field Value spin box.

Use the dialog box as follows:

1.	Edit the register value.

-or-

Use the list box to select a register field and edit it with the Field Value spin box.

2.	Choose  Write to write to the register.  This does not close the dialog box so you can make further changes and write again.

3.	When finished, choose Close to close the dialog box.  No writes take place when Close is chosen.

See also

Edit Peripheral Data Register Dialog Boxeditperiphregbufferdialog

# toolbar

$ ToolBar Window

K toolbar;window;emulation

# minimizingpowerviewsfrom

$ Minimizing MICEpack/SLD from the ToolBar

K icons;minimizing;window

# minimizingpowerviewsfrom

K icons;minimizing;window

$ Moving Between Windows

# movingbetweenwindows

K window

$ RAM

# ramterm

K RAM;memory

$ ROM

# romterm

K ROM;memory

$ Mapping Memory

# mappingmemory

K map;memory;toolbar;overlay;target;

$ Map Dialog Box

# mapdialogbox

K map;Dialog Box;memory

# mapshell

$ Map Shell Command

K map;memory;load;shell, toolbar

$ On-line Help

# onlinehelp

K help;window

$ Symbol Entry Syntax

# symbolentrysyntax

K symbols

# versionshell

$ Version Shell Command

K version;MICEpack/SLD;revision;shell, toolbar;

$ Swapfile

# swapfileterm

K swapfile

$ Exiting MICEpack/SLD

# exitingpowerviews

+ gettingstarted:0130

K exit;MICEpack/SLD

# periph

$ Peripheral Window

K peripherals;registers

# editperiph

$ Edit Peripheral Register Dialog Box

K peripherals;registers;edit peripherals;

# exformperiph

$ Expanded Format Peripheral Window

K peripherals;registers;expanded format

# comformperiph

$ Compressed Format Peripheral Window

K peripherals;registers;compressed format

# updateperiph

$ Update Peripheral Registers During Emulation

K peripherals;registers;run access;update peripherals;

# scrollperiph

$ Scrolling the Peripheral Window

K peripherals;

# bufferperiph

$ Receive and Transmit Buffers

K peripherals;receive buffers;transmit buffers;edit buffers;

# resettool

$ Reset

K reset;processor reset

# resetcpuonly

$ Reset CPU only

K CPU;reset CPU;

# pregistershell

$ Pregister Shell Command

K peripherals;shell, peripheral;registers;

# periphgotoaddressdialog

$ Peripheral Go To Address Dialog Box

K peripherals;go to address;dialog box;

# periphgotoperiphdialog

$ Go To Peripheral Dialog Box

K peripherals;go to peripheral;dialog box;

# periphgotoregisterdialog

$ Go To Register Dialog Box

K Peripheral;Register;Go To Register

# editperiphregbufferdialog

$ Edit Peripheral Data Register Dialog Box

K peripherals;edit peripheral;dialog box;

# editperiphregdialog

$ Edit Peripheral Register Dialog Box

K peripherals;edit peripheral;dialog box;