# # $ K Processor type not supported.

Mismatched error message.  Contacting Microtekcontactingmicrotek

# # $ K Cannot find module.

Cause:	The module specified is not in the symbol table.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell lists modules.

See also

Source Windowsource

Load Shell Commandloadshell

# # $ K Cannot add public symbol to symbol table: <symbol name>.

Mismatched error message.  Contacting Microtekcontactingmicrotek

# # $ K Out of memory during load.

Cause:	No more memory available from Windows.

Solution:	Close other applications or PowerViews windows to free memory.

# # $ K Module time stamp is later than the original load file.

Cause:	Modifications to the load file were made.

Solution:	Load symbols again.  If code was re-compiled and re-linked, symbols need to be reloaded to match those in the loadfile.

See also

Source Windowsource

# # $ K User abort requested.

Cause:	The Escape key aborted the load operation.

Solution:	The code and symbolic information may not be complete.

# # $ K Symbols already loaded, reload?

Cause:	Symbols from a load file are currently loaded.

Solution:	Symbols from one load file supported.  Delete symbols when queried or use the removeSymbols Shell commandremovesymbolsshell.

See also

ToolBar Windowtoolbar

Source Windowsource

# # $ K No starting PC available.

Cause:	The load file does not contain starting program counter information.

Solution:	The PC is not set.  Use the CPU Windowcpu or register Shell commandregistershell to set PC and stack before starting emulationemulationterm.

# # $ K Register translation table contains invalid value.

Cause:	The loader configuration file ldrcpu16.cfg or ldrcpu32.cfg is corrupt.

Solution:	Re-install the PowerViews software into a new directory and copy these files to the existing directory.  Or, use the new PowerViews installation.

# # $ K User abort requested.

Cause:	The Escape key aborted the load operation.

Solution:	The code and symbolic information may not be complete.

# # $ K Cannot load code while processor running.

Cause:	Code cannot be loaded while emulatingemulationterm unless run accessrunaccessterm is enabled.

Solution:	Stop emulation if loading code that is currently executing.

# # $ K Cannot update PC, SP while processor running.

Cause:	The registers cannot be written while emulatingemulationterm.

Solution:	Stop emulation to set register values.  The PC and SP are set if emulation is not running when loading.

# # $ K Symbols already loaded, specify 'reload' option.

Cause:	Symbols from a load file are currently loaded.

Solution:	Symbols from one load file supported.  Delete symbols using the removeSymbols Shell commandremovesymbolsshell or specified the reload option in the load Shell commandloadshell.

See also

ToolBar Windowtoolbar

Source Windowsource

# # $ K Loading is in progress, cannot perform the requested operation.

Cause:	Only one load can be active at a time.

Solution:	PowerViews is not currently multi-tasking.

# # $ K Cannot find Windows directory.

Cause:	The environment variable, windir, is missing.

Solution:	Set the environment variable to the location of Windows.  The Windows set up takes care of this typically.  There may be further problems with the Windows set up.

# # $ K Cannot find initialization file.

Cause:	The initialization file, pwrviews.ini, is missing.

Solution:	The initialization file must reside in the Windows directory.  The PowerViews installation places it there.  Re-install the PowerViews software into a new directory to get this file.  Use the existing or new PowerViews installation.

# # $ K Tool not defined in initialization file.

Cause:	The initialization file, pwrviews.ini, is missing the loader information.

Solution:	The initialization file may be missing or the [ToolChain] section in the file is missing.  Re-install the PowerViews software into a new directory to get this file.  Use the existing or new PowerViews installation.

# # $ K Invalid section name.

Cause:	An entry in the initialization file, pwrviews.ini, under the [ToolChain] section is incorrect.

Solution:	Check any editing changes.  Re-install PowerViews software into a new directory to get the original initialization file.  Use the existing or new PowerViews installation.

# # $ K Missing section name information.

Cause:	An entry in the initialization file, pwrviews.ini, under the [ToolChain] section is incorrect.

Solution:	Check any editing changes.  Re-install PowerViews software into a new directory to get the original initialization file.  Use the existing or new PowerViews installation.

# # $ K No compilers defined in initialization file.

Cause:	In the initialization file, pwrviews.ini, the [ToolChain] section is missing the Compilers= line.

Solution:	Check any editing changes.  Re-install PowerViews software into a new directory to get the original initialization file.  Use the existing or new PowerViews installation.

# # $ K Cannot get compiler defined in initialization file.

Cause:	In the initialization file, pwrviews.ini, the [ToolChain] section member Compilers= is corrupted.

Solution:	Check any editing changes.  Re-install PowerViews software into a new directory to get the original initialization file.  Use the existing or new PowerViews installation.

# # $ K Missing compiler information in initialization file.

Cause:	In the initialization file, pwrviews.ini, the [ToolChain] compiler information is corrupted.

Solution:	One of the compiler lines (e.g. MRI=) is bad.  Check any editing changes.  Re-install PowerViews software into a new directory to get the original initialization file.  Use the existing or new PowerViews installation.

# # $ K Compiler name too long.

Cause:	In the initialization file, pwrviews.ini, the [ToolChain] compiler information contains a compiler line with a name greater than 30 characters.

Solution:	Check any editing changes and reduce the number of characters on a compile line (e.g. MRI= is a compiler line).  Re-install PowerViews software into a new directory to get the original initialization file.  Use the existing or new PowerViews installation.

# # $ K Cannot write to initialization file.

Cause:	In the initialization file, pwrviews.ini, cannot be written.

Solution:	Check that the initialization file in the Windows directory (e.g. c:\windows) is writeable.

# # $ K Maximum number of sections reached; the rest will be ignored.

Mismatched error message.  Contacting Microtekcontactingmicrotek

# # $ K Incompatible load file processor type: <processor>.

Cause:	The compiler specified an unsupported processor type.

Solution:	Check the flags in the compiler.

# # $ K Cannot find section defined in initialization file: <section name>.

Mismatched error message.  Contacting Microtekcontactingmicrotek

# # $ K No line numbers in module: <module name>.

Cause:	A module has no line number debug information.

Solution:	Source debugging is not possible without line number information.  Check compiler switches to enable line number debug information.

# # $ K Loader configuration file corrupted.

Cause:	The loader configuration file is corrupt.

Solution:	Re-install the PowerViews software.

# # $ K Invalid frame number.

Cause:	The frame number is not valid.

Solution:	Enter a number (1234, 0x4444, 0177) for the frame number.

See also

Trace Windowtrace

# # $ K Too many template fields for one line.

Cause:	The trace configuration file in \pwrviews is corrupted.

Solution:	An event configuration file (evXXX.cfg) has too many entries on a line.  Re-install the PowerViews software.

See also

Trace Windowtrace

# # $ K Invalid search event name.

Cause:	The event name given does not exist.

Solution:	The Event Window Active Event list has no entry for event.  If it is saved on disk, use the Event Window'sevent File/Restore Events... to bring into the Active Event list.

See also

Trace Windowtrace

# # $ K Firmware not compatible with host software.

Cause:	The firmware in the PowerPack does not match the host software.

Solution:	Program the firmware as follows:

1.	Choose OK to exit PowerViews application.

2.	Exit from Windows to DOS.  The firmware installation will not work in a DOS shell under Windows.

3.	Place floppy 2 in disk drive.

4.	Enter "a:" or "b:" to set default directory to floppy.

5.	Enter "install".

6.	Specify comm port number when queried.

7.	A success or failure message is displayed when complete.

If failed, check the comm port number and the RS-232 connection.  Be sure to power on PowerPack (or hit the reset button) when queried.

# # $ K Timed out waiting for hardware to respond.

Cause:	The communications between the host PC and the hardware failed.

Solution:	The software retries many times before reporting this error.  Check the RS-232 connection and try operation again.  If it still fails, exit PowerViews and start again.  Success this time indicates a possible software bug to report to Microtekcontactingmicrotek

.

# # $ K Cannot open the connection.

Cause:	The communications port cannot be opened.

Solution:	The wrong comm port is specified or another application is using the comm port.  Close the other application.  If this is not the problem, exit Windows and re-boot the PC to re-initialize the comm port hardware.

See also

Changing Comm Portcommportswitching

# # $ K Session terminated.

Cause:	This is an informational message.

Solution:	Read the Up and Running manual for information on getting started with PowerViews.

See also

Solutions to Common Problemssolutionstocommon

# # $ K Communication time-out sending to box.

Cause:	The communications between the host PC and the hardware failed.

Solution:	The software retries many times before reporting this error.  Check the RS-232 connection and try operation again.  If it still fails, exit PowerViews and start again.  Success this time indicates a possible software bug to report to  Microtekcontactingmicrotek

.

# # $ K Excessive errors in communication from host to box.

Cause:	The communications between the host PC and the hardware failed.

Solution:	The software retries many times before reporting this error.  Check the RS-232 connection and try operation again.  If it still fails, exit PowerViews and start again.  Success this time indicates a possible software bug.  Contact Microtekcontactingmicrotek

.

# # $ K Cannot initialize probe.

Cause:	Communications with the probe cannot be established for initialization.

Solution:	Check the following:

1.	The cable from PowerPack chassis Probe Control to emulation pod has no bent pins and is secured in the connectors.

2.	The double cables from the emulation pod to the probe tip are locked in place.

3.	The two wire power cord from the emulation pod to the probe tip is connected.  The LED on the probe tip should be lit.

Re-seat all cables in case of a bad connection.  If probe still does not initialize, contact Microtekcontactingmicrotek

technical support.

# # $ K PowerScope probe version 1.4 or higher required for ROM breakpoints.

Mismatched error message.  Contact Microtekcontactingmicrotek

# # $ K PowerScope probe version 1.4 or higher required for internal termination.

Mismatched error message.  Contact Microtekcontactingmicrotek

# # $ K Cannot read mapped memory type.

Cause:	A hardware error with the mapped memory was detected.

Solution:	Contact Microtekcontactingmicrotek .

# # $ K Bus error occurred during memory cycle.

Cause:	A bus error occurred while accessing memory.

Solution:	Check the memory mapmappingmemory for valid memory in range.

# # $ K Probe transmission error.

Cause:	A communications error occurred between the emulation pod and the probe tip.

Solution:	Check the following:

1.	The double cables from the emulation pod to the probe tip are locked in place.

2.	The two wire power cord from the emulation pod to the probe tip is connected.  The LED on the probe tip should be lit.

3.	Reset the processorresettingtheprocessor since it may be hung.

Re-seat all cables in case of a bad connection.  If probe still does not initialize, contact Microtekcontactingmicrotek.

.

# # $ K Unterminated memory cycle; internal termination failed.

Cause:	Memory at an invalid address was accessed and PowerPack's attempt to terminate the memory failed.

Solution:	Reset the processorresettingtheprocessor.

# # $ K Processor reset during BDM exchange.

Cause:	The processor reset while communicating between the emulation pod and the probe tip.

Solution:	A possible hardware problem exists with the target.

# # $ K Processor clock speed is slower than 131 KHz.

Mismatched error message.  Contact Microtekcontactingmicrotek.

# # $ K Memory verification failed.

Cause:	Memory verification is enabled.  A written memory location failed to read back the proper data.

Solution:	Check the memory mapmappingmemory to verify memory at the location being accessed.

See also

Verify Shell Commandverifyshell

Memory Windowmemory

# # $ K Timed out waiting for emulation to stop.

Cause:	A communications failure between the emulation pod and the probe tip occurred while halting emulationemulationterm.

Solution:	Reset the processorresettingtheprocessor.

# # $ K Timed out waiting for single step.

Cause:	A communications failure between the emulation pod and the probe tip occurred while steppingemulationterm.

Solution:	Reset the processorresettingtheprocessor.

# # $ K Fill command aborted prior to completion.

Cause:	The Escape key aborted the memory fill in progressemulationterm.

Solution:	Check the range for the fill and reduce the range to speed operation.

# # $ K Execution breakpoint requires active AS or CS.

Mismatched error message.  Contacting Microtekcontactingmicrotek

# # $ K Fast termination CS is active; cannot map.

Mismatched error message.  Contacting Microtekcontactingmicrotek

# # $ K Timed out on reset entry to BDM.

Cause:	The communications with the processor's BDM port failed.

Solution:	Reset the processorresettingtheprocessor.

# # $ K BKPT pin was asserted spuriously.

Cause:	Emulation was halted because the BKPT pin was asserted.

Solution:	When breakpoints are being used, the BKPT is asserted by the PowerPack hardware to stop emulation.  A BKPT occurred, but PowerPack did not generate it.  Check the target hardware for spurious BKPT assertion.

# # $ K Attempted halt failed.

Cause:	The probe timed out responding to the halt request.

Solution:	The processor may be in a STOP state.

# # $ K SIM register bank cannot be located.

Cause:	Cannot read memory at either of the two possible locations for the SIM.

Solution:	Check for overlapping internal RAM, slow processor clock, reset assertion.

See also

Accessing Memorymemoryaccess

# # $ K Trace instruction processing aborted.

Cause:	The Escape key aborted the trace instruction calculation in progressemulationterm.

Solution:	This is an informational message.

See also

Trace Windowtrace

# # $ K No bus cycles collected in trace buffer.

Cause:	The current trace buffer does not contain any bus cycles for trace instruction processingemulationterm.

Solution:	This is an informational message.

See also

Trace Windowtrace

# # $ K Trigger programming aborted.

Cause:	The Escape key aborted the programming in progressemulationterm.  Emulationemulationterm is not started.

Solution:	This is an informational message.

# # $ K Invalid trace buffer.

Cause:	The trace buffer has no data collected in it yet.

Solution:	Check the sequential actionsselectingtriggeractions in the trigger programming.  A trigger may not have been reached yet.  Check the Status Windowstatus to see which buffer is currently collecting trace.

# # $ K Unterminated memory access; internal termination used.

Mismatched error message.  Contact Microtekcontactingmicrotek.

# # $ K Memory copy aborted.

Cause:	The Escape key aborted the memory copy in progressemulationterm.

Solution:	Only a portion of the memory was copied.

# # $ K Memory search aborted.

Cause:	The Escape key aborted the memory search in progressemulationterm.

Solution:	The search was terminated.

# # $ K Search found at <address>.

Cause:	The search pattern was found at the address shown.emulationterm

Solution:	This is an informational message.

See also

Memory Windowmemory

# # $ K Search not found.

Cause:	The search pattern was not found in the memory range specified.emulationterm

Solution:	This is an informational message.

See also

Memory Windowmemory

# # $ K Load Aborted.

Cause:	The Escape key was pressed to terminate the load.

Solution:	The code was not completely loaded.

See also

Load Shell Commandloadshell

Source Windowsource

# # $ K Probe firmware version incompatible with PowerViews.

Cause:	PowerViews 2.0 and above requires a newer version of the 8751 firmware.

Solution:	Contact Microtekcontactingmicrotek for upgrade information.

# # $ K Abort from Escape.

Cause:	The Escape key was pressed to terminate a lengthy operation.

Solution:	The operation was not completed.

# # $ K Expected ATN record type.

Cause:	An ATN record was expected but not found in the load file.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Unrecognized miscellaneous record skipped.

Cause:	A record of unknown type is in the load file, and was skipped during loading.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Attribute type not supported.

Cause:	An attribute type in the load file is not supported by PowerViews.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Cannot read file.

Cause:	Can't read the load file.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Invalid or unsupported ID.

Cause: The communications port selected is invalid or unsupported.

Solution: Select a communications port which is supported by your PC.

# # $ K Invalid module id.

Cause:	A module ID in the load file is invalid.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Bad ATI record in Loadfile.

Cause:	An ATI record in the loadfile is corrupted.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Corrupted file.

Cause:	The load file has a format error.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Invalid 695 file header.

Mismatched error message.    Contact Microtekcontactingmicrotek.

# # $ K Invalid line number block in file.

Cause:	There is an invalid line number block in the load file.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Nested block is not a procedure.

Cause:	A nested block in the load file is not a procedure.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Invalid part of file.

Cause:	A section of the load file is unrecognized or invalid.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Invalid register index.

Cause:	A register index in the load file is invalid.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Invalid section record.

Cause:	A section record in the load file is invalid.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Cannot seek load file.

Cause:	Read operations on the load file are not working.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Expected begin-block function record.

Cause:	The load file cannot be read.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Current function has no call or return instructions.

Cause:	This may be caused by normal compiler optimization, in some cases.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K PC-NFS is not installed.

Cause:	PC-NFS is not installed, but is needed.

Solution:	Install PC-NFS or reboot your computer if needed.

# # $ K The probe transmission is faster than BDM.

Mismatched error message.    Contact Microtekcontactingmicrotek.

# # $ K Must define CompilerUsed information, specify 'CompilerUsed <compiler>'

Mismatched error message.    Contact Microtekcontactingmicrotek.

# # $ K Cannot add module reference to symbol table.

Cause:	An internal module name can't be added.  This is usually not a serious problem.

Solution:	Check the switches on the compiler command line.

# # $ K Invalid clock frequency.

Cause:	The clock frequency is not in the range 0.01 Hz - 40 MHz.

Solution:	Enter a valid clock frequency.

See also

Clock frequency dialogcrystalfrequencydialog

# # $ K Breakpoint already set.

Cause:	A breakpoint has already been set at the current location.

Solution:	Re-check the address of the breakpoint.

# # $ K Invalid number of parameters.

Cause:	The Shell command you used takes a different number of parameters.

Solution:	Double-check the syntax of the Shell command.

# # $ K Operation not allowed while emulating.

Cause:	An operation cannot be done while emulation is running.

Solution:	Enable runaccessrunaccessshell from the ToolBar/Configure menu or by using the RunAccess Shell Commandrunaccessshell.  This will allow memory commands to occur during emulation.

# # $ K CPU configuration file missing.

Cause:	The CPU configuration file is missing.

Solution:	Reinstall PowerViews.

# # $ K Optimized code detected: source view and variable access may be incorrect.

Cause:	The loadfile used contains optimized code.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Invalid tab width.

Cause:	The tab width used is invalid.

Solution:	Choose a different tab width, between 1 and 32 spaces.

# # $ K Stack high water measurement is disabled.

Cause:	PowerViews has failed to enable the HighWaterMarkhighwatermark.

Solution:	Reset/Reinitialize stack area for the HighWaterMark.  Use the fillStackPatternfillstackpatternshell Shell command.

# # $ K Cannot open a stack session.

Cause:	This should only occur if you run out of memory, or some other fairly catastrophic system failure has occurred.

Solution:	Try restarting Windows and PowerViews -- if you get this error again, please contact Microtekcontactingmicrotek.

# # $ K Unable to access communication type.

Cause:	The communication type was not specified in the PWRVIEWS.INI file.

Solution:	In PWRVIEWS.INI, check the communication type in the "[Comm]" section; the line "type = serial" sets the communication type to serial, and "type = network" sets the communication type to network.

# # $ K Network communication style not selected.

Cause:	Although network system code in PowerViews is active, the indicated communication type is not network.

Solution:	In PWRVIEWS.INI, check the communication type in the "[Comm]" section; network communication must be specified as "type = network".

# # $ K Unable to find network emulator host names.

Cause:	The list of available emulators was not found in the PWRVIEWS.INI file.

Solution:	In PWRVIEWS.INI, check the emulator names in the "[Network]" section; the line starting with "emulators =" must contain a comma-separated list of one or more emulators available on the network.

# # $ K Unable to create host selection dialog.

Cause:	Internal error.

Solution:	  Contact Microtekcontactingmicrotek.

# # $ K PC-NFS is not running.

Cause:	PC-NFS must be resident for network communications to occur.  PC-NFS either hasn't been installed properly, or was not resident when PowerViews initialized.

Solution:	Install PC-NFS if necessary.  Activate PC-NFS prior to starting Windows.

# # $ K PC-NFS RTM is not resident.

Cause:	PC-NFS must be resident for network communications to occur.  PC-NFS either hasn't been installed properly, or was not resident when PowerViews initialized.

Solution:	Install PC-NFS if necessary.  Activate PC-NFS prior to starting Windows.

# # $ K PC-NFS TKLIB.DLL is not accessible.

Cause:	PC-NFS must be resident for network communications to occur.  PC-NFS either hasn't been installed properly, or was not resident when PowerViews initialized.

Solution:	Install PC-NFS if necessary.  Activate PC-NFS prior to starting Windows.

# # $ K PC_NFS RNM.DLL is not accessible.

Cause:	PC-NFS must be resident for network communications to occur.  PC-NFS either hasn't been installed properly, or was not resident when PowerViews initialized.

Solution:	Install PC-NFS if necessary.  Activate PC-NFS prior to starting Windows.

# # $ K Failed to establish communication endpoint.

Cause:	Internal error.

Solution:	  Contact Microtekcontactingmicrotek.

# # $ K Unable to access emulator address information.

Cause:	Internal error.

Solution:	Contact Microtekcontactingmicrotek technical support for help.

# # $ K Unable to access host address information.

Cause:	Internal error.

Solution:	  Contact Microtekcontactingmicrotek.

# # $ K Unable to establish network connection.

Cause:	The network communications failed to establish a connection.

Solution:	Verify the PC's and emulator's physical network connections.

Verify that the emulator is ready to receive connections (i.e., powered on and selftest completed).

# # $ K Network connection timed out.

Cause:	The network communications failed to establish a connection within the time-out period.

Solution:	Retry the operation.

Verify the PC's and emulator's physical network connections.

Verify that the emulator is ready to receive connections (i.e., powered on and selftest completed).

# # $ K The emulator is currently in use.

Cause:	The emulator is currently in use by another user.

Solution:	Only one user can use the emulator at a time.  You must wait for the current emulator user to finish, or bump the current user from the emulator.

Note that, if a user terminates a previous PowerViews session abnormally, the emulator may be still connected.  This spurious connection can be broken by physically resetting the emulator, or by overriding the connection.

Note that, if you override the current connection while the emulator is in use, the other user's PowerViews session is terminated.

# # $ K Network read operation failed.

Cause:	The network read operation failed.

Solution:	Retry the operation.

Verify the PC's and emulator's physical network connections.

Verify that the emulator is ready to communicate.

Verify network integrity and check network load.

Contact Microtekcontactingmicrotek.

# # $ K Network write operation failed.

Cause:	The network write operation failed.

Solution:

-	Retry the operation.

-	Verify the PC's and emulator's physical network connections.

-	Verify that the emulator is ready to communicate.

-	Verify network integrity and check network load.

-	  Contact Microtekcontactingmicrotek.

# # $ K Network close operation failed.

Cause:	The network close operation failed.

Solution:

-	Retry the operation.

-	Verify the PC's and emulator's physical network connections.

-	Verify that the emulator is ready to communicate.

-	Verify network integrity and check network load.

-	  Contact Microtekcontactingmicrotek.

# # $ K Network read operation timed out.

Cause:	The network read operation did not complete within the time limit.

Solution:

-	Retry the operation.

-	Verify the PC's and emulator's physical network connections.

-	Verify that the emulator is ready to communicate.

-	Verify network integrity and check network load.

-	  Contact Microtekcontactingmicrotek.

# # $ K OS/2 LAN Server is not running.

Cause:	LAN Server must be resident for network communications to occur.  LAN Server either hasn't been installed properly, or was not resident when PowerViews initialized.

Solution:	Install LAN Server if necessary.  Activate LAN Server prior to starting Windows.

# HE_NOT_CPU

$ Processor type not supported.

K error, o-t

# HE_MODULE_NOT_FOUND

$ Cannot find module.

K error, a-g

# HE_CANNOT_ADD_PUBLIC

$ Cannot add public symbol to symbol table: <symbol name>.

K error, a-g

# HE_NO_MEMORY

$ Out of memory during load.

K error, o-t

# HE_MODTIME

$ Module time stamp is later than the original load file.

K error, h-n

# HE_USER_ABORT

$ User abort requested.

K error, u-z

# HE_SYM_ALREADY

$ Symbols already loaded, reload?

K error, o-t

# HE_NO_PC

$ No starting PC available.

K error, h-n

# HE_REGTABLE

$ Register translation table contains invalid value.

K error, o-t

# HE_LDR_ABORT

$ User abort requested.

K error, u-z

# HE_NORUNACCESS

$ Cannot load code while processor running.

K error, a-g

# HE_NOREG_UPDATE

$ Cannot update PC, SP while processor running.

K error, a-g

# HE_SYM_ALREADY_CLI

$ Symbols already loaded, specify 'reload' option.

K error, o-t

# HE_IN_PROGRESS

$ Loading is in progress, cannot perform the requested operation.

K error, h-n

# HE_CANNOT_FIND_WDIR

$ Cannot find Windows directory.

K error, a-g

# HE_CANNOT_FIND_PFILE

$ Cannot find initialization file.

K error, a-g

# HE_NO_SECTION_NAME_DEFINED

$ Tool not defined in initialization file.

K error, o-t

# HE_INVALID_SECTION_NAME

$ Invalid section name.

K error, h-n

# HE_MISSING_SECTION_NAME

$ Missing section name information.

K error, h-n

# HE_NO_TOOLS_SUPPORT_DEFINED

$ No compilers defined in initialization file.

K error, h-n

# HE_CANNOT_GET_TOOL_NAME

$ Cannot get compiler defined in initialization file.

K error, a-g

# HE_MISSING_TOOLS_SUPPORT

$ Missing compiler information in initialization file.

K error, h-n

# HE_TOOL_NAME_TOO_LONG

$ Compiler name too long.

K error, a-g

# HE_CANNOT_WRITE_TO_FILE

$ Cannot write to initialization file.

K error, a-g

# HE_TOOMANY_SEC

$ Maximum number of sections reached; the rest will be ignored.

K error, h-n

# HE_INCOMPATIBLE_CPU

$ Incompatible load file processor type: <processor>.

K error, h-n

# HE_DEFSEC_NOT_FOUND

$ Cannot find section defined in initialization file: <section name>.

K error, a-g

# HE_NO_LINENUMS_ADDED

$ No line numbers in module: <module name>.

K error, h-n

# HE_CONFIG_FILE_FORMAT

$ Loader configuration file corrupted.

K error, h-n

# HE_INVALID_FRAME

$ Invalid frame number.

K error, h-n

# HE_TOO_MANY_FIELDS

$ Too many template fields for one line.

K error, o-t

# HE_NO_SUCH_EVENT

$ Invalid search event name.

K error, h-n

# HE_MISMATCHED_SHARED_DATA

$ Firmware not compatible with host software.

K error, a-g

# HE_TIMED_OUT

$ Timed out waiting for hardware to respond.

K error, o-t

# HE_COM_OPEN

$ Cannot open the connection.

K error, a-g

# HE_ABORT_SESSION

$ Session terminated.

K error, o-t

# HE_COMM_PACKET_TIMEOUT

$ Communication time-out sending to box.

K error, a-g

# HE_COMM_RETRY_EXHAUSTED

$ Excessive errors in communication from host to box.

K error, a-g

# HE_CANT_INIT_PROBE

$ Cannot initialize probe.

K error, a-g

# HE_PROBE_VERSION_NO_XBRKS

$ PowerScope probe version 1.4 or higher required for ROM breakpoints.

K error, o-t

# HE_PROBE_VERSION_NO_RETRY

$ PowerScope probe version 1.4 or higher required for internal termination.

K error, o-t

# HE_OVERLAY_HARDWARE

$ Cannot read mapped memory type.

K error, a-g

# HE_BUS_ERROR

$ Bus error occurred during memory cycle.

K error, a-g

# HE_ILLEGAL_CMD

$ Probe transmission error.

K error, o-t

# HE_BUS_TIMEOUT

$ Unterminated memory cycle; internal termination failed.

K error, u-z

# HE_RESET_DURING_BDM

$ Processor reset during BDM exchange.

K error, o-t

# HE_SLOW_CLOCK

$ Processor clock speed is slower than 131 KHz.

K error, o-t

# HE_MEMORY_VERIFY

$ Memory verification failed.

K error, h-n

# HE_WAIT_EMOFF_TIMEOUT

$ Timed out waiting for emulation to stop.

K error, o-t

# HE_STEP_TIMEOUT

$ Timed out waiting for single step.

K error, o-t

# HE_FILL_ABORT

$ Fill command aborted prior to completion.

K error, a-g

# HE_NO_STROBE

$ Execution breakpoint requires active AS or CS.

K error, a-g

# HE_FAST_TERMINATION

$ Fast termination CS is active; cannot map.

K error, a-g

# HE_WAIT_BDM_TIMEOUT

$ Timed out on reset entry to BDM.

K error, o-t

# HE_CAUSE_STATUS

$ BKPT pin was asserted spuriously.

K error, a-g

# HE_HALT_TIMEOUT

$ Attempted halt failed.

K error, a-g

# HE_SIM_UNREADABLE

$ SIM register bank cannot be located.

K error, o-t

# HE_DQ_ABORTED

$ Trace instruction processing aborted.

K error, o-t

# HE_DQ_NO_FRAMES_AVAILABLE

$ No bus cycles collected in trace buffer.

K error, h-n

# HE_TRIGGER_ABORTED

$ Trigger programming aborted.

K error, o-t

# HE_INVALID_BUFFER

$ Invalid trace buffer.

K error, h-n

# HE_INTERNAL_TERMINATION

$ Unterminated memory access; internal termination used.

K error, u-z

# HE_COPY_ABORT

$ Memory copy aborted.

K error, h-n

# HE_SEARCH_ABORT

$ Memory search aborted.

K error, h-n

# HE_SEARCH_FOUND

$ Search found at <address>.

K error, o-t

# HE_SEARCH_NOT_FOUND

$ Search not found.

K error, o-t

# HE_LOAD_ABORT

$ Load Aborted.

K error, h-n

# HE_PROBE_FW_VERSION

$ Probe firmware version incompatible with PowerViews.

K error, o-t

# HE_ABORT_FROM_ESC

$ Abort from Escape.

K error, a-g

# HE_ATN_EXPT

$ Expected ATN record type.

K error, a-g

# HE_ATN_NOT_HANDLED

$ Unrecognized miscellaneous record skipped.

K error, u-z

# HE_ATTR_NOT_HANDLED

$ Attribute type not supported.

K error, a-g

# HE_BADGET

$ Cannot read file.

K error, a-g

# HE_BADID

$ Invalid or unsupported ID.

K error, h-n

# HE_BADMODULE

$ Invalid module id.

K error, h-n

# HE_BAD_ATI_RECORD_IN_FILE

$ Bad ATI record in Loadfile.

K error, a-g

# HE_BAD_FILE

$ Corrupted file.

K error, a-g

# HE_BAD_HDR

$ Invalid 695 file header.

K error, h-n

# HE_BAD_LNUM_BLOCK_IN_FILE

$ Invalid line number block in file.

K error, h-n

# HE_BAD_NEST

$ Nested block is not a procedure.

K error, h-n

# HE_BAD_PART

$ Invalid part of file.

K error, h-n

# HE_BAD_REG_INDEX

$ Invalid register index.

K error, h-n

# HE_BAD_SECREC

$ Invalid section record.

K error, h-n

# HE_BAD_SEEK

$ Cannot seek load file.

K error, a-g

# HE_BBFUNCTION_EXPT

$ Expected begin-block function record.

K error, a-g

# HE_BX_NO_CALL_RETURN

$ Current function has no call or return instructions.

K error, a-g

# HE_CLIULIB_PCNFS_NOT_INSTALLED

$ PC-NFS is not installed.

K error, o-t

# HE_INPUT_OVERRUN

$ The probe transmission is faster than BDM.

K error, o-t

# HE_MUST_DEFINE_TOOLUSE

$ Must define CompilerUsed information, specify 'CompilerUsed <compiler>'

K error, h-n

# HE_NAME_NOT_ADDED

$ Cannot add module reference to symbol table.

K error, a-g

# HE_BAD_FREQUENCY

$ Invalid clock frequency.

K error, h-n

# HE_BX_ALREADY_HAS_BKPT

$ Breakpoint already set.

K error, a-g

# HE_DAD_BAD_NUM_PARM

$ Invalid number of parameters.

K error, h-n

# HE_ILLEGAL_WHILE_EMON

$ Operation not allowed while emulating.

K error, o-t

# HE_NO_CONFIG

$ CPU configuration file missing.

K error, a-g

# HE_OPTIMIZED

$ Optimized code detected: source view and variable access may be incorrect.

K error, o-t

# HE_SRC_INVALID_INPUT_TAB

$ Invalid tab width.

K error, h-n

# HE_STACK_HW_DISABLED

$ Stack high water measurement is disabled.

K error, o-t

# HE_STACK_NO_SESSION

$ Cannot open a stack session.

K error, a-g

# HE_NET_PROFILE_MISSING

$ Unable to access communication type.

K error, u-z

# HE_NET_NOT_NETWORK

$ Network communication style not selected.

K error, h-n

# HE_NET_HOST_NAMES_NOT_FOUND

$ Unable to find network emulator host names.

K error, u-z

# HE_NET_HOST_SELECT_DIALOG

$ Unable to create host selection dialog.

K error, u-z

# HE_PCNFS_NOT_RUNNING

$ PC-NFS is not running.

K error, o-t

# HE_PCNFS_RTM_NOT_RESIDENT

$ PC-NFS RTM is not resident.

K error, o-t

# HE_PCNFS_TKDLL_NOT_FOUND

$ PC-NFS TKLIB.DLL is not accessible.

K error, o-t

# HE_PCNFS_RNM_NOT_FOUND

$ PC_NFS RNM.DLL is not accessible.

K error, o-t

# HE_NET_COMM_ENDPOINT

$ Failed to establish communication endpoint.

K error, a-g

# HE_NET_EMULATOR_INFO

$ Unable to access emulator address information.

K error, u-z

# HE_NET_HOST_INFO

$ Unable to access host address information.

K error, u-z

# HE_NET_CONNECT

$ Unable to establish network connection.

K error, u-z

# HE_NET_CONNECT_TIMEOUT

$ Network connection timed out.

K error, h-n

# HE_NET_EMULATOR_IN_USE

$ The emulator is currently in use.

K error, o-t

# HE_NET_READ

$ Network read operation failed.

K error, h-n

# HE_NET_WRITE

$ Network write operation failed.

K error, h-n

# HE_NET_CLOSE

$ Network close operation failed.

K error, h-n

# HE_NET_READ_TIMEOUT

$ Network read operation timed out.

K error, h-n

# HE_OS2_LAN_SERVER_NOT_RUNNING

$ OS/2 LAN Server is not running.

K error, o-t