# $ # + K Breakpoint Window

The Breakpoint Window allows software and hardware breakpointsbreakpointterm to be set and cleared.

Screen Description

The window lists the breakpoints.  The state is enabled or disabled.  The type is permanent or temporary.  The Breakpoints line lists the address and the symbolic equivalent, if the address is in a function for which source is available.

Buttons or the Breakpoints menu can be used to change settings.

Button Bar

Click on one of the buttons to access the feature:

Set	Set a breakpoint.

Clear	Clear the selected breakpoint.

Go To	Scroll the Source Window to the selected breakpoint.

Enable	Enable the selected breakpoint.

Disable	Disable the selected breakpoint.

Clear All	Clear all breakpoints.

Enable All	Enable all breakpoints.

Disable All	Disable all breakpoints

File

Exit	Close the Breakpoint Window.

Breakpoints

Set Breakpoint...	Set a breakpoint.

Clear	Clear the selected breakpoint.

Enable	Enable the selected breakpoint.

Disable	Disable the selected breakpoint.

Clear All	Clear all breakpoints.

Enable All	Enable all breakpoints.

Disable All	Disable all breakpoints

Go to Source	Scroll the Source Window at the selected breakpoint.

Windows	Move between windowsmovingbetweenwindows

Help	On-line helponlinehelp

Using the Breakpoint Window

Setting and Clearing Breakpointssettingandclearing

Search Keywords

shell, breakpoint

# $ # + K Setting and Clearing Breakpoints

To set a breakpoint:

-	Use the Breakpoint Windowbreakpoint:

-	Choose the Set button, or

-	Choose the Breakpoints/Set Breakpoint...setbreakointdialog menu item.

-	Use the Source Windowsource:

1.	Move the cursor to the left screen margin.

2.	The cursor changes to a bull's eye.

3.	Click the left mouse button to set a permanent breakpoint, the right mouse button to set a temporary breakpoint.

4.	The line is highlighted in red to indicate a breakpoint.

-	Use the Shell Windowshell:

-	Use the bkpt Shell commandbkptshell.

To clear a breakpoint:

-	Use the Breakpoint Windowbreakpoint:

-	Choose the Clear button, or

-	Choose the Breakpoint/Clear or Breakpoint/Clear All menu items.

-	Use the Source Windowsource:

1.	Move the cursor to the left side of the screen.

2.	The cursor changes to a bulls eye.

3.	Click the left mouse button on an existing breakpoint line.

4.	The red highlight is removed to indicate the breakpoint is cleared.

-	Use the Shell Windowshell:

-	Use the bkptClear Shell commandbkptclearshell.

Breakpoints can be temporarily disabled, then re-enabled.  Use the Breakpoint Window's Enable, Disable, Enable All, or Disable All buttons or the Breakpoints menu items.

# # $ K Bkpt Shell Command

Set a breakpointbreakpointterm.

The Breakpoint Window'sbreakpoint Set Button and Breakpoints/Set Breakpoint... are equivalent to this command.

Syntax

bkpt [enable | disable] [temporary | permanent] [<address> ] [@<id> ] [<space>]

Parameters

enable	Enable breakpoint.

disable	Disable breakpoint.

temporary	Breakpoint is temporary.  It is removed when reached.

permanent	Breakpoint is permanent.  It is not removed when reached.

address	Address of breakpoint.

id	Number to identify breakpoint.

<space>	smm or user.  User is the default.

smm	Set breakpoint in SMM space.  Up to 4 total breakpoints can be set in SMM space.

user	Set breakpoint in User space.  Up to 4 breakpoints can be set in user space.

Examples

Display breakpoints.

> bkpt

Set a breakpoint and display it.

> bkpt #main; bkpt

Disable the breakpoint and display it.

> bkpt disable @0; bkpt

Create a temporary breakpoint in SMM space and display it.

> bkpt temporary #qsm#QMCR_smm; bkpt

Start emulation until permanent breakpoint reached and display breakpoints.

> go; while($EMULATING){}; bkpt

Go to the temporary breakpoint and check that it is removed.

> go; while($EMULATING){}; bkpt

Clear all breakpoints

> bkptclear all

See also

BkptClear Shell Commandbkptclearshell

Shell Syntax Conventionsshellsyntaxconventions

# # $ K BkptClear Shell Command

Clear one or all breakpointsbreakpointterm.

The Breakpoint Window'sbreakpoint Clear Button and Breakpoints/Clear and Breakpoints/Clear All are equivalent to this command.

Syntax

bkptClear @<id>|all

Parameters

id	Number given or assigned when breakpoint created with bkpt Shell commandbkptshell.

all	Remove all breakpoints.

Examples

Create a breakpoint and display it.

> bkpt #main; bkpt

Clear a breakpoint by id.

> bkptclear @1; bkpt

Clear all breakpoints.

> bkptclear all; bkpt

See also

Bkpt Shell Commandbkptshell

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Breakpoint

A breakpoint is a specified address where emulationemulationterm halts.  The CS:EIP pcmarkerterm points to the breakpoint address (before executing the instruction at that address).

A software breakpoint is always used if the memory is read/write (RAMramterm).  A hardware breakpoint is used for ROMromterm.

PowerPack implements hardware breakpoints in real time. PowerPack has 8 hardware breakpoints.  You can set up to 4 hardware breakpoints in user space and up to 4 hardware breakpoints in SMM space, for a total of 8 hardware breakpoints.  If you have hardware breakpoints set in both spaces, there is a few millisecond delay when switching between spaces.

A permanent breakpoint stops emulation whenever the address is reached.  A temporary breakpoint is cleared when it is reached.

Breakpoints are set:

-	From the Breakpointbreakpoint Window.

-or-

-	From the Sourcesource Window.  Move the cursor to the left side of the window.  Click to set, or clear, a breakpoint on the line.

-or-

-	From the Shellshell window.  Use the bkpt Shellbkptshell command.

# # $ K Set Breakpoint Dialog Box

Choosing Breakpoints/Set Breakpoint... opens this dialog box:

1.	Select a module and a function within that module.

2.	Select the state (enable or disable).

3.	Select the type (permanent or temporary).

4.	Select the space (User or SMM).

5.	Select the Set button.

6.	Repeat steps 1-3 to set multiple breakpoints

7.	Choose Close to make changes.

Modules	List of modules.

Functions	List of functions within the selected module.

Enable	Initial state is active.

Disable	Initial state is inactive.

Permanent	The breakpoint remains in the list after it is reached.

Temporary	The breakpoint is cleared from the list once it is reached.

User	The breakpoint is set in User space.

SMM	The breakpoint is set in System Management Mode space.

See also

Setting and Clearing Breakpointssettingandclearing

$ Breakpoint Window

# breakpoint

+ powerviews:0030

K breakpoint;window

$ Setting and Clearing Breakpoints

# settingandclearing

+ usingthebreakpoint:0010;usingthesource:0090

K breakpoint;source

# bkptshell

$ Bkpt Shell Command

K breakpoint;bkpt;shell, breakpoint;

# bkptclearshell

$ BkptClear Shell Command

K breakpoint;bkpt clear;shell, breakpoint;

$ Breakpoint

# breakpointterm

K breakpoint

# setbreakointdialog

$ Set Breakpoint Dialog Box

K set breakpoint;breakpoint;dialog box