# $ # + K Trigger Window

The Trigger Window allows bus eventseventterm to cause triggertriggerterm actions.  Triggers allow the detection of real-time bus activity and the making of a real-time decision based on the activity.

Screen Description

The left side sets the conditionselectingtriggerconditions to trigger on.  The right side is the trigger actionselectingtriggeractions to perform when the condition is true.

Menu Selections

File

Exit	Close the Trigger window.

Edit

Events...	Go to the Event Windowevent.

Options

Trace Control...	Configure trace parameterstracecontroldialog.

Counter	Use two 10-bit countersusingcountersand.

Timer	Use the timerusingcountersand.

Bus	Trigger at bus cyclebuscycleterm level.

Clock	Trigger at clock cycleclockcycleterm level.

Level

Show Level 0	Display actions for sequence level 0.  The conditions are the same for all levels.  The actions are different.

Show Level 1	Display actions for sequence level 1.

Show Level 2	Display actions for sequence level 2.

Show Level 3	Display actions for sequence level 3.

Windows	Move between windowsmovingbetweenwindows

Help	On-line helponlinehelp

Using the Trigger Window

Trace and Trigger Fundamentalstraceandtriggerfundamentals

Creating Eventscreatingevents

Selecting Trigger Conditionsselectingtriggerconditions

Selecting Trigger Actionsselectingtriggeractions

Using Multiple Trigger Levelsusingmultipletrigger

Using Counters and Timersusingcountersand

External Trigger In and Outexternaltriggerin

Clock vs. Bus Mode Triggeringclockvsbus

Trigger Examplestriggerexamples

# $ # + K Trace and Trigger Fundamentals

The trace features are:

-	128k or 256k trace frames.

-	Trace is collected on each processor clock cycleclockcycleterm.

-	Display trace as clock cycles, bus cyclesbuscycleterm, and disassembled instructions.

-	Synchronize trace frames with source using the linked cursorlinkedcursor.

-	1 to 256 buffers.  Multiple buffers allow collection of several copies of similar code or to trap separate portions of code execution.

-	Trigger trace collection within buffers as:

-	Pre to collect cycles before the triggertriggerterm.

-	Center to collect an equal amount before and after the trigger.

-	Post to collect the trigger point and cycles after the trigger.

-	Trace control programming can halt emulationemulationterm when the last trace buffer is filled.

-	Trace search allows eventeventterm pattern recognition.

The trigger features are:

-	8 eventseventterm or combinations of address range, data range, and processor status.

-	4 levels of sequencingselectingtriggeractions.  Each level has unique actionsselectingtriggeractions.  The 8 events are the same for all levels.

-	2 counters combine with the events to cause actions when a user-specified count is reached.

-	1 timer, in place of the counters, measures clock cycles.  The timer can trigger actions when it reaches a certain value.

Use trace to:

-	Record the real-time execution of code.

-	Linklinkedcursor the trace execution to the source.

-	View processor status pins.

Use triggers to:

-	Stop emulation on an eventevent.

-	Control collection of trace.

-	Detect complex sequencesselectingtriggeractions of events.

-	Detect a subroutine that is taking too much timeusingcountersand.

-	Count the number of times a subroutine is called before performing an action.

See also

Event Windowevent

Trace Windowtrace

Trigger Windowtrigger

# # $ K + Triggering on Access to a C Variable

To trigger on a variable:

1.	In the Event Windowevent, create an eventcreatingevents:

a.	Enter the symbolic name in the address start field.

b.	Enter a specific data value or leave the data fields blank to trigger on any value.

c.	Select 1 in the r/w- field for trigger on read, 0 for write, or leave as X to trigger on a read or a write.

2.	In the Trigger Windowtrigger, set the trigger:

a.	Choose the event name from the event name list.

b.	Click on enable to activate the event.

c.	Click on brk to halt emulation when the event occurs.  Choose other actions as appropriate.

3.	Start emulationemulationterm:

a.	Emulation stops when the trigger is detected.

See also

Trace and Trigger Fundamentalstraceandtriggerfundamentals

Symbol Entry Syntaxsymbolentrysyntax

# $ # + K Selecting Trigger Conditions

The Trigger Windowtrigger conditions are:

-	0 to 8 events.  Use the Edit/Events... menu option to create events.

-	Cnt0 and Cnt1 are the counter values if Options/Counter chosen.

-	Tmr is the timer value if Options/Timer chosen.

-	Ext in the bottom left is the external trigger inputexternaltriggerin.

-	Select enable to activate an event, counter, timer, or the external trigger.

If the ext column is selected, the condition must be satisfied and the external trigger input must be low for the action to occur.

Event conditions are verified in parallel at each clockclockcycleterm or busbuscycleterm cycle.  All qualified actions are executed, even if multiple events are satisfied.

See also

Selecting Trigger Actionsselectingtriggeractions

# $ # + K Selecting Trigger Actions

The Trigger Windowtrigger actions are:

-	Seq to sequence to the next level.  The next level can contain the same or different conditions and actions.

-	Rst to reset the sequence level to level 0.

-	Brk to break or stop emulationemulationterm.

-	Toff to turn trace off.  This occurs:

-	In pre-trigger mode, trace is turned off immediately.

-	In center trigger mode, trace is turned off after enough data has been collected such that the trigger is close to the center of the trace buffertracebufterm.

-	In post trigger mode, trace is turned off after enough data has been collected such that the trigger is near the beginning of the trace buffer.

-	In all cases (pre, post, or center) the Toff action is ignored if a trigger has already occurred in this trace buffer.

-	Next performs a Toff and then starts collecting trace in the next buffer.

-	With Options/Counters chosen:

-	Inc0 increments the first counter by 1.

-	Rst0 sets the first counter to 0.

-	Inc1 and Rst1 are for the second counter.

-	With Options/Timer chosen:

-	Start starts the timer.

-	Stop stops the timer.  The timer value is suspended.  It can be started again.

-	Reset sets the timer value to 0.

-	Ext lo sets the external trigger outputexternaltriggerin logic level to 0.

-	Ext hi sets the external trigger output logic level to 1.

See also

Selecting Trigger Conditionsselectingtriggerconditions

# $ # + K Using Multiple Trigger Levels

Four trigger levels allow the real-time detection of complex eventseventterm

An example of multiple levels is:

-	There are two functions that contain calls to the same subroutine.

-	Halt emulation if the subroutine is called from the one function, but not the other.

-	The subroutine may or may not be called each time the functions are executed since the subroutine is in conditional code.

To program the Trigger Windowtrigger to accomplish this:

-	Create an eventcreatingevents called breakfunction for the subroutine being called.

-	Create an event called goodcaller with the address of the caller that causes the break.

-	Create a third event called badcaller with its address.

-	Create a fourth event called goodcallerreturn with the return address of the goodcaller function.

-	On level 0:

-	Breakfunction is not enabled.

-	Goodcaller has seq set to sequence to the next level.

-	Badcaller is not enabled.

-	Goodcallerreturn is not enabled.

-	On level 1:

-	Breakfunction has brk set to stop emulation.

-	Goodcaller is not enabled.

-	Badcaller has rst set to return to level one if it is called before Breakfunction can break.

-	Goodcallerreturn has rst set to return to level one if Breakfunction not called.

Use the Trigger Window'strigger Level menu to change the level, which is displayed in the window title bar.

See also

Selecting Trigger Actionsselectingtriggeractions

Selecting Trigger Conditionsselectingtriggerconditions

# # $ K + Using Counters and Timers

Use the Trigger Window'strigger Options/Counter or Options/Timer menu options to choose  two 10-bit counters or one 20-bit timer.   The maximum value of a counter is 1023; of a timer, 0xfffff.

Counters are incremented and reset one of the trigger actionsselectingtriggeractions.  Use the counters to perform an action after a condition occurs a certain number of times.  For example:

-	Stop emulation on the 7th iteration of a for loop.

-	Program the trigger:

-	Create an eventcreatingevents with the address of a line within the for loop that is fetched each time.

-	Select the event in the Trigger Window's event name conditionselectingtriggerconditions, and select enable and inc0.

-	Set the cnt0 condition to 7, and select the enable and the brk boxes.

The timer starts and stops as part of the trigger actionselectingtriggeractions.  Use the timer to detect when something is taking too long to execute.  For example:

-	Stop emulation if a subroutine takes more than 1 millisecond.

-	Program the trigger:

-	Create an eventcreatingevents with the starting address of the subroutine.

-	Create another event for the return address of the subroutine.  If there is more than one return, use up to 7 events.

-	Select the starting address event with enable, start, and reset selected.

-	Select the ending address event with enable and stop selected.

-	For a processor clock rate of 16.778 MHz, select the tmr condition with 16778 as the value and enable and brk selected.  (16778 is calculated for 16.778 MHz;  i.e. there are 16778 clock cycles in one millisecond).

See also

Selecting Trigger Actionsselectingtriggeractions

Selecting Trigger Conditionsselectingtriggerconditions

# $ # + K External Trigger-In and Out

The Trigger Windowtrigger allows the Trigger-In BNC connection to be combined with eventseventterm and counter/timerusingcountersand conditionsselectingtriggerconditions to perform trigger actionsselectingtriggeractions.

External Trigger-In and Out BNC signals are TTL and CMOS level compatible.

The Trigger-Out BNC level is set with ext lo and ext hi as one of the trigger actionsselectingtriggeractions.  The initial output state is fixed at a high level.  The external trigger condition, when enabled, is recognized when the level on the Trigger-In BNC is low.  It is sampled once per clock in the clock trigger mode and once per bus cycle on the clock immediately following -AS going high at the end of the cycle in bus trigger mode.

# $ # + K Clock vs. Bus Mode Triggering

Clock trigger looks at the address, data, and processor status at each clock cycleclockcycleterm.

Busbuscycleterm trigger aligns the address, data, and processor status.  Since the address strobe, data strobe, and other status lines are not active at the same time, bus triggering allows combining these without concern about individual clock cycles.

The Trigger Window'strigger Options/Bus and Options/Clock select the mode.

See also

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # + K Trigger Examples

Many of the trigger subjects have examples.  Some additional ones are:

Stopping Emulation When Invalid Stack Area Accessedstoppingemulationwhen

Triggering on Access to a C Variabletriggeringonaccess

See also

Trigger Windowtrigger  for all the trigger subjects.

# $ # K Trigger

A trigger causes an action or actions to occur when one or more eventseventterm are matched on specified sequence levels.  Triggers allow the detection of real-time bus activity and the making of a real-time decision.

For example, "halt emulation when a variable is written" is a trigger.

The trigger point in the trace is controlled by the Trigger and Trace Windows' Options/Trace Control...menu item.  The modes are:

-	Pre to collect trace before the trigger.  The trigger is near the end of the buffer.

896 to 768 frames are collected before the trigger.

128 to 256 frames are collected after the trigger.

-	Post to collect trace after the trigger.  The trigger is near the beginning of the buffer.

128 to 256 frames are collected before the trigger.

896 to 768 frames are collected after the trigger.

-	Center to collect an equal amount before and after the trigger.  The trigger is near the center of the buffer.

About 512 frames are collected before and after the trigger.

# $ # K Trace Control Dialog Box

The fields are:

-	Halt When Last Trace Buffer Fullhaltwhenlast.  Sequencingselectingtriggeractions through multiple buffers is circular.  The last buffer sequences to the first buffer.  When there is only one trace buffer, the Halt When Last Trace Buffer Full is ignored.

-	The triggertriggerterm position within each trace buffer.

-	The number of trace buffers.  Use multiple buffers with the next trigger actionselectingtriggeractions to collect several versions of a specific section of code, or to isolate different events.

See also

Pre, Post, and Centered Trace Collectionprepostand

Multiple Trace Buffer Supportmultipletracebuffer

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # K Bus Cycle

Bus cycles are aligned address, data, and processor status events.  Multiple clock cyclesclockcycleterm are combined to form a snapshot of the address, data, and processor status.

# $ # K Clock Cycle

The CPU CLKOUT signal supplies the processor clock cycle to external devices.  The emulator uses this clock to sample the address, data, processor status lines, and I/O signals when collecting trace data.  The hardware triggers look at the signals on every clock cycle, even for processor wait states and during times when certain signals are not valid.

Trigger definitions at the clock and bus cyclebuscycleterm level are supported with the Trigger Window's Options/Bus and Options/Clock menu items.

# $ # K Trace Buffer

A trace buffer is a collection of program execution information.  You can partition trace into one or more buffers.

$ Trigger Window

# trigger

+ powerviews:0120

K trigger;window

$ Trace and Trigger Fundamentals

# traceandtriggerfundamentals

+ technical:0250

K trace;trigger;event

# triggeringonaccess

$ Triggering on Access to a C Variable

K trigger;C;variable

+ TECHNICAL:0260

$ Selecting Trigger Conditions

# selectingtriggerconditions

+ usingthetrigger:0020

K trigger

$ Selecting Actions

# selectingtriggeractions

+ usingthetrigger:0030

K trigger;trigger actions;actions

$ Using Multiple Trigger Levels

# usingmultipletrigger

+ usingthetrigger:0040

K trigger

# usingcountersand

$ Using Counters and Timers

K trace;timer;counter;trigger;

+ USINGTHETRIGGER:0050

$ External Trigger In and Out

# externaltriggerin

+ usingthetrigger:0060

K trigger

$ Clock vs. Bus Mode Triggering

# clockvsbus

+ usingthetrigger:0070

K trigger

$ Trigger Examples

# triggerexamples

+ usingthetrigger:0080

K trigger

$ Trigger

# triggerterm

K trigger;event trigger

$ Trace Control Dialog Box

# tracecontroldialog

K trace;trace control;dialog box

$ Bus Cycle

# buscycleterm

K bus cycle;trigger

$ Clock Cycle

# clockcycleterm

K clock cycle;trigger

$ Trace Buffer

# tracebufterm

K trace;