# # $ K + Event Window

The Event Window defines an eventeventterm for triggeringtrigger and searching the Trace windowtrace display.

Screen Description

The Event window is a template to define an event:

-	Active Event is a drop-down list box of all defined events.  Use the Edit menu to add or delete events.

-	Addr defines the address range.  Leave Addr blank to match any address combination.

-	Not defines a match outside of the range.

-	The end address can be absolute address or the number of bytes.

-	The mask uses 0's for don't care's.  For example, 0xff matches only the lower byte.

-	The range is inclusive (i.e., to trigger on 1xxx, use range 1000 to 1fff).

-	Data defines the data range.  Leave blank to match any data combination.

- 	The external In bits are:

-   TRC0, TRC1, TRC2, TRC3, TRC4, TRC5, TRC6 and TRC7.

-   Use low voltage(0), high voltage(1) or don’t care(X) buttons to toggle   external In bits.

-	The status codes are:

-   INST_FETCH (Instruction Fectch Cycle)

-   MEM_READ (Memory Read Cycle)

-   MEM_WRITE (Memory Write Cycle)

-   I/O_READ (Input Cycle)

-   I/O_WRITE (Output Cycle)

-   INT_ACK (Interrupt Acknowledge Cycle)

-   HALT (Halt Cycle)

-   DMA (DMA Cycle)

More than one status codes can be specified in an event setting.

Menu Selections

File

Save Events As...	Save event definitions to a file.

Restore Events...	Restore event definitions from a file.  These definitions are overwritten to existing events.

Exit	Close the Event window.

Edit

Add Event...	Create a new event.

Delete Event	Delete the Active Event.

Clear Event	Set the Active Event back to the default values of all fields don't cared.

Delete All Events	Delete all events.

Windows	Move between windowsmovingbetweenwindows

Help	On-line helponlinehelp

Using the Event Window

Creating Eventscreatingevents

Deleting Eventsdeleteevent

See also

Restore Events Dialog Boxrestoreeventsdialog

Save Events As Dialog Boxsaveeventsasdialog

Trigger Windowtrigger

Search Keywords

shell, event

# $ # + K Creating Events

## To Create an Event:

1.	Open the Event Windowevent.

2.	If no events are defined, the Edit/Add Event... dialog boxaddeventdialog appears.

3.	If events are defined, choose Edit/Add Event... to create a new one.

## To Move Between Events:

-	The Active Event box lists all the events.

-	Select an event from the drop-down list box.

## Examples of Events Are:

-	Writing to a single memory location.

1.	Set start address to the memory location.  The end address and mask default values are set up are filled-in if you click on another part of the window.

2.	Click on the radio button for the r/w- field.  Write is active low.

-	Executing outside of a function.

1.	Enter the name of the function in the Start Address field.

2.	Enter the end address of the function returned by the addressOf Shell commandaddressofshell.

a.	Movemovingbetweenwindows to the Shell Window.

b.	Enter addressOf and the function name.  For example:

AddressOf #main#main

c.	AddressOf displays the range.  The end address is the second number.

3.	Click on the Not box for outside the function.

4.	Click on r/w- 1 for read.

See also

Event Windowevent

Restore Events Dialog Boxrestoreeventsdialog

Save Events As Dialog Boxsaveeventsasdialog

Trigger Windowtrigger

# $ # K Event

An event is any combination of address, data, external in signals and status lines.  A match occurs when the processor bus cycle matches the specified combination.  Matching inside and outside of address and data ranges is supported.

# $ # K Save Events As Dialog Box

1.	Select the directory.

2.	Type the file name.

3.	Choose Open to create the file.  If the file already exists, it is overwritten.

The default event file extension is .evt.

See also

Restore Events Dialog Boxrestoreeventsdialog

Event Windowevent

# $ # K Restore Events Dialog Box

1.	Select the directory.

2.	Type the file name.

3.	Choose Open to open the file.  Delete the old Active Event list first. Then events are added to the Active Event list.

The default event file extension is .evt.

See also

Save Events As Dialog Boxsaveeventsasdialog

Event Windowevent

# $ # K Add Event Dialog Box

-	Type the new event name.

The Event windowevent displays default values for that event.

If the name already exists, the dialog box displays that event's current values.

Choose Edit/Clear Event to reset an event to the default values.

See also

Creating Eventscreatingevents

Event Windowevent

# # $ K EventSave Shell Command

Save events to a file.

Syntax

eventSave "<filename> "

Parameters

"<filename>"	saves event definitions to this file.

Examples

SaveEvent "events.dat"

See also

EventRestoreeventrestoreshell

# # $ K EventRestore Shell Command

Restore events from a file.

Syntax

eventRestore "<filename> "

Parameters

"<filename>"	restores event definitions from this file.

Examples

RestoreEvent "events.dat"

See also

EventSaveeventsaveshell

# # $ K Deleting Events

## To Delete An Event:

1.	Open the Event Window.

2.	The Active Event is shown in a drop-down list box.

3.	Choose Edit/Delete Event to delete the Active Event.

4.	Choose Edit/Delete All Events to delete all the events defined.

## To Move Between Events:

-	The Active Event box lists all the events.

-	Select an event from the drop-down list box.

## If the Delete Events Command Fails:

-	The event is probably being used in a trigger during emulation or trace. You can delete the event when emulation halts and no trigger uses it.

See also

Event Windowevent

Restore Events Dialog Boxrestoreeventsdialog

Trigger Windowtrigger

# event

$ Event Window

K event;window

+ POWERVIEWS:0045

$ Creating Events

# creatingevents

+ usingthetrigger:0010

K trigger

$ Event

# eventterm

K event;trigger event

$ Save Events As Dialog Box

# saveeventsasdialog

K event;save events;dialog box

$ Restore Events Dialog Box

# restoreeventsdialog

K event;restore events;dialog box

$ Add Event Dialog Box

# addeventdialog

K event;add event;dialog box

# eventsaveshell

$ EventSave Shell Command

K event;shell, event;

# eventrestoreshell

$ EventRestore Shell Command

K event;shell, event;

# deleteevent

$ Deleting Events

K events