# # $ K + CPU Window

The CPU Window displays registers and allows registers and signals to be modified.

Screen Description

The registers are shown with their values.  Double click on the value field to edit.

Menu Selections

Options

Reset CPU	Reset the processorresettingtheprocessor.

Signals	Enable or disable signalsenablinganddisablingsignals.

Help Index	MICEpack/SLD table of contents.

Help with Help	Using the help system.

Help with CPU	CPU Window table of contents.

Exit	Close the CPU Window

Using the CPU Window

Resetting the Processorresettingtheprocessor

Enabling and Disabling Signalsenablinganddisablingsignals

Warning on Changing Certain CPU Registerswarningonchanging

See also

CPU Shell Commandscpushellcommands

# $ # + K Warning on Changing Certain CPU Registers

If the program counter, the stack pointer, or the supervisor/user bit of the status register is changed, the source and stack context may be invalid.

The program counter is changed by the Source Window'ssource Run/Go From Cursor, the CPU Windowcpu, and the register Shell commandregistershell.

The stack pointer and status register are changed from the CPU Windowcpu or by the register Shell commandregistershell.

See also

Resetting the Processorresettingtheprocessor

# $ # + K Enabling and Disabling Signals

The signals for the MICEpack 68K series are:

68302 - AS, DS, Reset_HALT, CP_RUNNING, Berr, BrQ, Int, FRZ, DTACK_ex

68306 - AS, DS, Reset, HALT, Berr, BrQ, Int, DTACK_ex

68307 - AS, DS, Reset&&HALT, Berr, BrQ, Int, DTACK_ex

68328 - AS, DS, Reset, HALT, Berr, BrQ, Int, delay_strb, DTACK_ex

Enable a signal to connect the signal from the emulator probe head to the user's target.  A check mark indicates an enabled signal.

Disable a signal to connect the signal to a source in the probe head and disconnect it from the target.

Note that enable and disable control of signals AS and DS applies only to emulation under halt state.  Hence these signals are in effect only when MICEpack is not emulating.  When MICEpack is on running state, As and DS signals will remain transparent to the target system regardless of its enable/disable control status.

When DTACK_ext signal is enabled (default), emulation memory DTACK is asserted by target.  Otherwise, it is asserted by MICEpack.

The enable and disable control of signal CP_RUNNING (communication processor free-running) applies only to 68302 emulation under halt state.  When enabled, MICEpack 68302 will assert a high level signal to FRZ~ pin of MC68302 under halt state.  When SCR’s (System Control Register) bits FRZ1, FRZ2, and FRZW are set to freeze Timer1, Timer2, and Watchdog Timer respectively, the CP_RUNNING signal has to be at disable mode (CP_RUNNING = LOW) to carry out the desired performance.  The schematic diagram below shows the interface relations between target and emulation processor (68302 CPU) FRZ signals.

Use the CPU Window'scpu Options/Signals or the signal Shell commandsignalshell to enable and disable signals.

# $ # + K Resetting the Processor

To reset the processor, choose one of the following:

-	The CPU Window'scpu Options/Reset CPU menu option.

-	The Source window'ssource Run/Reset menu option.

-	The Toolbar’ssource Configure/Reset menu option.

-	The reset Shell commandresetshell.

A reset causes the following actions:

-	The processor reset pin is asserted.

-	The program counter and stack pointer values are read from memory.

-	All open windows are updated.

The consequences of reset are:

-	The stack is reset so the Stack Window contents are invalid.

-	The Source Windowsource points at the start of code which probably has no source.

If the reset command fails:

-	Reset the processsor only by choosing one of the following:

-	CPU Window'scpu Options/Reset CPU only.

OR

-	Toolbar window’ssource Configure/Reset CPU only.

OR

-	Enter the reset cpuonlyresetshell shell command.

-	Initialize the program counter and the stack pointer to valid values using the CPU Windowcpu or the register Shell commandregistershell.

-	Reset the processor and update the windows by choosing one of the following:

-	CPU Window'scpu Options/Reset

OR

-	Source Window’source Run/Reset

OR

-	Toolbar’s.Configure/Reset

OR

-	Enter the reset Shell commandresetshell.

# # $ K Register Shell Command

Display and change CPU registers.

Syntax

register [<register name> [value]]...

Parameters

register name	The name of a register, such as SR, PC, D0, D1, ..., D7, A0, A1, ..., A7, USP, or SSP for 68000 processor.

value	The new value for the register.  Be careful on changing certain registerswarningonchanging.

Examples

Display all registers.

> register

//     PC = 0x00000642

//     A7 = 0x00001DF6

//     A6 = 0x00000000

//     D0 = 0x00000020

//     D1 = 0x0000001E

//     D2 = 0xFFFFFFFF

//     D3 = 0xFFFFFFFF

//     D4 = 0x00008490

//     D5 = 0xFFFFFFFF

//     D6 = 0xFFFFFFFF

//     D7 = 0x00008490

//     A0 = 0x00000BA1

//     A1 = 0x00FFFD20

//     A2 = 0x00000B84

//     A3 = 0xFFFFFFFF

//     A4 = 0xFFFFFFFF

//     A5 = 0xFFFFFFFF

//     SR =     0x2700

//        = 0S7xnzvc

//    SSP = 0x00001DF6

//    USP = 0x30403A30

Change a register value and display the new value.

> register d3 0x12345678; register d3

//     D3 = 0x12345678

See also

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Reset Shell Command

Reset the processorresettingtheprocessor.

The CPU Window'scpu Options/Reset CPU is equivalent to this command.

Syntax

reset [cpuonly]

Parameters

cpuonly	Reset the processor and do not update windows.  Normally, all windows are updated on a reset.  If the program counter or the stack pointer memory contents is invalid at reset, the processor can hang again while updating these windows.  Use cpuonly in this case and then, set valid program counter and stack pointer values.  Issue reset again to update windows.

Examples

See also

Resetting the Processorresettingtheprocessor

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Signal Shell Command

Enable and disable signalsenablinganddisablingsignals.

The CPU Window'scpu Options/Signals is equivalent to this command.

Syntax

signal [<signal name> [enable | disable]]

Parameters

signal name	The signal name:  clk or reset.

enable	Enable the signal.

disable	Disable the signal.  The processor does not see the signal.

Examples

Display current signal values.

> signal

// AS-        ENABLE

// DS-        ENABLE

// RESET        DISABLE

// HALT        DISABLE

// BERR        DISABLE

// BRQ       DISABLE

// INT       DISABLE

// DTACK_ext     ENABLE

Disable the reset line and display the new value.

> signal reset disable; signal reset

// RESET  DISABLE

// RESET  DISABLE

See also

Shell Syntax Conventionsshellsyntaxconventions

# $ # K CPU Shell Commands

CPU related commands are:

registerregistershell [<register name> [value]]...

resetresetshell [cpuonly]resetshell

signalsignalshell [<signal name> [enable | disable]]

See also

Shell Command List (alphabetic)shellcommandlist

Shell Command List (topical)shellcommandlisttopical

Shell Syntax Conventionsshellsyntaxconventions

# cpu

$ Warning on Changing Certain CPU Registers

K cpu;registers;signal;reset;

+ POWERVIEWS:0040

$ Warning on Changing Certain CPU Registers

# warningonchanging

+ usingthecpu:0010

K cpu;register

$ Enabling and Disabling Signals

# enablinganddisablingsignals

+ usingthecpu:0020

K signal;enabling signals;disabling signals

$ Resetting the Processor

# resettingtheprocessor

+ usingthesource:0120

K reset;chip;initialization;cpu

# registershell

$ Reset Shell Command

K cpu;registers

$ Reset Shell Command

# resetshell

K cpu;reset;

$ Signal Shell Command

# signalshell

K cpu;signal;reset;clk;

$ CPU Shell Commands

# cpushellcommands

K cpu;shell