# $ # + K Solutions to Common Problems

Can't establish communications with MICEpackcantestablishcommunications

Can't communicate with probecantcommunicatewith

Can't load codecantloadcode

Can't write memorycantwritememory

Can't access target memorycantaccesstarget

Unterminated Memory Cycleunterminatedmemorycycle

Can't start or stop emulationcantstartor

Not reaching breakpointnotreachingbreakpoint

Not getting triggernotgettingtrigger Getting false triggergettingfalsetrigger

Not collecting tracenotcollectingtrace

See also

Error Listerrorlist

Recommended Readingrecommendedreading

# $ # + K Error List

1.	Choose the Help window Search button.

2.	Type error to see a list of all the MICEpack/SLD errors.

3.	Select an error.

4.	Choose the GoTo button to see the error help.

# $ # + K Can't Establish Communications

Host can not establish communications with MICEpack.

1.	Verify that all external cable connectors are securely connected.

2.	Make sure the MICEpack is turned on and the Power LED is lit.

3.	Make sure that the communication link setup selection corresponds with the type of host interface in use, i.e.-

When MS-PCE is used, check your MP Setup dialog box (by double clicking the “MP/Setup” icon from Program Manager) for correct setup or as follows:

CommType:  MicrotekParallel
	BasePort:  0x200

When MIF-KEY is used, check your MP Setup dialog box (by double clicking “MP/Setup” icon from Program Manager) for correct setup or as follows:

CommType:  LPT1 (or LPT2/LPT3)

See also

Device already open.HE_OPEN

Timed out waiting for hardware to respond.HE_TIMED_OUT

Cannot open the connection.HE_COM_OPEN

Communication time-out sending to box.HE_COMM_PACKET_TIMEOUT

Excessive errors in communication from host to box.HE_COMM_RETRY_EXHAUSTED

Timed out waiting for emulation to stopHE_WAIT_EMOFF_TIMEOUT.

Timed out waiting for single step.HE_STEP_TIMEOUT

# $ # + K Can't Communicate with Probe

-	Check cables between ICE probe and chassis.

-	Check proper connection of emulator to target.  Try reseating the adapter where it connects to the target.

-	Make sure target power is turned on.

See also

Cannot initialize probe.HE_CANT_INIT_PROBE

Probe transmission error.HE_ILLEGAL_CMD

Timed out waiting for hardware to respond.HE_TIMED_OUT

Communication time-out sending to box.HE_COMM_PACKET_TIMEOUT

Excessive errors in communication from host to target.HE_COMM_RETRY_EXHAUSTED

# $ # + K Can't Load Code

Loading a program is a two-step process:

1.	Downloading code to overlay or target memory

2.	Loading symbols to the symbol table.

Problems can occur during code download if:

-	You did not map overlay memory in the region your program expects to be loaded into.

-	You did not map sufficient memory.

These problems result in Memory verification errors.  To fix:

-	Verify the overlay mapmappingmemory addresses and size, if present.

-	You did not specify the toolchain you are using.

To fix, use the compilerUsedcompilerusedshell Shell command.

If you are using a non-supported toolchain, you may be able to load and debug your code, but please contact customer supportcontactingmicrotek so we can better serve your needs.  Warnings on loading do not prevent a successful load of code or symbols.  Turn off warnings with the Shell load nowarnloadshell option, or from the Load Options dialog boxloadoptionsdialog.

If necessary, load code only by turning off symbol loading with the Shell Load nosymloadshell option, or turn off symbol loading from the Load Options dialog boxloadoptionsdialog.  Then, debug at the disassembly level.

See also

Cannot open load file.HE_CANNOT_OPEN

Out of memory during load.HE_NO_MEMORY

Module time stamp is later than original load file.HE_MODTIME

Cannot load code while processor running.HE_NORUNACCESS

Symbols already loaded, specify 'reload' option.HE_SYM_ALREADY_CLI

Incompatible load file processor type: <processor>.HE_INCOMPATIBLE_CPU

Loader configuration file corrupted.HE_CONFIG_FILE_FORMAT

Load Aborted.HE_LOAD_ABORT

Bad ATI record in Loadfile.HE_BAD_ATI_RECORD_IN_FILE

Cannot seek load file.HE_BAD_SEEK

Loadfile compiled using <compiler> tool is incompatible with the current Compiler Used.HE_LOADFILE_HAD_INCOMPATIBLE_TOOLCHAIN

No symbols are loaded.HE_SYMBOL_NOT_A_MODULE

# $ # + K Can't Write Memory

1.	Make sure a chip selects is programmed for this memory location.

2.	Make sure your chip select is not set to read-only.

3.	If you are using overlay memory, make sure the overlay map is set up for this memory location.

4.	Make sure you are not trying to write outside target memory.

5.	Make sure that the processor clock is not too slow.

6.	Make sure that memory access size is set correctly for the memory being written (see the Size Shell commandsizeshell).

7.	Make sure you are writing to valid processor memory; some peripheral address ranges are non-existant.

See also

No memory available to store breakpoints.HE_BKPT_MEM_FAIL

Unable to read memory.HE_DASM_MEMORY

No map RAM installed on probe.HE_NO_MEMORY_PRESENT

Cannot read mapped memory type.HE_OVERLAY_HARDWARE

Bus error during memory cycle.HE_BUS_ERROR

Unterminated memory cycle; internal termination failed.HE_BUS_TIMEOUT

Memory verification failed.HE_MEMORY_VERIFY

Unterminated memory access; internal termination used.HE_INTERNAL_TERMINATION

# $ # + K Can't Access Target Memory

1.	Make sure you are not trying to write outside of target memory.

2.	Make sure the chip selects are set correctly.

3.	Make sure that the processor clock is not too slow.

4.	Make sure that memory access size is set correctly for the memory being written (see the Size Shell commandsizeshell).

See also

No memory available to store breakpoints.HE_BKPT_MEM_FAIL

Unable to read memory.HE_DASM_MEMORY

No map RAM installed on probe.HE_NO_MEMORY_PRESENT

Cannot read mapped memory type.HE_OVERLAY_HARDWARE

Bus error during memory cycle.HE_BUS_ERROR

Unterminated memory cycle; internal termination failed.HE_BUS_TIMEOUT

Memory verification failed.HE_MEMORY_VERIFY

Unterminated memory access; internal termination used.HE_INTERNAL_TERMINATION

# $ # + K Unterminated Memory Cycle

1.	Enable bus monitor.

2.	Verify chip select programming.

3.	A memory access to an unimplemented address occurred.  This could be either explicit or implicit (for example, RESET causes a read from the address contained in the PC register).

See also

Bus error during memory cycle.HE_BUS_ERROR

Unterminated memory cycle; internal termination failed.HE_BUS_TIMEOUT

Unterminated memory access; internal termination used.HE_INTERNAL_TERMINATION

# $ # + K Can't start or stop emulation

1.	The processor may be in a STOP state as a result of executing a STOP instruction.  In this state, RESET must be used to regain control of the processor.

2.	The processor may be waiting for bus termination.  Enable the bus monitor function so that unterminated cycles does not occur.

3.	Too many breakpoints may have been set.  (MICEpack allows 2 hardware and 254 software breakpoints.)  Clear breakpoints.

4.	The processor clock rate may be too slow.

5.	The target may be asserting RESET, preventing emulation from starting.

See also

Timed out waiting for emulation to stop.HE_WAIT_EMOFF_TIMEOUT

# $ # + K Not Reaching Breakpoint

1.	The program execution is not what was expected.  Use earlier breakpoints and single stepping to find why execution does not reach this address.

2.	The software breakpoint may be overwritten.  When emulation begins, a software breakpoint opcode is written into memory at breakpoint addresses.  If user code then overwrites this address, the breakpoint is overwritten.

See also

No breakpoint available.HE_BKPT_NONE_AVAILABLE

No breakpoint selected <action>.HE_BKPT_NONE_SELECTED

No breakpoint matches the specified address.HE_BX_NO_BKPT_MATCH_ADDR

The number of hardware breakpoints exceeds capability.HE_TOO_MANY_XBRKS

# $ # + K Not Getting Trigger

1.	Recheck trigger programming.  Verify that no unnecessary, and all necessary, fields are set.

2.	Check that the trigger is defined on its correct sequence level.

3.	Check that the correct code is executing by examining the execution history in the Trace windowtrace.

See also

Cannot change an event used by trigger while trace is on.HE_TRACING_ON

Cannot change an event used by trigger while emulatingHE_TRIGGER_USING_EVENT.

Cannot change trigger while tracing and/or sequencer is enabled.HE_TRACE_TRIG_ON

No active trigger found.HE_NO_TRIG_DEFINED

Invalid trigger event index.HE_BAD_EVENT

# $ # + K Getting False Trigger

1.	Processor's instruction prefetch is causing a trigger.

If an event specifies an address that is inside the processor's prefetch queue, a false trigger can occur, especially in loops.  That is:

nop

bne <top of loop>

afterloop:	jsr function

If the trigger event specifies #afterloop as the address to trigger on, the processor's prefetch queue causes this address to appear on the bus even though the jsr is not executed.

2.	Trigger sequencing has logical errors.  Reduce the complexity and/or number of triggers that cause the trigger level to change.  Also, reduce the number of triggers on the sequenced-to level.

3.	Target signal noise problems may generate bogus data which causes a trigger match.

See also

Cannot change an event used by trigger while trace is on.HE_TRACING_ON

Cannot change an event used by trigger while emulating.HE_TRIGGER_USING_EVENT

Cannot change trigger while tracing and/or sequencer is enabled.HE_TRACE_TRIG_ON

No active trigger found.HE_NO_TRIG_DEFINED

Invalid trigger event index.HE_BAD_EVENT

# $ # + K Not Collecting Trace

1.	Verify correct trigger programming.  Select the toff action check box to turn off trace.

2.	Verify that the trace control configuration is correct for your application using the Trace Control dialog boxtracecontroldialog.

3.	Verify that your program is running.

See also

Invalid request with only one trace buffer.HE_SINGLE_TRACE_BUFFER

No trace collected in the next trace buffer.HE_TRACE_DATA

A NEXT action requires multiple trace buffer configuration.HE_MULTI_BUFS_NEEDED

No bus cycles collected in trace buffer.HE_DQ_NO_FRAMES_AVAILABLE

Invalid trace buffer.HE_INVALID_BUFFER

# $ # K Address missing.

Cause:	The GoTo, Start, From, or To address is not specified correctly.

Solution:	Enter a value (0x1234) or a symbol (#main) in the field.

See also

Symbol Entry Syntaxsymbolentrysyntax

# $ # K Address cannot start on an odd boundary.

Cause:	The program counter specified is not an even value.

Solution:	Enter an even value.  If a symbol with an odd address specified, fetch the value with the addressOf Shell commandaddressofshell, then enter the closest even address.

# $ # K No memory available to store breakpoints.

Cause:	Could not add breakpoint to list.

Solution:	Window ran out of memory.  Close other applications or other MICEpack/SLD windows.

See also

Breakpoint Windowbreakpoint

# $ # K No breakpoint available.

Cause:	Enabling, disabling, or clearing all breakpoints.

Solution:	There are no breakpoints active.  View breakpoints from the Breakpoint windowbreakpoint

See also

Breakpoint Windowbreakpoint

Source Windowsource

# $ # K No breakpoint selected <action>.

Cause:	Attempt to delete, disable, enable, or go to the source of a breakpoint when none is selected.

Solution:	Click on a breakpoint line.  Then, choose the action.

See also

Breakpoint Windowbreakpoint

# $ # K Cannot get total bytes in file.

Cause:	Could not access file length when opening a file.

Solution:	The file length is estimated and processing continues.  Subsequent errors are possible since there is a problem accessing the file.

See also

Source Windowsource

# $ # K Unable to open source file <file name>

Cause:	The path specified does not exist.

Solution:	Check the path and enter the correct value.  On a network, read permission may not be set.

See also

Source Windowsource

# $ # K Unable to open UNIX source file.

Cause:	The path specified does not exist.

Solution:	Check the path and enter the correct value.  On a network, read permission may not be set.

See also

Source Windowsource

# $ # K Cannot reopen file.

Cause:	A file that was once opened cannot be reopened.

Solution:	The file has been deleted,or permission to the file or its directory changed.

See also

Source Windowsource

# $ # K Cannot reopen mixed data source.

Cause:	A file that was once opened cannot be reopened.

Solution:	The file has been deleted, or permission to the file or its directory changed.

See also

Source Windowsource

# $ # K Invalid decimal constant.

Cause:	The decimal number is incorrect.

Solution:	Start decimal numbers with 1 to 9.  Octal numbers start with 0.

# $ # K Invalid hexadecimal constant.

Cause:	The hexadecimal number is incorrect.

Solution:	Start hexadecimal numbers with 0x.

# $ # K Invalid octal constant.

Cause:	The octal number is incorrect.

Solution:	Start octal numbers with 0.  Decimal numbers start with 1 to 9.

# $ # K Command too long: <size>.

Cause:	The Shell command is too long.

Solution:	Do not exceed the  5500 character limit.

See also

Shell Windowshell

# $ # K A command is being executed; hit escape to terminate command.

Cause:	Attempting to close the Shell Window while a command is still executing.

Solution:	Wait for the command to complete or press the Escape key to terminate the command.

See also

Shell Windowshell

# $ # K File error: <type> failure on <file name>: access denied.

Cause:	Error during open, read, write, or close of a file.

Solution:	Change the access permission to the file.  On a network, check file and directory permissions.

# $ # K File error: <type> failure on <file name>: file not found.

Cause:	Error opening a file.

Solution:	Check the path and file name.  On a network, check file and directory permissions.

# $ # K Error opening file: <file name>.

Cause:	The file cannot be opened.

Solution:	Check the path and file name.  On a network, check file and directory permissions.

# $ # K File error: <type> failure on <file name>: path not found.

Cause:	An invalid path was specified.

Solution:	Check the path and enter text again.

# $ # K File error: <type> failure on <file name>: too many files open.

Cause:	Too many files are opened.

Solution:	Close other applications that are using files.

Or, increase the FILES= number in config.sys.  Exit MICEpack/SLD and re-boot PC to take affect.

# $ # K History size must be between 0 and 100.

Cause:	The Shell history size is out of range.

Solution:	Use 0 to disable the history feature, or 1 to 100 for the number of recall lines.

See also

Shell Windowshell

# $ # K Include file or alias definition nesting limit exceeded.

Cause:	The number of include files or the expansion of an alias exceeds the Shell limits.

Solution:	The limit is roughly 5500 characters.  Simplify the construction of the macro.

See also

Shell Windowshell

# $ # K Statement list too large: <line count>.

Cause:	The statement is over 150 lines.

Solution:	Simplify the construction of the macro so each statement is under 150 lines.  All lines within an if or while are considered one statement.

See also

Shell Windowshell

# $ # K Invalid variable: %s.

Cause:	The syntax of the variable is incorrect.

Solution:	Check the variable syntaxsymbolentrysyntax.

See also

Shell Windowshell

Variable Windowvariable

# $ # K Command syntax error.

Cause:	The syntax is incorrect.

Solution:	Use on-line help for the command or use the help Shell command (help <topic>) for the syntax description.

See also

Shell Syntax Conventionsshellsyntaxconventions

Shell Command List (alphabetic)shellcommandlist

Shell Command List (topical)shellcommandlisttopical

Shell Windowshell

$ # K Undeclared variable: $<name>.

Cause:	The Shell variable has not been declared.

Solution:	Create the Shell variable.shellvariableterm

See also

Shell Windowshell

# $ # K Unterminated comment.

Cause:	The closing portion (*/) of the comment is missing.

Solution:	Comments begin with /* and terminate with */.  The C++ comment convention (//) comments the rest of the line.

See also

Shell Windowshell

# $ # K Unterminated string.

Cause:	The closing double quote (") is missing.

Solution:	Strings begin and end with double quotes.  Use quotes when spaces or non alphanumeric characters are used.

See also

Shell Windowshell

# $ # K Command aborted by user.

Cause:	The Escape key was pressed to terminate the current command.

Solution:	Enter control-Up arrow to re-execute the command, if desired.

See also

Shell Windowshell

# $ # K The target processor is emulating.  Pre-emulation values shown.

Cause:	Invoking the CPU Window while emulatingemulationterm displays this warning.

Solution:	CPU registers are not updated while emulating.  The values displayed in the window are the values when emulation began.

Run accessrunaccessterm reads and writes memory while emulating, but does not affect registers.

See also

CPU Windowcpu

# $ # K Cannot set break on a non-opcode location.

Cause:	Attempting to set a breakpoint on the operand part of an assembly instruction results in this error if a long instruction spans multiple lines.  For example:

| Line # | Address | Opcode | Mnemonic |
| --- | --- | --- | --- |
| 1 | 00021C | CFBB DFF9 FFFF | AND.L D7,([-00002001,ZPC]) |
| 2 | 000222 | DFFF |  |

Line 2 is part of the instruction on line 1.  Setting a breakpoint on line 2 would corrupt the instruction on line 1.

Note: This error is issued from the Source window in either ASM or MIXED mode.

Solution:	None.  Information only.

# $ # K Invalid alias path.

Cause:	The alias path has an invalid entry.

Solution:	Check each path.  Use absolute path (\mydir).  Specify the directory if not the current default directory (e.g. d:\mydir).

See also

Source Windowsource

# $ # K Unable to read memory.

Cause:	The Source Window cannot read memory for disassembled instructions.

Solution:	Check the memory mapmappingmemory for valid memory.  Reset the processorresettingtheprocessor, if necessary.

See also

Source Windowsource

# $ # K No breakpoint to <action>.

Cause:	Breakpoint cannot be cleared, enabled, or disabled.

Solution:	A breakpoint is typically set on the line selected.  If the line is a comment, the next code line is selected.  On multiple line commands, the breakpoint appears on the last line.

Selecting a line at the end of the file causes this error.  There are no more code lines following the insertion point.

See also

Source Windowsource

# $ # K Failed to reopen a mixed mode data source.

Cause:	A file that was once opened cannot be reopened.

Solution:	The file has been deleted or permission to the file or its directory changed.

See also

Source Windowsource

# $ # K Invalid field name.

Cause:	The field contains unprintable characters.

Solution:	Use printable characters in name.

See also

Trigger Windowtrigger

# $ # K Field name already defined.

Cause:	The field name entered is already being used.

Solution:	Use a unique name.

See also

Trigger Windowtrigger

# $ # K File not found: <file name>

Cause:	The file was not found.

Solution:	Add the path of the file to the Source Window'ssource Options/Source Paths... menu option.

# $ # K Source path is invalid.

Cause:	The path entered is invalid.

Solution:	Check the path and use backslash (\) to delimit directories.  For example, \mydir\otherdir or d:\newdir\moredir

See also

Source Windowsource

# $ # K Invalid breakpoint specified.

Cause:	Text entered is incorrect.

Solution:	Enter a value (0x1234) or a symbol (#mod#func).

See also

Symbol Entry Syntaxsymbolentrysyntax

Breakpoint Windowbreakpoint

# $ # K Invalid input value.

Cause:	The stack size, base, or alarm limit is not a valid number.

Solution:	Enter a valid decimal (1234), hexadecimal (0x1234), or octal (0177) number.

See also

Stack Windowstack

# $ # K Invalid module symbol to select function.  Select the module first.

Cause:	A module was not selected before action.

Solution:	Select a module from the list first, then invoke action.

See also

Breakpoint Windowbreakpoint

# $ # K Line Number Missing.

Cause:	No line was entered.

Solution:	Enter a number (1234, 0x4444, 0177).

See also

Source Windowsource

# $ # K From and to addresses out of order.

Cause:	The start address is greater than the end address.

Solution:	Check the start and end addresses.  Set the start address to be less than or equal to the end address.

See also

Memory Windowmemory

# $ # K Non-numeric pattern.

Cause:	One or more elements in the data pattern are not numeric.

Solution:	Enter only numeric values (1234, 0x4444, 0177).

See also

Memory Windowmemory

# $ # K Save/restore error.  File is not in the correct format.

Cause:	Obsolete error.

Solution:	Contact Microtekcontactingmicrotek

# $ # K No symbolic information on the selected region.

Cause:	There are symbols in the source which do not have debug information.

Solution:	Either select another item which has debug information, or choose other compiler switches to add debug information for those symbols.

# $ # Frame not found.  Trace search failed.

Cause:	The search event was not found in the address range specified.

Solution:	This is an informational message.  Invoke trace search again with a new event, if desired.

See also

Trace Windowtrace

# $ # K Number does not match base.

Cause:	The input value is expected to be a numeric value for the timer or counter value of a trigger.

Solution:	Use C syntax for entering numeric values.  For decimal values, do not use any letters (just digits).  For hex values, prefix the value with "0x".

# $ # K Pattern missing.

Cause:	The search pattern is blank.

Solution:	Enter one or more numbers separated by blanks (12 0x44 017).

See also

Memory Windowmemory

# $ # K Scrolling requires accessing memory: <message>.

Cause:	Cannot disassembly memory in current source mode while emulatingemulationterm.

Solution:	Mixed and disassembly modessourcemixedand read memory.  Enable run accessrunaccessterm to read memory while emulating.

Use the Source Window'ssource View/Source Only menu option to look at source without reading memory.

# $ # K Invalid request with only one trace buffer.

Cause:	Goto buffer requires multiple trace buffers.

Solution:	Use the Trace Window'strace or Trigger Window'strigger Options/Trace Control... menu option to set the number of buffers.

# $ # K Source module is newer than load file.

Cause:	The current source file's date is newer than the current load file.

Solution:	Modifications to the source file since the last load causes line numbers and symbolic information synchronization problems.  Re-compile, re-link, and re-load the code.

See also

Source Windowsource

# $ # K Cannot find <search text>.

Cause:	The search did not find the text.

Solution:	In Source modesourcemixedand, the search is from the cursorcursorterm to the end of the file.  In Mixed and Disassembly modes, the search is from the cursor for a limited range.  It is limited since a search of all memory is impractical.  Use Edit/Search Next to search the next region.

See also

Source Windowsource

# $ # K Cannot go forward in file browse history.

Cause:	Browse historybrowsehistoryterm is at the most recent entry.

Solution:	Use File/Previous Browsed Module to back up in the browse history.

See also

Source Windowsource

# $ # K Function <name>:  Address starts at: <address>.

Cause:	The address of the function is displayed.

Solution:	This is an information message.

See also

Source Windowsource

# $ # K History depth must be in the range 5 to 100.

Cause:	The source browse historybrowsehistoryterm input is out of range.

Solution:	Set to a value from 5 to 100.

See also

Source Windowsource

# $ # K Invalid address.

Cause:	The Goto address specified is not valid.

Solution:	Enter a number (1234, 0x4444, 0177) or a symbol (#main).

See also

Symbol Entry Syntaxsymbolentrysyntax

Source Windowsource

# $ # K Invalid line number.

Cause:	The entry is not a valid number.

Solution:	Enter a number for the line (12, 0x44, 017).

See also

Source Windowsource

# $ # K <Feature> requires accessing memory: <message>.

Cause:	Inspecting a variable and switching to Mixed mode cannot be done while emulatingemulationterm.

Solution:	Enable run accessrunaccessterm to read memory while emulating or stop emulation.

See also

Source Windowsource

# $ # K Failed to open a new view of data source.

Cause:	Opening the file failed.

Solution:	Check the Source Window's  Option/Source Paths... for the directory of the source module.  On a network, check the permission to the file and its directory.

# $ # Failed to reopen a data source.

Cause:	A file that was once opened cannot be reopened.

Solution:	The file has been deleted or permission to the file or its directory changed.

See also

Source Windowsource

# $ # Cannot back up in file browse history.

Cause:	Browse historybrowsehistoryterm is at the oldest entry.

Solution:	Choose File/Browse History Depth... to expand the number of files in the history.

See also

Source Windowsource

# $ # K Invalid step count.

Cause:	The entry is not a valid number.

Solution:	Enter a valid number from 1 to 0xffffffff in decimal, hexadecimal, or octal.

See also

Source Windowsource

# $ # K Tab width is out of range.

Cause:	The value is outside the range of 1 to 32.

Solution:	Enter a value from 1 to 32.

See also

Source Windowsource

# $ # K View mode already selected.

Cause:	The current view modesourcemixedand is already active.

Solution:	Choose another Source Windowsource View option, if desired.

# $ # K Stack alarm limit exceeded.

Cause:	The stack alarmstackalarm exceeded the percentage set.

Solution:	This is an information message enabled with the Stack Window'sstack Options/Alarm Limit... or Options/Enable Alarm Limit.

# $ # K Alarm limit is out of range.

Cause:	The percentage entered is not from 1 to 100.

Solution:	Enter an alarm limitalarmlimitterm value from 1 to 100.

See also

Stack Windowstack

# $ # K Invalid stack size specified. Set to 1K.

Cause:	The initialization fileinitializationfile, MICEPACK.INI, [Stack Info] StackSize member contains an invalid number.

Solution:	MICEpack/SLD sets the stack size to 1024.  Use the Stack Window'sstack Options/Stack Size to change, which updates the initialization file.

# $ # K Stack overflow.

Cause:	A stack overflowstackoverflowmessage occurred.

Solution:	The stack pointer is outside the range of the stack.  Change the Stack Window'sstack Options/Base Address... and Options/Stack Size... menu options to reflect the correct stack values.

See also

Stack Overflow Messagestackoverflowmessage

# $ # K Stack size missing.

Cause:	The stack size input is blank.

Solution:	Enter a number for the stack size (1234, 0x4444, 0177).

See also

Stack Windowstack

# $ # K Requested stack size is too large. Set to 1K.

Cause:	The maximum stack size is 0x80000000.

Solution:	The stack size is set to 1024.  Enter a new value between 0 and 0x80000000, if desired.

See also

Stack Windowstack

# $ # K Stack underflow.

Cause:	The stack pointer is above the top of stack.

Solution:	A programming error is possible.  Or, change the Stack Window'sstack Options/Base Address... and Options/Stack Size... menu options to reflect the correct stack values.

See also

Stack Overflow Messagestackoverflowmessage

# $ # K No trace collected in the next trace buffer.

Cause:	Cannot search into next trace buffer since there is no data collected.

Solution:	Change the search parameters to exclude empty buffers.  Use the Trace Window's Go To menu options to check buffers for data.

See also

Trace Windowtrace

# $ # K Disabling linked cursor.

Cause:	The linked cursorlinkedcursor is disabled while emulatingemulationterm since the Source Windowsource reads memory.

Solution:	Enable run accessrunaccessterm to allow memory reads during emulation or halt emulation.

See also

Trace Windowtrace

# $ # K Cannot find function: <name>.

Cause:	The function name shown is not in the symbol table.

Solution:	The two possibilities are:

1.	If entered in the Shell or a dialog box, the name is misspelled or has the wrong syntaxsymbolentrysyntax.

2.	The symbol type is not supportedcandc.

Use the displaySymbols Shell commanddisplaysymbolsshell for a list of symbols.

See also

Variable Windowvariable

# $ # K Cannot find module: <name>.

Cause:	The module name shown is not in the symbol table.

Solution:	The two possibilities are:

1.	If entered in the Shell or a dialog box, the name is misspelled or has the wrong syntaxsymbolentrysyntax.

2.	The symbol type is not supportedcandc.

Use the displaySymbols Shell commanddisplaysymbolsshell for a list of symbols.

See also

Variable Windowvariable

# $ # K Cannot find variable: <name>.

Cause:	The variable name shown is not in the symbol table.

Solution:	The two possibilities are:

1.	If entered in the Shell or a dialog box, the name is misspelled or has the wrong syntaxsymbolentrysyntax.

2.	The symbol type is not supportedcandc.

Use the displaySymbols Shell commanddisplaysymbolsshell for a list of symbols.

See also

Variable Windowvariable

# $ # K Symbols cannot end with #.

Cause:	The symbol entered ends with a #.

Solution:	Enter the proper symbol syntaxsymbolentrysyntax.

See also

Variable Windowvariable

# $ # K Select highlighted text.

Cause:	The selected text cannot be edited.

Solution:	Double click on a valid variable edit field.

See also

Variable Windowvariable

# $ # K Variable: <name> is at address <address>.

Cause:	The variable name and its address are displayed.

Solution:	This is an informational message.

See also

Variable Windowvariable

# $ # K No selected variable to delete.

Cause:	No variable is highlighted for deletion.

Solution:	Double click on a variable and then delete it.

See also

Variable Windowvariable

# $ # K No more variables to undo.

Cause:	All deleted variables are restored.

Solution:	This is grayed out when no undo.

# $ # K Token keyword not found.

Cause:	Syntax error.

Solution:	Look for the syntax description for the command you are using.

See also

CPU Shell Commandscpushellcommands

CPU Windowcpu

# # $ K Network communications option not compatible with host software

Cause:	The MICEpack/SLD and Network Software version numbers do not match.

Solution:	Check the version number of MICEpack/SLD, which is on its disk label, in the README file, or obtainable with the Shell versionversionshell  command.  Check the version number of the network code, which is on its disk label and in the README file.

Install the latest version of MICEpack/SLD or the network software.

$ Solutions to Common Problems

# solutionstocommon

+ powerviews:0140;technical:0220

K help;answers;problems

$ Error List

# errorlist

+ solutions:0050

K help;error;list;target

$ Can't Establish Communications with PowerPack/PowerScope

# cantestablishcommunications

+ solutions:0010

K error

$ Can't Communicate with Probe

# cantcommunicatewith

+ solutions:0020

K error

$ Can't Load Code

# cantloadcode

+ solutions:0030

K error

$ Can't Write Memory

# cantwritememory

+ solutions:0040

K error

$ Can't Access Target Memory

# cantaccesstarget

+ solutions:0050

K error

$ Unterminated Memory Cycle

# unterminatedmemorycycle

+ solutions:0055

K error

$ Can't Start or Stop Emulation

# cantstartor

+ solutions:0060

K error

$ Not Reaching Breakpoint

# notreachingbreakpoint

+ solutions:0070

K error

$ Not Getting Trigger

# notgettingtrigger

+ solutions:0080

K error

$ Getting false trigger

# gettingfalsetrigger

+ solutions:0090

K error

$ Not Collecting Trace

# notcollectingtrace

+ solutions:0090

K error

$ %s address missing.

# HE_ADDRESS_MISSING

K error;address

$ Address cannot start on an odd boundary.

# HE_ADDR_ODD_ADDRESS

K error;address;odd address

$ No memory available to store breakpoints.

# HE_BKPT_MEM_FAIL

K error;breakpoint;out of memory

$ No breakpoint available.

# HE_BKPT_NONE_AVAILABLE

K error;breakpoint

$ No breakpoint selected <action>.

# HE_BKPT_NONE_SELECTED

K error;breakpoint

$ Cannot get total bytes in file.

# HE_CANT_GET_SIZE

K error

$ Unable to open source file <file name>

# HE_CANT_OPEN_SOURCE_FILE

K error;source;file

$ Unable to open UNIX source file.

# HE_CANT_OPEN_UNIX_FILE

K error;UNIX;file

$ Cannot reopen file.

# HE_CANT_REOPEN_FILE

K error;source;file

$ Cannot reopen mixed data source.

# HE_CANT_REOPEN_MIXED

K error;source;file

$ Invalid decimal constant.

# HE_CLI_BAD_DEC

K error;shell

$ Invalid hexadecimal constant.

# HE_CLI_BAD_HEX

K error;shell

$ Invalid octal constant.

# HE_CLI_BAD_OCT

K error;shell

$ Command too long: <size>.

# HE_CLI_COMMAND_SIZE

K error;shell

$ A command is being executed; hit escape to terminate command.

# HE_CLI_DOING_COMMAND

K error;shell

$ File error: <type> failure on <file name>: access denied.

# HE_CLI_FILE_ACCESS

K error;shell;file

$ File error: <type> failure on <file name>: file not found.

# HE_CLI_FILE_NOT_FOUND

K error;shell;file

$ Error opening file: <file name>.

# HE_CLI_FILE_OPEN_ERR

K error;shell;file

$ File error: <type> failure on <file name>: path not found.

# HE_CLI_FILE_PATH

K error;shell;file

$ File error: <type> failure on <file name>: too many files open.

# HE_CLI_FILE_TOO_MANY

K error;shell;file

$ History size must be between 0 and 100.

# HE_CLI_HISTORY_SIZE

K error;shell;browser history;history

$ Include file or alias definition nesting limit exceeded.

# HE_CLI_INPUT_STACK_OVERFLOW

K error;shell

$ Statement list too large: <line count>.

# HE_CLI_LINE_COUNT

K error;shell;source lines

$ Invalid variable: %s.

# HE_CLI_MALFORMED_VARIABLE

K error;shell;variable

$ Command syntax error.

# HE_CLI_SYNTAX

K error;shell;syntax

$ Undeclared variable: $<name>.

# HE_CLI_UNDECL_VAR

K error;shell;variable

$ Unterminated comment.

# HE_CLI_UNTERM_COMMENT

K error;shell;comment

$ Unterminated string.

# HE_CLI_UNTERM_STRING

K error;shell;string

$ Command aborted by user.

# HE_CLI_USER_ABORT

K error;shell;abort

$ The target processor is emulating.  Pre-emulation values shown.

# HE_CPU_PROC_EM

K error;cpu;emulation;registers

$ Cannot set break on a non-opcode location.

# HE_DASM_CANT_SET_BKPT

K error;source;breakpoint

$ Invalid alias path.

# HE_DASM_INVALID_ALIAS

K error;source;alias

$ Unable to read memory.

# HE_DASM_MEMORY

K error;source;memory

$ No breakpoint to <action>.

# HE_DASM_NO_BKPT

K error;source;breakpoint

$ Failed to reopen a mixed mode data source.

# HE_DASM_REOPEN

K error;source

$ Invalid field name.

# HE_FIELD_NAME

K error

$ Field name already defined.

# HE_FIELD_NAME_DUP

K error

$ File not found: <file name>.

# HE_FILE_NOT_FOUND

K error;file;file not found

$ Source path is invalid.

# HE_INVALID_ALIAS_PATH

K error;alias

$ Invalid breakpoint specified.

# HE_INVALID_BKPT_INFO

K error;breakpoint

$ Invalid input value.

# HE_INVALID_INPUT

K error;

$ Invalid module symbol to select function.  Select the module first.

# HE_INVALID_MODULE_SYMBOL

K error;module

$ Line Number Missing.

# HE_LINE_MISSING

K error

$ From and to addresses out of order.

# HE_MEM_FROM_TO

K error

$ Non-numeric pattern.

# HE_MEM_NON_NUM

K error

$ Save/restore error.  File is not in the correct format.

# HE_NOT_MEM_CONF

K error

$ No symbolic information on the selected region.

# HE_NO_SYMBOLIC_INFO

K error

$ Frame not found.  Trace search failed.

# HE_NO_TRACE_FRAME

$ Number does not match base.

# HE_NUM_BASE_MISMATCH

K error

$ Pattern missing.

# HE_PATTERN_MISSING

K error

$ Scrolling requires accessing memory: <message>.

# HE_SCROLL_NEEDS_MEM

K error

$ Invalid request with only one trace buffer.

# HE_SINGLE_TRACE_BUFFER

K error;trace

$ Source module is newer than load file.

# HE_SOURCE_NEWER

K error;source

$ Cannot find <search text>.

# HE_SRC_CANT_FIND

K error

$ Cannot go forward in file browse history.

# HE_SRC_END_OF_HISTORY

K error

$ Function <name>:  Address starts at: <address>.

# HE_SRC_FUNC_START_INFO

K error

$ History depth must be in the range 5 to 100.

# HE_SRC_HISTORY_DEPTH

K error

$ Invalid address.

# HE_SRC_INVALID_ADDRESS

K error

$ Invalid line number.

# HE_SRC_INVALID_LINE

K error

$ <Feature> requires accessing memory: <message>.

# HE_SRC_MEM_NEEDED

K error

$ Failed to open a new view of data source.

# HE_SRC_OPEN_FAIL

K error

$ Failed to reopen a data source.

# HE_SRC_REOPEN_FAIL

$ Cannot back up in file browse history.

# HE_SRC_START_OF_HISTORY

$ Invalid step count.

# HE_SRC_STEP_COUNT

K error

$ Tab width is out of range.

# HE_SRC_TAB_RANGE

K error

$ View mode already selected.

# HE_SRC_VIEW_ALREADY

K error

$ Stack alarm limit exceeded.

# HE_STACK_ALARM_LIM

K error

$ Alarm limit is out of range.

# HE_STACK_ALARM_RANGE

K error

$ Invalid stack size specified. Set to 1K.

# HE_STACK_INVALID_SIZE

K error

$ Stack overflow.

# HE_STACK_OVERFLOW

K error

$ Stack size missing.

# HE_STACK_SIZE_MISSING

K error

$ Requested stack size is too large. Set to 1K.

# HE_STACK_TOO_LARGE

K error

$ Stack underflow.

# HE_STACK_UNDERFLOW

K error

$ No trace collected in the next trace buffer.

# HE_TRACE_DATA

K error

$ Disabling linked cursor.

# HE_TRACE_DISABLE_CURS

K error

$ Cannot fine function: <name>.

# HE_VAR_CANT_FIND_FUN

K error

$ Cannot find module: <name>.

# HE_VAR_CANT_FIND_MOD

K error

$ Cannot find variable: <name>.

# HE_VAR_CANT_FIND_VAR

K error

$ Symbols cannot end with #.

# HE_VAR_END_PROB

K error

$ Select highlighted text.

# HE_VAR_HIGHLIGHT

K error

$ Variable: <name> is at address <address>.

# HE_VAR_IS_AT_ADDR

K error

$ No selected variable to delete.

# HE_VAR_NO_DEL_SEL

K error

$ No more variables to undo.

# HE_VAR_NO_UNDO

K error

$ Token keyword not found.

# HE_KEY_TOKEN_NOT_FOUND

K error

# HE_NET_INCOMPATIBLE_VERSION

$ Network communications option not compatible with host software

K error;network