# $ # K Register cannot be set to an odd value.

Cause:	For 68000, PC, A7, SSP, and USP cannot be odd.

Solution:	These registers always access memory on word boundaries.  Use an even address.

See also

CPU Shell Commandscpushellcommands

CPU Windowcpu

# $ # K Value out of range.

Cause:	The register value is too large for the size of the register.

Solution:	Table in the file cpu68000.cfg show the number of bits for each register.  Use a value within the register's range.

See also

CPU Shell Commandscpushellcommands

CPU Windowcpu

# $ # K CPU configuration file missing.

Cause:	The CPU configuration file is not in MICEpack/SLD default directory (e.g. c:\mp68k).

Solution:	The file cpu68000.cfg is missing.  Re-install the MICEpack/SLD software into a new directory and copy the file to the existing directory.  Or, use the new MICEpack/SLD installation.

See also

CPU Shell Commandscpushellcommands

CPU Windowcpu

# $ # Processor configuration file format error.

Cause:	The CPU configuration file is corrupted.

Solution:	The file cpu68000.cfg is no good.  Re-install the MICEpack/SLD software into a new directory and copy these files to the existing directory.  Or, use the new MICEpack/SLD installation.

See also

CPU Shell Commandscpushellcommands

CPU Windowcpu

# $ # K Signals are not available in MICEpack.

Cause:	The signal Shell command is not available in MICEpack.

Solution:	MICEpack does not support signals.

See also

CPU Shell Commandscpushellcommands

CPU Windowcpu

# $ # K Invalid command line arguments.

Cause:	Syntax error.

Solution:	Look for the syntax description for the command you are using.

See also

CPU Shell Commandscpushellcommands

CPU Windowcpu

# $ # K Invalid address text string.

Cause:	The numeric address is invalid.

Solution:	Enter a number (1234, 0x4444, 01777) or a symbol (#main).

See also

Symbol Entry Syntaxsymbolentrysyntax

# $ # K Address is larger than the target processor's maximum address.

Cause:	The maximum input address specified is too large for the processor.

Solution:	The maximum input address is 0x00fffff (24 bits) for 68302/307/328 and 0xpfffffff (32 bits) for 68306.

# $ # K All available breakpoints are assigned.

Cause:	This error is now obsolete...if you get it with MICEpack/SLD version 2.0 or greater, please contact Microtek to report it.

Solution:	Please report this problem to Microtekcontactingmicrotek.

See also

Breakpoint Windowbreakpoint

Breakpoint Shell Commandsbreakpointshellcommands

# $ # K No breakpoint matched the specified ID.

Cause:	The identification number is not active.

Solution:	Display the current breakpoints with the bkpt Shell commandbkptshell for a list of active IDs.

See also

Breakpoint Windowbreakpoint

Breakpoint Shell Commandsbreakpointshellcommands

# $ # K Exceeded the maximum breakpoint ID value.

Cause:	The breakpoint identification number is too big.

Solution:	The value is 0 to 0xfffe.

See also

Breakpoint Windowbreakpoint

Breakpoint Shell Commandsbreakpointshellcommands

# $ # K The emulator is halted.

Cause:	Informational message indicating emulator is halted.

Solution:	Informational message only.

See also

Breakpoint Shell Commandsbreakpointshellcommands

# $ # K Breakpoint address not on an instruction boundary; breakpoint not set.

Cause:	The address specified does not line up with an instruction boundary.

Solution:	Use the dasm Shell commanddasmshell to disassembly memory and find the correct boundary.

See also

Breakpoint Windowbreakpoint

Breakpoint Shell Commandsbreakpointshellcommands

# $ # K Step count cannot be zero.

Cause:	Step count of zero has no meaning.

Solution:	Specify a step count of 0 to 0xffffffff.

See also

Breakpoint Windowbreakpoint

Breakpoint Shell Commandsbreakpointshellcommands

# $ # K No breakpoint matches the specified address.

Cause:	The address does not match a breakpoint.

Solution:	The breakpoint is not deleted.  Use the bkpt Shell commandbkptshell to display breakpoints with their addresses.

See also

Breakpoint Windowbreakpoint

Breakpoint Shell Commandsbreakpointshellcommands

# $ # K The emulator must be halted before performing this operation.

Cause:	Access to this feature requires the emulator to be halted.

Solution:	Halt emulationemulationterm or read and write memory while emulating using run accessrunaccessterm.

# $ # K Out of memory.

Cause:	No more memory available from Windows.

Solution:	Close other applications or MICEpack/SLD windows to free memory.

# $ # K Invalid instruction.

Cause:	Instruction mnemonic is not valid.

Solution:	Check assembly syntax.

See also

Recommended Readingrecommendedreading

# $ # K Incorrect number of operands for instruction.

Cause:	Number of operands does not match instruction mnemonic.

Solution:	Check assembly syntax.

See also

Recommended Readingrecommendedreading

# $ # K Invalid mode for instruction.

Cause:	The operand syntax is correct, but the mode is not valid for this instruction.

Solution:	Check assembly syntax.

See also

Recommended Readingrecommendedreading

# $ # K Immediate value incorrectly given.

Cause:	Immediate data operands start with a #.

Solution:	Add a # for immediate data (#1234, #0x4444).

See also

Recommended Readingrecommendedreading

# $ # K Symbol name given but symbols disabled.

Cause:	Symbol parsing not enabled.  (This error is obsolete with MICEpack/SLD version 2.0 and greater.  If you get this error with version of MICEpack/SLD of 2.0 or greater, please report it to Microtek.)

Solution:	Use dasmSym Shell commanddasmsymshell to enable symbols.

See also

Please report this problem to Microtekcontactingmicrotek.

Recommended Readingrecommendedreading

# $ # K Illegal bit for pshm/pulm instructions.

Cause:	Bit is incorrect for pshm or pulm instruction.

Solution:	Valid bits are upper or lower case D, E, X, Y, Z, K, or C.

See also

Recommended Readingrecommendedreading

# $ # K Masked instruction format wrong.

Cause:	Syntax is incorrect for masked instruction.

Solution:	Syntax is <opcode> <address operand>,#mask,displacement.

See also

Recommended Readingrecommendedreading

# $ # K Illegal index register.

Cause:	The index register is not correct..

Solution:	Use X, Y, or Z for index register.

See also

Recommended Readingrecommendedreading

# $ # K Bad displacement.

Cause:	The displacement is out of range..(This error message should be obsolete with version 2.0.  If you get this error with version 2.0 or greater, please contact Microtek.)

Solution:	Range is from -127 (0xffffff80) to +254 (0xfe).

See also

Please report this problem to Microtekcontactingmicrotek.

Recommended Readingrecommendedreading

# $ # K Invalid opcode.

Cause:	Instruction mnemonic is invalid.

Solution:	Check assembly syntax.

See also

Recommended Readingrecommendedreading

# $ # K At top of memory.

Cause:	Cannot disassembly past end of memory.

Solution:	Set the address for disassembly.

# $ # K Invalid instruction address.

Cause:	The address is greater than the maximum for the processor.

Solution:	The maximum address for 68302/307/328 is 0xffffff and for 68306 is 0xffffffff.

# $ # K Instruction mnemonic missing or invalid.

Cause:	The instruction mnemonic is wrong.

Solution:	Check assembly syntax.

See also

Recommended Readingrecommendedreading

# $ # K No queue flushes found in trace buffer.  Cannot view instructions.

Cause:	No processor flushes found in trace buffer.

Solution:	A queue flush occurs when a branch or interrupt occurs to change the linear flow of code.  With no flushes in a buffer, there is no way to line up instructions.

# $ # K Assembly syntax error.

Cause:	The syntax is incorrect.

Solution:	Check assembly syntax.

See also

Recommended Readingrecommendedreading

# $ # K Invalid displacement.

Cause:	The displacement is out of range.

Solution:	Check the negative or positive byte or word displacement.

See also

Recommended Readingrecommendedreading

# $ # K Invalid operand.

Cause:	The operand is invalid.

Solution:	Check the assembly syntax.

See also

Recommended Readingrecommendedreading

# $ # K Invalid register.

Cause:	The register name is incorrect.

Solution:	Use register A0, A1, ... A7 or D0, D1, ..., D7.

See also

Recommended Readingrecommendedreading

# $ # K Invalid effective address.

Cause:	The operand or operand combination is invalid.

Solution:	Check the operands for validity with instruction.

See also

Recommended Readingrecommendedreading

# $ # K Map block too large.

Cause:	The map block specified is too large.

Solution:	Check size of available overlay memory installed.  The available memory may be mapped up to 16 memory settings at 2K-byte block mapping resolution each.

See also

Mapping Memorymappingmemory

# $ # K Address range already mapped.

Cause:	The map block specified is already covered by mapped memory.

Solution:	Use the ToolBar'stoolbar map button or the map Shell commandmapshell to get the current map.  Change request so it does not overlap existing mapped memory.

See also

Mapping Memorymappingmemory

# $ # K There are no map blocks available.

Cause:	Not enough mapped memory blocks exist to satisfy request.

Solution:	Use the ToolBar'stoolbar map button or the map Shell commandmapshell to get the current map.  The available memory may be mapped up to 16 memory settings at 2K-byte block mapping resolution each.

See also

Mapping Memorymappingmemory

# $ # K No map ram installed on probe.

Cause:	No mapped memory is installed, so map request is not possible.

Solution:	Contact Microtekcontactingmicrotek to discuss mapped memory options.

See also

Mapping Memorymappingmemory

# $ # K Map block cannot be mapped.

Cause:	All memory map blocks currently used..

Solution:	Use the ToolBar'stoolbar map button or the map Shell commandmapshell to get the current map.

See also

Mapping Memorymappingmemory

# $ # K Data pattern too long.

Cause:	The number of data entries is too great.

Solution:	The fill, search, and write pattern is 1 to 256 entries.

See also

Fill Shell Commandfillshell

Search Shell Commandsearchshell

Write Shell Commandwriteshell

# $ # K The end address is less than the start address.

Cause:	The end address cannot be less than the start address.

Solution:	Check the addresses.  Memory is filled in ascending memory order.

See also

Fill Shell Commandfillshell

# $ # K Data values must be less than 256.

Cause:	Data must be a byte stream.

Solution:	Use byte data.

See also

Fill Shell Commandfillshell

Search Shell Commandsearchshell

Write Shell Commandwriteshell

Invalid stack frame.

Cannot read variable in stack frame.

Cannot write variable in stack frame.

# $ # K Stack base unknown

Cause:	The stack base was never set.

Solution:	Use the Stack Window'sstack Options/Stack Area or the setStackArea Shell commandsetstackareashell to set the base.

# $ # K Stack area too large.

Cause:	The stack base plus the stack size exceeds the processor's memory space.

Solution:	Use the Stack Window'sstack Options/Stack Area or the setStackArea Shell commandsetstackareashell to set the base.

# $ # K Stack area too small.

Cause:	The stack base plus the stack size exceeds the processor's memory space.

Solution:	Use the Stack Window'sstack Options/Stack Area or the setStackArea Shell commandsetstackareashell to set the base.

# $ # K Invalid high water mark.

Cause:	The high water mark is not inside the stack area.

Solution:	Use Stack Window'sstack Options/Stack Area or the setStackArea Shell commandsetstackareashell to view the stack area.

# $ # K Invalid stack size.

Cause:	The stack size contains non-numeric data.

Solution:	Check the setStackArea Shell commandsetstackareashell syntax.

# $ # K Invalid alarm limit.

Cause:	The alarm limit is not inside the stack area.

Solution:	Use the Stack Window'sstack Options/Stack Area or the setStackArea Shell commandsetstackareashell to view the stack area.

# $ # K Out of memory.

Cause:	No more memory available from Windows.

Solution:	Close other applications or MICEpack/SLD windows to free memory.

# $ # K Cannot find symbol.

Cause:	The symbol is not in the table..

Solution:	Check that the ToolBar Window'stoolbar Load button Options.../Load Symbols or Source Window'ssource File/Load Code... Options.../Load Symbols is checked.  Use the displaySymbols Shell commanddisplaysymbolsshell to check the name of symbols.

# $ # K Cannot find line number.

Cause:	The line number given is greater than the number of lines in the file.

Solution:	Use the Source Window'ssource View/Line Numbers to check line numbers in source.

# $ # K Symbols already loaded.

Cause:	Symbols from a load file are currently loaded.

Solution:	Symbols from one load file supported.  Delete symbols when queried or use the removeSymbols Shell commandremovesymbolsshell.

See also

ToolBar Windowtoolbar

Source Windowsource

# $ # K The load file time stamp does not match previously loaded file's time stamp.

Cause:	Modifications to the load file were made.

Solution:	When the load Option.../On Demand Symbol Loading is selected, symbols are loaded as needed.  The load file time stamp changed between the initial load and now.  To correct this inconsistency, load symbols again.  If code was re-compiled and re-linked, it needs to be loaded to match the symbols.

See also

Source Windowsource

# $ # K The current execution context does not match any known module.

Cause:	The program counter is outside of any module with symbolic information.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell displays module information.

See also

Source Windowsource

# $ # K Symbol is a local variable, thus it does not have a static address.

Cause:	The symbol is stack resident and has a different address each time its function is invoked.

Solution:	Only static symbols have permanent addresses.

See also

Source Windowsource

# $ # K No symbol information available about source modules.

Cause:	The line number information does not correspond to a source module.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell displays module and line number information.

See also

Source Windowsource

# $ # K Variable not found.

Cause:	The variable is not loaded.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell displays variable information.

See also

Variable Windowvariable

# $ # K Module name not found.

Cause:	There is no module by the name given.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell displays module information.

See also

Variable Windowvariable

# $ # K Function name not found.

Cause:	There is no function by the name given.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell displays module and function information.

See also

Variable Windowvariable

# $ # K Variable name not found.

Cause:	There is no variable by the name given.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell displays module, function, and variable information.

See also

Variable Windowvariable

# $ # K Invalid value: unsigned short int value must be 0 to 65535.

Cause:	The value is outside the range of the variable.

Solution:	The variable is an unsigned short integer and the value must be from 0 to 0xffff (65535).

See also

Variable Windowvariable

# $ # K Invalid value: unsigned long int value must be 0 to 4294967295.

Cause:	The value is outside the range of the variable.

Solution:	The variable is an unsigned integer and the value must be from 0 to 0xffffffff (4,294,967,295).

See also

Variable Windowvariable

# $ # K Invalid value: signed short int value must be -32768 to 32767.

Cause:	The value is outside the range of the variable.

Solution:	The variable is a short integer and the value must be from -32768 to 32767 (0x7fff).

See also

Variable Windowvariable

# $ # K Invalid value: signed long int value must be -2147483648 to 2147483647.

Cause:	The value is outside the range of the variable.

Solution:	The variable is a long integer and the value must be from -2,147,483,648 to 2,147,483,647 (0xffffffff).

See also

Variable Windowvariable

# $ # K Comm port hardware not present.

Cause:	The port specified is not there.

Solution:	The wrong port number was specified or a hardware problem exists with the serial board.

See also

Can't Establish Communicationscantestablishcommunications

# $ # K Device already open.

Cause:	The communications port is already open.

Solution:	Another application is using the comm port.  Close the other application.  If this is not the problem, exit Windows and re-boot the PC to re-initialize the comm port hardware.

# $ # K No active event has been selected.

Cause:	An event must be selected before this action.

Solution:	Set the Event Window'sevent Active Event and then execute action again.

# $ # K Invalid number.

Cause:	The syntax of the number is incorrect.

Solution:	Enter a number (1234, 0x4444, 0177).

See also

Event Windowevent

# $ # K The new event name is too long.

Cause:	The name is more than 63 characters.

Solution:	Event names of 1 to 63 characters are supported.

See also

Event Windowevent

# $ # K Cannot create any more events.

Cause:	8 events can be active at one time.

Solution:	Use the Event Window'sevent File/Save Events As... and File/Restore Events... to save various configurations of events to expand the effective number of events.

# $ # K Cannot open file.

Cause:	The file for saving or restoring events cannot be opened.

Solution:	If saving, the number of open files limit is exceeded.  Close other Windows applications using files or increase BUFFERS= in config.sys.

If restoring, the file name or path is incorrect.

See also

Event Windowevent

# $ # K Cannot write file.

Cause:	A write failed while saving the events.

Solution:	The hard disk drive is full.

See also

Event Windowevent

# $ # K Cannot read file.

Cause:	A read failed when attempting to restore an event file with the wrong format.

Solution:	Check the filename.  If the filename is correct, the format may be bad, or the file may have been corrupted or truncated.

See also

Event Windowevent

# $ # K File name required.

Cause:	The file name is blank.

Solution:	Enter a DOS file name.

See also

Event Windowevent

# $ # K Cannot close file.

Cause:	The file for saving or restoring events cannot be closed.

Solution:	A DOS error has occurred.  Check the file and try again.

See also

Event Windowevent

# $ # K Cannot change an event used by trigger while trace is on.

Cause:	Events currently being used cannot be modified.

Solution:	Turn tracing off or stop emulation using the ToolBartoolbar.

See also

Event Windowevent

# $ # K Cannot change an event used by trigger while emulating.

Cause:	Events currently being used cannot be modified.

Solution:	Stop emulation using the ToolBartoolbar Halt button, the Source Window'ssource Halt button or Run/Halt menu option, or the halt Shell commandhaltshell.

See also

Event Windowevent

# $ # K Cannot calculate end offset without a start.

Cause:	The start address must be defined before the length.

Solution:	Fill in the start address, then the length.

See also

Event Windowevent

# $ # K Cannot convert text to a value.

Cause:	The address or number is invalid.

Solution:	Enter a symbol or a number (1234, 0x4444, 0177).

See also

Symbol Entry Syntaxsymbolentrysyntax

Event Windowevent

# $ # K Cannot open configuration file.

Cause:	The trace configuration file is not in MICEpack/SLD default directory (e.g. c:\MICEPACK).

Solution:	An event configuration file (evXXX.cfg) is missing.  Re-install the MICEpack/SLD software into a new directory and copy these files to the existing directory.  Or, use the new MICEpack/SLD installation.

See also

Event Windowevent

# $ # K Unexpected end of file.

Cause:	The trace configuration file is corrupted in MICEpack/SLD default directory (e.g. c:\MICEPACK).

Solution:	An event configuration file (evXXX.cfg) has been truncated.  Re-install the MICEpack/SLD software into a new directory and copy these files to the existing directory.  Or, use the new MICEpack/SLD installation.

See also

Event Windowevent

# $ # K Cannot open event configuration file

Cause:	The trace configuration file is not in MICEpack/SLD default directory (e.g. c:\MICEPACK).

Solution:	An event configuration file (evXXX.cfg) is missing.  Re-install the MICEpack/SLD software into a new directory and copy these files to the existing directory.  Or, use the new MICEpack/SLD installation.

See also

Event Windowevent

# $ # K Counter value too large; setting to maximum value.

Cause:	The counter value range is 1 to 1023 (0x3ff).

Solution:	The counter was set to the maximum value of 1023.

See also

Using Counters and Timersusingcountersand

Trigger Windowtrigger

# $ # K Invalid event name.

Cause:	The event specified in the trigger is not defined.

Solution:	Check the events that are active in the Trigger Windowtrigger for one that is not defined.

See also

Event Windowevent

# $ # K Cannot change trigger while tracing and/or sequencer is enabled.

Cause:	Triggers cannot be modified while tracing or the sequencer is enabled.

Solution:	Turn tracing off or stop emulation then change the trigger setting.

See also

Trigger Windowtrigger

# $ # K A NEXT action requires multiple trace buffer configuration.

Cause:	The NEXT action starts collection in the next trace buffer.  Only 1 trace buffer is configured.

Solution:	Use the Trigger Window'strigger Options/Trace Control... to set the number of buffers.

See also

Selecting Trigger Actionsselectingtriggeractions

# $ # K Counter value too small; setting to minimum value.

Cause:	The counter value range is 1 to 1023 (0x3ff).

Solution:	The counter was set to the minimum value of 1.

See also

Using Counters and Timersusingcountersand

Trigger Windowtrigger

# $ # K Cannot open load file.

Cause:	The file does not exist.

Solution:	Check file name and path.

# $ # K Cannot add symbol to symbol table: <name>.

Cause:	The CompilerUsed setting may be inconsistent with the loadfile, and this can cause symbol loading to fail with some symbols.

Solution:	Verify the Compiler Usedcompilerusedshell setting from the Shell window.  If you are using a non-supported C++ toolchain, please refer to the Teknotes files in your \MICEPACK\teknotes directory.

# $ # K Cannot add function symbol to symbol table; local symbols will be lost: <function name>.

Cause:	The symbol table cannot be expanded.

Solution:	Window's has run out of memory.  Close other applications or other MICEpack/SLD windows.

# $ # K Cannot add module to symbol table: <module name>.

Cause:	The symbol table cannot be expanded.

Solution:	Window's has run out of memory.  Close other applications or other MICEpack/SLD windows.

$ Register cannot be set to an odd value.

# HE_CPU_CANT_BE_ODD

K error

$ Value out of range.

# HE_REG_OUT_OF_RANGE

K error

$ CPU configuration file missing.

# HE_NO_CPU_CONFIG_FILE

K error

$ Processor configuration file format error.

# HE_CPU_CONFIG_FILE_FORMAT

$ Signals are not available in PowerScope.

# HE_SIG_NOT_AVAIL_IN_POWERSCOPE

K error

$ Invalid command line arguments.

# HE_INVALID_ARG

K error

$ Invalid address text string.

# HE_ADR_INVALID_ADDRESS_STR

K error

$ Address is larger than the target processor's maximum address.

# HE_ADR_ADDRESS_TOO_LARGE

K error

$ All available breakpoints are assigned.

# HE_BX_NO_BKPT_AVAILABLE

K error

$ No breakpoint matched the specified ID.

# HE_BX_INACTIVE_BKPT

K error

$ Exceeded the maximum breakpoint ID value.

# HE_BX_ID_TOO_BIG

K error

$ The emulator is halted.

# HE_BX_EMU_IS_HALTED

K error

$ Breakpoint address not on an instruction boundary; breakpoint not set.

# HE_BX_NOT_ON_INST_BOUNDARY

K error

$ Step count cannot be zero.

# HE_BX_INVALID_COUNT

K error

$ No breakpoint matches the specified address.

# HE_BX_NO_BKPT_MATCH_ADDR

K error

$ The emulator must be halted before performing this operation.

# HE_BK_EMULATOR_MUST_BE_HALTED

K error

$ Out of memory.

# HE_HEAP_OUT_OF_MEMORY

K error;out of memory;memory

$ Invalid instruction.

# HE_DAD_NO_SUCH_INST

K error

$ Incorrect number of operands for instruction.

# HE_DAD_INVALID_NUM_OPERANDS

K error

$ Invalid mode for instruction.

# HE_DAD_INVALID_MODE_FOR_INST

K error

$ Immediate value incorrectly given.

# HE_DAD_IMMED_FIELD_INCORRECT

K error

$ Symbol name given but symbols disabled.

# HE_DAD_SYMBOL_USE_BUT_DISABLED

K error

$ Illegal bit for pshm/pulm instructions.

# HE_DAD_INVALID_REGLIST_BIT

K error

$ Masked instruction format wrong.

# HE_DAD_BAD_MASKED_INST_FORMAT

K error

$ Illegal index register.

# HE_DAD_INVALID_INDEX_REG

K error

$ Bad displacement.

# HE_DAD_INCORRECT_DISP

K error

$ Invalid opcode.

# HE_DAD_INVALID_OPCODE

K error

$ At top of memory.

# HE_DAD_AT_TOP_OF_MEMORY

K error

$ Invalid instruction address.

# HE_DAD_INVALID_INST_ADDR

K error

$ Instruction mnemonic missing or invalid.

# HE_DAD_INST_NOT_FOUND

K error

$ No queue flushes found in trace buffer.  Cannot view instructions.

# HE_DAD_NO_FLUSH_FRAMES_FOUND

K error

$ Assembly syntax error.

# HE_DAD_ASM_ERROR

K error

$ Invalid displacement.

# HE_DAD_DISPLACEMENT_INVALID

K error

$ Invalid operand.

# HE_DAD_OPERAND_INVALID

K error

$ Invalid register.

# HE_DAD_REG_INVALID

K error

$ Invalid effective address.

# HE_DAD_EA_INVALID

K error

$ Map block too large.

# HE_BLOCK_TOO_LARGE

K error

$ Address range already mapped.

# HE_BLOCK_ALREADY_MAPPED

K error

$ There are no map blocks available.

# HE_NO_FREE_MAP_BLOCKS

K error

$ No map ram installed on probe.

# HE_NO_MEMORY_PRESENT

K error

$ Map block cannot be mapped.

# HE_BLOCK_UNMAPPABLE

K error

$ Data pattern too long.

# HE_MEM_PAT_LEN

K error

$ The endaddress is less than the start address.

# HE_MEM_END_LESS_THAN_START

K error

$ Data values must be less than 256.

# HE_MEM_DATA_GREATER_THAN_BYTE

K error

$ Stack base unknown

# HE_STK_STK_BASE_INVALID

K error

$ Stack area too large.

# HE_STK_RANGE_TOO_LARGE

K error

$ Stack area too small.

# HE_STK_RANGE_TOO_SMALL

K error

$ Invalid high water mark.

# HE_STK_INVALID_HWM

K error

$ Invalid stack size.

# HE_STK_INVALID_STK_SIZE

K error

$ Invalid alarm limit.

# HE_STK_INVALID_ALARM_LIMIT

K error

$ Out of memory.

# HE_SYM_OUT_OF_MEMORY

K error

$ Cannot find symbol.

# HE_SYMBOL_NOT_FOUND

K error

$ Cannot find line number.

# HE_LINENUM_NOT_FOUND

K error

$ Symbols already loaded.

# HE_SYMBOLS_LOADED

K error

$ The load file time stamp does not match previously loaded file's time stamp.

# HE_TIMESTAMPS_DONT_MATCH

K error

$ The current execution context does not match any known module.

# HE_SYM_NO_CURRENT_CONTEXT

K error

$ Symbol is a local variable, thus it does not have a static address.

# HE_SYM_LOCAL_VAR_ADDRESS

K error

$ No symbol information available about source modules.

# HE_SYM_INVALID_LN_END_ADDR

K error

$ Variable not found.

# HE_VS_VAR_NOT_FOUND

K error

$ Module name not found.

# HE_VS_MODULENAME_NOT_FOUND

K error

$ Function name not found.

# HE_VS_FUNCNAME_NOT_FOUND

K error

$ Variable name not found.

# HE_VS_VARNAME_NOT_FOUND

K error

$ Invalid value: unsigned short int value must be 0 to 65535.

# HE_VS_USHORT_OVERFLOW

K error

$ Invalid value: unsigned long int value must be 0 to 4294967295.

# HE_VS_ULONG_OVERFLOW

K error

$ Invalid value: signed short int value must be -32768 to 32767.

# HE_VS_SHORT_OVERFLOW

K error

$ Invalid value: signed long int value must be -2147483648 to 2147483647.

# HE_VS_LONG_OVERFLOW

K error

$ Comm port hardware not present.

# HE_HARDWARE

K error

$ Device already open.

# HE_OPEN

K error

$ No active event has been selected.

# HE_NO_ACTIVE_EVENT

K error

$ Invalid number.

# HE_INVALID_DIGIT

K error

$ The new event name is too long.

# HE_NAME_TOO_LONG

K error

$ Cannot create any more events.

# HE_TOO_MANY_EVENTS

K error

$ Cannot open file.

# HE_CANT_OPEN_FILE

K error

$ Cannot write file.

# HE_WRITING_FILE

K error

$ Cannot read file.

# HE_READING_FILE

K error

$ File name required.

# HE_NO_FILE_GIVEN

K error

$ Cannot close file.

# HE_CANT_CLOSE_FILE

K error

$ Cannot change an event used by trigger while trace is on.

# HE_TRACING_ON

K error

$ Cannot change an event used by trigger while emulating.

# HE_TRIGGER_USING_EVENT

K error

$ Cannot calculate end offset without a start.

# HE_DEFINE_START_FIRST

K error

$ Cannot convert text to a value.

# HE_CANT_CONV_TEXT

K error

$ Cannot open configuration file.

# HE_CANT_OPEN_TEMPLATE_FILE

K error

$ Unexpected end of file.

# HE_UNEXPECTED_END

K error

$ Cannot open event configuration file

# HE_NO_EVFILE_ENTRY

K error

$ Counter value too large; setting to maximum value.

# HE_MAX_COUNTER_VALUE

K error

$ Invalid event name.

# HE_INVALID_EVENT_NAME

K error

$ Cannot change trigger while tracing and/or sequencer is enabled.

# HE_TRACE_TRIG_ON

K error

$ A NEXT action requires multiple trace buffer configuration.

# HE_MULTI_BUFS_NEEDED

K error

$ Counter value too small; setting to minimum value.

# HE_MIN_COUNTER_VALUE

K error

$ Cannot open load file.

# HE_CANNOT_OPEN

K error

$ Cannot add symbol to symbol table: <name>.

# HE_CANNOT_ADD_SYM

K error

$ Cannot add function symbol to symbol table; local symbols will be lost: <function name>.

# HE_CANNOT_ADD_FUNC

K error

$ Cannot add module to symbol table: <module name>.

# HE_CANNOT_ADD_MOD

K error