# $ # + K Memory Window

The Memory Window is used to view and change the contents of memory.  You can open more than one Memory Window.

Screen Description

Memory can be viewed and edited as follows:

-	Disassembled instructions, which are editablesinglelineassembler.  You can display code symbols with Toolbartoolbar/Configure/Symbolic Disassembly.

-	Raw memory in hexadecimal or decimal formats.  From left to right the columns are:

-	The hex memory addresses, which can be repositionedchangingthememory.

-	The contents of memory, which is editableeditingmemory.

-	The ASCII equivalent of the memory contents, which is editable.

Menu Selections

File

Exit	Close the Memory Window.

Edit

Go To Address...gotoaddressdialog	Go to a specific address.

Search Memory...searchmemorydialog	Search memory for a particular pattern.

Fill Memory...fillmemorydialog	Fill a region of memory with a pattern.

Copy Memory...copymemorydialog	Copy one region of memory to another.

Checksumchecksumshell	Calculate the checksum of the specified memory block.

Compare Memorycompareshell	Compare two memory blocks.

Test Memoryramtestshell	Test serviceability of overlay or target memory, and check memory type (RAM or ROM) in use.

View

Disassembly	Show memory as disassembled instructions.

Hex Bytes	Show 16 hexadecimal bytes on each line.

Hex Words	Show 8 hexadecimal words on each line.

Hex Dwords	Show 4 hexadecimal dwords on each line.

Decimal Bytes	Show 16 decimal bytes on each line.

Decimal Words	Show 8 decimal words on each line.

Decimal Dwords	Show 4 decimal dwords on each line.

SP	Select supervisor program space.

SD	Select supervisor data space.

UP	Select user program space.

SD	Select user data space.

Refresh Display	Refresh memory displaymemoryrefresh.

Options

Byte Access	Read and write memory as bytes.  Memory accessmemoryaccess does not determine the View options for display.

Word Access	Read and write memory as words.

Dword Access	Read and write memory as dwords.

Write Verify	Verify each write by reading the memory back.

Read Ahead	Enable caching of memorymemorycaching.  Memory outside the Memory Window's viewing area is read to speed scrolling.

Reread On Write	Reread the memory after each write.  Normally the window is updated, but all the memory is not read again.  Rereading is useful when writing one memory location affects another, which is the case with some internal registers.

Windows	Move between windowsmovingbetweenwindows  Choosing the

Memory Window brings up the Memory Window Dialog Boxmemorydialogbox.

Help	On-line helponlinehelp

Using the Memory Window

Memory Access (sizes and spaces)memoryaccess

Accessing Read-only or Write-only Memoryaccessingreadonly

When is Memory Written?whenismemory

Changing the Memory Addresschangingthememory

Memory Cachingmemorycaching

Memory Refreshmemoryrefresh

Run Access (access memory while emulating)runaccessterm

Single-line Assemblersinglelineassembler

See also

Memory Shell Commandsmemoryshellcommands

# $ # + K Memory Access (sizes and spaces)

Memory size for reading and writing memory is independent of how memory is displayed:

In the Memory Windowmemory:

-	Memory reads and writes use the Options/Byte Access, Options/Word Access, Options/Dword Access sizes.

-	Memory is displayed using the View options.

With the Memory Shell commandsmemoryshellcommands:

-	Memory reads and writes use the size Shell commandsizeshell setting.

-	Memory is displayed using the [byte | word | long | dword] parameter in each Shell command.

The algorithm for reading and writing memory is:

-	Word writes on word boundaries use word accesses.

-	Byte accesses always read a word, replace the byte in the word, then write the word back to memory.

-	Dwords (Longs)

-	Dword writes on dword boundaries use dword accesses.

-	Byte or word accesses always read a dword, replace the byte or word, then write the dword back to memory.

For address spaces:

In the Memory Windowmemory:

-	Reads and writes always use supervisor data space.

-	Use the Memory Shell commandsmemoryshellcommands to access other spaces.

Note that all Memory Shell commands use the parameters [cpu | up | ud | sp | sd].

# $ # + K Accessing Read-only or Write-only Memory

The Memory Windowmemory assumes memory is readable and writable.  For read-only or write-only memory or I/O, use the Memory Shell commandsmemoryshellcommands.

# $ # + K When is Memory Written?

A modified field in the Memory Windowmemory is written to memory when the cursor moves out of the field.  The center columns are fields and each of the 16 ASCII characters is a field.

In View/Disassembly mode, double clicking on a line brings up the assemblersinglelineassembler.  Click OK to write to memory and refresh the display.

See also

Memory Refreshmemoryrefresh

Memory Shell Commandsmemoryshellcommands

# $ # + K Changing the Memory Address

To scroll to a new address in the Memory Windowmemory:

-	Choose Edit/Go To Address... menu option.

-or-

-	Double click on the address field on the left side of the display.  The Go To Address... dialog box appears.

See also

Go To Address Dialog Boxgotoaddressdialog

# $ # + K Memory Caching

The Memory Window'smemory Options/Read Ahead enables memory caching.  Memory is read before and after the currently displayed region.  This feature speeds up scrolling.

Caching should be turned off when viewing an area near a non-existent memory region, because read ahead could fail.

# $ # + K Memory Refresh

The Memory Window'smemory View/Refresh Display reads a fresh image of memory and updates the display.  This is useful in multiple-processor systems where memory can be modified while emulationemulationterm is halted.

1.	If you want to be able to manually update memory during emulation then, from the Toolbartoolbar, choose Options/Enable Run Access before starting emulation.

2.	Choose View/Refresh Display.

See also

Dump Shell Commanddumpshell

# $ # + K Single-line Assembler

The Memory Window'smemory View/Disassembly option displays disassembled lines of code.

1.	Double click on a line to bring up the Single Line Assembly dialog boxsinglelineassemblydialog.

2.	Enter the new instruction and choose OK to write the results to memory.

3.	The display is then updated.

The single-line assembler accepts:

1.	Symbolic entry for absolute targets.

2.	Numbers must be entered in hexadecimal and may be prefixed by 0x.

Note that this is the only place which assumes hexadecimal radix.  Other places assume decimal radix.

See also

Symbol Entry Syntaxsymbolentrysyntax

Asm Shell Commandasmshell

# $ # K Memory Go To Address Dialog Box

Reposition the Memory Windowmemory to the specified numeric or symbolic addresssymbolentrysyntax.  Numeric addresses are hexadecimal addresses such as 0x100. address.

# $ # K Copy Memory Dialog Box

The Copy Memory Dialog Box enables you to copy memory from one region to another.  The regions can overlap.  Memory can be copied from target memory to overlay memory, and visa versa, even at the same address.

The From box defines the source of the copy:

-	The Start address of the memory region.

-	The End address or Length in bytes of the region.

-	Use the memory mapmappingmemory or override the memory map and use target memory.

The To box defines the destination of the copy:

-	The Start address of the memory range.

-	The region size specified in the From box is used.

-	Use the memory mapmappingmemory or override the memory map and use target memory.

See also

Copy Shell Commandcopyshell

Memory Windowmemory

# $ # K Search Memory Dialog Box

The fields are described as follows:

-	From is the starting address of a range.

-	To is the ending address of the range.  This address must be greater than or equal to the From address.

-	Pattern is one or more bytes, words, or dwords to search for.  Separate values with spaces.  A two digit number is interpreted as a byte, four digits as word, and eight digits as dword.  For example, 0x1122 0x33 0x4455667788 is 7 totals bytes (2 for word, 1 for byte, and 4 for dword).

See also

Search Shell Commandsearchshell

Memory Windowmemory

# $ # K Fill Memory Dialog Box

The fields are described as follows:

-	From is the starting address of a range.

-	To is the ending address of the range.  This address must be greater than or equal to the From address.

-	Pattern is one or more bytes, words, or dwords to search for.  Separate values with spaces.  A two digit number is interpreted as a byte, four digits as word, and eight digits as dword.  For example, 0x1122 0x33 0x4455667788 is 7 totals bytes (2 for word, 1 for byte, and 4 for dword).

See also

Fill Shell Commandfillshell

Memory Windowmemory

# $ # K Single Line Assembly Dialog Box

-	Enter the assembly instruction to replace the current line.

The Memory Window is updated.  If the new instruction is shorter or longer than the original instruction, the new display reflects this.

See also

Single Line Assemblersinglelineassembler

# # $ K + Run Access Definition

You can enable access to memory during emulation emulationterm only if run access is enabled.  Emulation is stopped briefly during a memory access, then restarted.

To enable:

-	Choose Run Access from the ToolBar Option menu

-or-

-	Enter the run access Shell commandrunaccessshell.

The Run Access setting affects the scrolling of the following windows, as well as other reads and writes:

-	The Memory windowmemory and the memory Shell commandsmemoryshellcommands.

-	The Peripheral windowperiph

-	The Variable windowvariable and the variable Shell commandsvariableshellcommands.

Note that Run Access does not affect the access of CPU registerscpu.

$ # K Memory Shell Commands

Memory related commands are:

asmasmshell <string>

asmAddrasmaddrshell [<address> [up | ud | sp | up]]

copycopyshell <from_start_address> {<from_end_address> | length <len>} [<space>] TARGET] to <to_address> [<space>]

checksumchecksumshell <start_address> <end_address> [up|ud|sp|sd]

comparecompareshell <from_start_address> {<from_end_address> | length <len>} <space> to <to_address> <space>

dasmdasmshell [ <start address> [<end address> ]] [up | ud | sp | up]

dasmSymdasmsymshell [on | off]

dumpdumpshell <start_address> [<end_address>] [byte | word | long | dword] [up | ud | sp | sd]

fillfillshell <start_address> <end_address> <data>... [byte | word | long | dword] [up | ud | sp | sd]

ramtstramtestshell <address1> <address2> [sd | sp | ud | up]

runAccessrunaccessshell [on | off]

searchsearchshell <start_address> <end_address> <data>... [byte | word | long | dword] [cpu | up | ud | sp | sd]

sizesizeshell [byte | word | long | dword]

verifyverifyshell [on | off]

writewriteshell <address> <data> ... [byte | word | long | dword] [sd | sp | ud | up]

See also

Shell Command List (alphabetic)shellcommandlist

Shell Command List (topical)shellcommandlisttopical

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Asm Shell Command

Assemble an assembly instruction into memory.

Syntax

asm <string>

Parameters

string	Text string to assemble and place in memory at current asmAddrasmaddrshell location.

Examples

Assemble a short program into memory.

> asmaddr 0x1000; asm "nop"; asm "bra 0x1000"

// asm address offset: 0x1000

//  001000  4E71            nop

// Number of bytes: 0x2

//  001002  60FC            bra 0x1000

// Number of bytes: 0x2

AsmAddr is incremented to the next memory location.

> asmaddr

// asm address offset: 0x1004

See also

AsmAddr Shell Commandasmaddrshell

Dasm Shell Commanddasmshell

DasmSym Shell Commanddasmsymshell

Shell Syntax Conventionsshellsyntaxconventions

# $ # K AsmAddr Shell Command

Set the address for assembling instructions into memory.  The address will be used by subsequent asm Shell commandsasmshell.

Syntax

asmAddr [<address> [sd | sp | ud |up]]

Parameters

address	Address to start assembling instructions.

up	Read from user program address space.

ud	Read from user data address space.

sp	Read from supervisor program address space.

sd	Read from supervisor data address space.

Examples

Set address and display value;

> asmaddr 0x2000; asmaddr

// asm address offset: 0x2000

// asm address offset: 0x2000

See also

Asm Shell Commandasmshell

Dasm Shell Commanddasmshell

DasmSym Shell Commanddasmsymshell

Shell Syntax Conventionsshellsyntaxconventions

# # $ K Checksum Shell Command

Calculate the checksum of the specified memory block. The checksum could be calculated by byte size or word size according to the setting of Size Commandsizeshell

Syntax

checksum <start_address> <end_address> [up|ud|sp|sd]

Parameters

<start_address>	The start address of memory block.

<end_address>	The end address of memory block.

up	Read from user program address space.

ud	Read from user data address space.

sp	Read from supervisor program address space.

sd	Read from supervisor data address space.

Examples

Size byte

> checksum 0 0xffff

// checksum is 0xfe

See also

Size Shell Commandsizeshell

Shell Syntax Conventionsshellsyntaxconventions

# # $ K Compare Shell Command

Compare two memory blocks.

Syntax

Compare <from_start_address> {<from_end_address> | length <len>} <space> to <to_address> <space>

Parameters

<from_start_address>	specifies the starting address of the source memory block.

<from_end_address>	specifies the ending address of the source memory block.

length <len>			specifies the compared memory block length .

space:  specifies one of the following spaces:

sd	Read from supervisor data address space.

sp	Read from supervisor program address space.

ud	Read from user data address space.

up	Read from user program address space.

to [<to_address>	]	Specifies the starting address of the destination memory block.

Examples

> compare 0 length 200 to 500

See also

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Copy Shell Command

Copy a region of memory to another region.  For PowerPack with overlay memory, you can copy memory to/from mapped memory to/from target memory.  Regions can overlap.

The Memory Window'smemory Edit/Copy Memory... is equivalent to this command.

Syntax

copy <from_start_address> [<from_end_address> | length <len>] [<space>] [target]  to <to_address> [<space>]  [target]

Parameters

from_start_address

The start address of the source range.

from_end_address

The end address of the source range.

len	The length of the source range in bytes.

space	The source address space: cpu | up | ud | sp | sd.

target	Override the memory map and read from target memory.

to address	The start address of the destination range.

space	The destination address space: cpu | up | ud | sp | sd.

target	Override the memory map and write to target memory.

Examples

Copy 128 KB of ROM code to overlay memory at the same address.

> map clear; map 0; copy 0 0x1fff target to 0

// All memory mapped to target

// Mapped block starting at address 0x000000 0x00FFFF RAM

See also

RunAccess Shell Commandrunaccessshell

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Dasm Shell Command

Disassemble memory into assembly instructions.

The Memorymemory window's View/Disassembly is equivalent to this command.

Syntax

dasm [[<start address> [<end address>]] [up | ud | sp | sd]

Parameters

start address	Address to start disassembly.  Disassembly continues at last address if no start address specified.

end address	Address to end display.  Completes instruction containing this address.  10 lines are displayed if no end address specified.

sd	Read from supervisor data address space.

sp	Read from supervisor program address space.

ud	Read from user data address space.

up	Read from user program address space.

Examples

Disassemble memory.

> dasm 0x45a 0x468

// 00045A  2F0A            MOVE.L      A2,-(A7)

// 00045C  247C 0000 0B84  MOVEA.L     #00000B84,A2

// 000462  4EB9 0000 04A2  JSR         QSM_setup

// 000468  2F0A            MOVE.L      A2,-(A7)

Disassemble next 10 lines.

> dasm

// 00046A  4EB9 0000 062A  JSR         Load_TxBuf

// 000470  588F            ADDQ.L      #4,A7

// 000472  4EB9 0000 0646  JSR         Load_CmdBuf

// 000478  4EB9 0000 05F6  JSR         SPCR1_Enable

// 00047E  4EB9 0000 067A  JSR         UnLoad_RxBuf

// 000484  4EB9 0000 0610  JSR         SPCR1_Disable

// 00048A  4EB9 0000 0698  JSR         Blank_TxBuf

// 000490  4EB9 0000 06B0  JSR         Blank_CmdBuf

// 000496  4EB9 0000 06C8  JSR         Blank_RxBuf

// 00049C  60CA            BRA.B       000468

See also

Asm Shell Commandasmshell

AsmAddr Shell Commandasmaddrshell

DasmSym Shell Commanddasmsymshell

Shell Syntax Conventionsshellsyntaxconventions

# $ # K DasmSym Shell Command

Show symbolic functions as targets of calls in the dasm Shell commanddasmshell and the Source Window'ssource mixed source and assembly display modesourcemixedand.  This command also affects the Tracetrace window display.

Toolbartoolbar/Configure/Symbolic Disassembly is equivalent to this command.

Syntax

dasmSym [on | off]

Parameters

on	Display symbols (default).

off	Do not display symbols.

Examples

Display current value.

> dasmsym

// Symbolic disassembler output is ON

Disassemble a line.

> dasm 0x46a 0x46a

// 00046A  4EB9 0000 062A  JSR         Load_TxBuf

Turn symbol display off and disassemble the same line.

> dasmsym off; dasm 0x46a 0x46a

// Symbolic disassembler output is OFF

// 00046A  4EB9 0000 062A  JSR         00062A

See also

Asm Shell Commandasmshell

AsmAddr Shell Commandasmaddrshell

Dasm Shell Commanddasmshell

Shell Syntax Conventionsshellsyntaxconventions

# # $ K Dump Shell Command

Display a region of memory.

The Memory Windowmemory displays the data displayed by this command.

Syntax

dump <start_address> [<end_address>] [byte | word | long | dword] [up | ud | sp | sd]

Parameters

start_address1	The starting address to display.

end_address2	The last address to display.

byte	Display as bytes.  Memory accessmemoryaccess is determined by the size Shell commandsizeshell.

word	Display as words.

long	Display as long words.

dword	Display as long words.

up	Read from user program address space.

ud	Read from user data address space.

sp	Read from supervisor program address space.

sd	Read from supervisor data address space.

Examples

Read memory as a series of bytes and display as long words.

> size byte; dump 0x1000 0x100f long

// 0x001000  9BBCB9DD FF9DF5DA 41457F73 86195415

See also

Fill Shell Commandfillshell

Write Shell Commandwriteshell

RunAccess Shell Commandrunaccessshell

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Fill Shell Command

Fill a region of memory with a byte, word, or long word pattern.

The Memory Window'smemory Edit/Fill Memory... is equivalent to this command.

Syntax

fill <address1> <address2> <data> ... [byte | word | long | dword] [cpu | up | ud | sp | sd]

Parameters

address1	The starting address to fill.

address2	The last address to fill.  Filling stops at this address even if the data pattern is not completely written.

data	A byte, word, or long word pattern.

byte	Interpret the fill data as bytes.  Memory accessmemoryaccess is determined by the size Shell commandsizeshell.

word	Interpret the fill data as words.

long	Interpret the fill data as long words.

dword	Interpret the fill data as long words.

cpu	Read from cpu address spacememoryaccess.

up	Read from user program address space.

ud	Read from user data address space.

sp	Read from supervisor program address space.

sd	Read from supervisor data address space.

Examples

Fill memory with a byte pattern.  Write memory as long words.  Display first line.

> size long; fill 0x1000 0x105f 0x11 0x22 byte; dump 0x1000 0x100f

// Fill successful.

// 0x001000  1122 1122 1122 1122 1122 1122 1122 1122

Repeat, but write as a word pattern.

> size long; fill 0x1000 0x105f 0x11 0x22 word; dump 0x1000 0x100f

// Fill successful.

// 0x001000  0011 0022 0011 0022 0011 0022 0011 0022

Repeat with a long word pattern.

> size long; fill 0x1000 0x105f 0x11 0x22 long; dump 0x1000 0x100f

// Fill successful.

// 0x001000  0000 0011 0000 0022 0000 0011 0000 0022

See also

Dump Shell Commanddumpshell

Write Shell Commandwriteshell

RunAccess Shell Commandrunaccessshell

Shell Syntax Conventionsshellsyntaxconventions

# # $ K Ramtst Shell Command

Test serviceability of overlay or target memory, and check memory type (RAM or ROM) in use.

Syntax

Ramtst <start_address> <end_address> [up | ud | sp | sd]

Parameters

<start_address1>	Starting address to test.

<end_Address2>	Last address to test.

Up		Read from user program address space.

ud		Read from user data address space.

sp		Read from supervisor program address space.

sd		Read from supervisor data address space.

Examples

Test a specified area of memory

> ramtst 0000 FFFF

See also

Shell Syntax Conventionsshellsyntaxconventions

selfTest Shell Command Selftest_Shell_Command

# $ # K RunAccess Shell Command

Run accessrunaccessterm allows memory reads and writes while emulatingemulationterm.

The ToolBar Window'stoolbar Options/Run Access is equivalent to this command.

Syntax

runAccess [on | off]

Parameters

on	Allow reading and writing during emulation.

off	Not allow reading and writing during emulation (default).

Examples

Display current setting.

> runaccess

// run access OFF

Set to allow reading and writing memory while emulating.

> runaccess on

// no output displayed.

Use run access.

> go; dump 0 //start emulation and dump memory while emulating

// 0x000000  0000 0000 0000 0408 6811 4763 1149 56D5

See also

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Search Shell Command

Search a region of memory for a pattern.

The Memory Window'smemory Edit/Search Memory... is equivalent to this command.

Syntax

search <start address> <end address> <data> ... [byte | word | long | dword] [cpu | up | ud | sp | sd]

Parameters

start address	The start of the search range.

end address	The last address of the search range.

data	A byte, word, or long word pattern.

byte	Interpret the search data as bytes.  Memory accessmemoryaccess is determined by the size Shell commandsizeshell.

word	Interpret the search data as words.

long	Interpret the search data as long words.

dword	Interpret the search data as long words.

cpu	Read from cpu address spacememoryaccess.

up	Read from user program address space.

ud	Read from user data address space.

sp	Read from supervisor program address space.

sd	Read from supervisor data address space.

Examples

Search for a byte pattern.

> fill 0x1000 0x10ff 0x11 byte; write 0x1055 0x22 byte; search 0x1000 0x10ff 0x22 byte

// Fill successful.

// Write successful.

// Pattern found at 0x1055.

Search for a word that does not match a pattern.

> fill 0x1000 0x10ff 0x1234 word; write 0x1077 0x99 byte; search 0x1000 0x10ff not 0x1234 word

// Fill successful.

// Write successful.

// Pattern found at 0x1077.

See also

RunAccess Shell Commandrunaccessshell

Shell Syntax Conventionsshellsyntaxconventions

# # $ K Size Shell Command

Set the memory accessmemoryaccess size.  This determines how memory is read and written for all memory Shell commandsmemoryshellcommands and for the Memory window.

The Memory Window'smemory  Options/Byte Access, Options/Word Access, and Options/Dword Access are equivalent commands.

Syntax

size [byte | word | long | dword]

Parameters

byte	Read and write memory as bytes (default).

word	Read and write memory as words.

long	Read and write memory as long words.

dword	Read and write memory as long words.

Examples

Show current setting.

> size

// byte

See also

RunAccess Shell Commandrunaccessshell

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Verify Shell Command

Verify each memory write by reading the memory back and comparing this to the written value.

The Memory Window'smemory Options/Write Verify is equivalent to this command.

Syntax

verify [on | off]

Parameters

on	Verify memory writes (default).

off	Don't verify memory writes.

Examples

Show current setting.

> verify

// on

Do not verify writes.

> verify off; verify

// off

See also

RunAccess Shell Commandrunaccessshell

Shell Syntax Conventionsshellsyntaxconventions

# # $ K Write Shell Command

Write a series of bytes, words, or long words to memory.

You can also edit the Memory Windowmemory directly.

Syntax

write <address> <data> ... [byte | word | long | dword] [up | ud | sp | sd]

Parameters

address	Starting address to write data.

data	A byte, word, or long word pattern to write.

byte	Interpret the write data as bytes.  Memory accessmemoryaccess is determined by the size Shell commandsizeshell.

word	Interpret the write data as words.

long	Interpret the write data as long words.

dword	Interpret the write data as long words.

cpu	Read from cpu address spacememoryaccess.

up	Read from user program address space.

ud	Read from user data address space.

sp	Read from supervisor program address space.

sd	Read from supervisor data address space.

Examples

Write a single byte of data and verify it is written.

> verify on; write 0x1000 0x55 byte

// Write successful.

Write a word pattern on an odd boundary.

> fill 0x1050 0x105f 0 long; write 0x1051 0x1234 0x5678 0x9999 word; dump 0x1050 0x105f word

// Fill successful.

// Write successful.

// 0x001050  0012 3456 7899 9900 0000 0000 0000 0000

See also

Dump Shell Commanddumpshell

Fill Shell Commandfillshell

RunAccess Shell Commandrunaccessshell

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Editing Memory

To edit a memory field, or its ASCII equivalent:

1.	Click on the field you want to change.

2.	Type over the field to modify it.  A field is defined as either a center column or one of the 16 ASCII characters.

3.	Move the cursor out of the field to change the field.

An illegal entry causes a beep.  Illegal entries include:

-	Non-numeric values in Decimal display.

-	Non-hexadecimal values in Hex display.

Note:

-	In decimal display, values longer than the defined field width are truncated (all digits to the right of the digit entered become zero).

-	A value must fall within the range of the displayed radix.  For example, if 199 is displayed in decimal byte radix, and you replace the "1" with a "2", resulting in 299, then 200 is displayed (the maximum value in this field is 255).

# # $ K + Memory Window Dialog Box

Choose the New button to open a new Memory window.

-or-

Choose a Memory window from the list box to activate it.

Memory window displays are numbered and described by the type of display and the beginning address of the display.

$ Memory Window

# memory

+ powerviews:0050

K memory;assembler;disassembler;window

$ Memory Access (sizes and spaces)

# memoryaccess

+ usingthememory:0010

K memory

$ Accessing Read-only or Write-only Memory

# accessingreadonly

+ usingthememory:0020

K memory

$ When is Memory Written?

# whenismemory

+ usingthememory:0030

K memory

$ Changing the Memory Address

# changingthememory

+ usingthememory:0040

K memory;address

$ Memory Caching

# memorycaching

+ usingthememory:0060

K memory

$ Memory Refresh

# memoryrefresh

+ usingthememory:0070

K memory

$ Single Line Assembler

# singlelineassembler

+ usingthememory:0090

K memory;assembler

$ Go To Address Dialog Box

# gotoaddressdialog

K memory;go to address dialog box; dialog box

$ Copy Memory Dialog Box

# copymemorydialog

K memory;dialog box;copy memory

$ Search Memory Dialog

# searchmemorydialog

K memory;dialog box;search memory

$ Fill Memory Dialog Box

# fillmemorydialog

K memory;dialog box;fill memory

$ Single Line Assembly Dialog Box

# singlelineassemblydialog

K memory;dialog box;assembler

# runaccessterm

$ Run Access Definition

K memory;target;peripherals;cpu;variable;scrolling;refresh;

+ USINGTHEMEMORY:0080

$ Memory Shell Commands

# memoryshellcommands

K memory;shell

$ Asm Shell Command

# asmshell

K memory;asm;assembler

$ AsmAddr Shell Command

# asmaddrshell

K memory;asmAddr

# checksumshell

$ Checksum Shell Command

K Checksum Shell Command

# compareshell

$ Compare Shell Command

K Compare Shell Command

$ Copy Shell Command

# copyshell

K memory;copy memory;copy

$ Dasm Shell Command

# dasmshell

K memory;dasm;disassembler

$ DasmSym Shell Command

# dasmsymshell

K memory;dasmSym;symbol;source;

# dumpshell

$ Dump Shell Command

K memory;dump;read memory

$ Fill Shell Command

# fillshell

K memory;fill;fill memory;write memory

# ramtestshell

$ RamTest Shell Command

K RamTest Shell Command

$ RunAccess Shell Command

# runaccessshell

K memory;run access

$ SearchShell Command

# searchshell

K memory;search;

# sizeshell

$ Size Shell Command

K memory access size;size;dump;write memory;fill;search;

$ VerifyShell Command

# verifyshell

K memory;verify;write memory;write verify

# writeshell

$ Write Shell Command

K memory;write memory

$ Editing Memory

# editingmemory

K memory;editing;

# memorydialogbox

$ Memory Window Dialog Box

K memory;

+ MEMORY