# # $ K ToolBar Window

The ToolBar contains buttons to access the most commonly used windows and commands.

Button Bar

Click on one of the buttons to access the feature:

Map	Set up memory mapmappingmemory.

Load	Load code and symbols.

Trigger	Set up trigger.trigger.

Source	Show source codesource.

Stack	Display program stackstack.

CPU	Show CPU registers.cpu.

Mem	Display memory regionmemory.

Periph	Show Peripheral Registersperiph.

Go	Start emulationemulationterm.

Halt	Stop emulation.

Start	Start trace collection.

Stop	Stop trace collection.

Show	Show trace collectedtrace.

Shell	Command lineshell.

Menu Selections

File

Exit MICEpack/SLD	Exit MICEpack/SLD applicationexitingpowerviews.

Configure

Map	Set up memory mapmappingmemory.

Run Access	Enable or disable run accessrunaccessterm.

Symbolic Disassembly	Enable or disable symbols in disassembly, which is displayed in the Sourcesource and Memorymemory windows.

Mask Interrupts For StepMask_Interrupts_for_Step>main	Enables single stepping in systems with periodic interrupts.

Reset	Resetresettool and reinitialize the target processor.

Reset CPU Only	Reset only the processorresetcpuonly and does not update the windows.

Layout

Save Settings Now	Save the current coordinates for the MICEpack/SLD windows and icons, to be restored the next time you invoke the tool.

Save Settings on Exit	Save the window and icon coordinates upon exiting MICEpack/SLD.

Windows	Move between windowsmovingbetweenwindows.

Help	On-line helponlinehelp

Using the ToolBar Window

ToolBar Shell Commandstoolbarshellcommands

Reading and Writing Memory while Emulating using Run Accessrunaccessterm

Minimizing MICEpack/SLD from the ToolBarminimizingpowerviewsfrom

See also

Getting Startedgettingstarted

# $ # K Minimizing MICEpack/SLD from the ToolBar

Minimizing the ToolBar window minimizes all of the MICEpack/SLD windows and icons into a single icon.  Maximizing the ToolBar icon will restore the ToolBar and access to the windows and other icons.

See also

Moving Between Windowsmovingbetweenwindows

# # $ K Toolbar Shell Commands

Shell commands related to the ToolBar:

gogoshell

halthaltshell

loadloadshell "<filename>" [symbols | nosymbols] [code | nocode] [progress | noprogress] demand | nodemand] [module <module path name>] [reload] [warn | nowarn] mangle | nomangle] [asm | noasm] [supervisor | user

mapmapshell [clear | <addr base> [<end addr>] [target] [ram | rombrk | none | off]] [<space>]

restoreMaprestoremapshell “<filename>“

runAccessrunaccessshell [on | off]

saveMapsavemapshell “<filename>“

versionversionshell

Memory Shell Commandsmemoryshellcommands

Shell Commandsshellcommands

Source Shell Commandssourceshellcommands

Stack Shell Commandsstackshellcommands

Status Shell Commandsstatusshellcommands

Trace Shell Commandstraceshellcommands

Trigger Shell Commandstriggershellcommands

Variable Shell Commandsvariableshellcommands

See also

Shell Command List (Alphabetic)shellcommandlist

Shell Command List (Topical)shellcommandlisttopical

# $ # K Moving Between Windows

To move the cursor to a MICEpack/SLD window:

-	Click on the window

-or-

-	Choose it from the Windows menu

-or-

-	Press F6.

To move to a different application's Window:

-	Click on it or press Alt-Tab.

# $ # K RAM

Random access memory (RAM) is memory that is readable and writable.

# $ # K ROM

Read only memory (ROM) is readable, but not writable memory.

# $ # K Mapping Memory

Mapped memory is MICEpack-supplied RAMramterm that replaces faulty or non-existent target memory (RAM or ROMromterm).  An access to target memory will be intercepted by mapped memory.  Signals will be active in the target, but data will be read or written to the mapped memory.

The Copy Shell commandcopyshell or the Memory Window's Edit/Copy... menu item moves memory between target and mapped areas.

Map features are:

-	Options for 512k, 1M, 2M, 4M, or 8M bytes of mapped memory.  Up to 16 memory settings at 2K-byte block mapping resolution may be assigned to a mapped block..

-	Selectable access rights:

-	Allow read and write accesses (RAM).

-	Allows read access only with break on an attempted write (ROMBRK).  Target memory is corrupted if a write to target memory is attempted.  A write to overlay memory is blocked and memory remains intact.

-	No read nor write access; break on an attempted access (NONE).  Target memory is corrupted if a write to 	target memory is attempted.

-	Allows setting of existing memory blocks to target (off).

-	Allows mapped block to be defined with one or more of the available memory access space (SP, UP, SD, UD) options.

To map memory:

-	Use the map dialog boxmapdialogbox by clicking on the Map buttonmappingmemory in the ToolBar.

-	Use the map Shell commandmapshell.

Mapped memory is a MICEpack emulator feature.

See also

Memory Copy Shell Commandcopyshell

# $ # K Map Dialog Box

MICEpack overlay memory may be mapped up to 16 memory settings at 2K-byte block mapping resolution.  Memory access rights (ram, rombrk, none) may be designated to every mapped block and defined with one or more of the available memory access space (SP, UP, SD, UD) options.

The current mapped memory settings are displayed in the List Box.

Click appropriate memory mapping button to perform desired function as explained below:

Add	Displays Add Dialog Box for adding new mapped memory settings.

Edit	Displays Edit Dialog Box to revise a current mapped memory setting.  (Double clicking on memory setting to be edited, will also display the Edit Dialog Box).

Delete	Disables a mapped memory setting (selected from List Box).

Save	Displays Save Dialog Box for saving mapped settings to a file.

Restore	Displays Restore Dialog Box for retrieving the saved mapped settings.

See also

Mapping Memorymappingmemory

Map Shell Commandmapshell

# $ # K Map Shell Command

Set memory map for target and mapped memory.  You can also choose the ToolBar Mapmapdialogbox button.

Syntax

map [clear | <addr base> [<end addr>] [target] [ram | rombrk | none | off]] [<space>]

Parameters

clear	clear all mapped blocks.

address base	the starting address of the mapped overlay memory block.  If the specified address does not start from a 2k-byte block boundary, it is rounded down to the nearest boundary block (e.g., 0, 0x800, 0x1000, 0x1200,  etc.)

end address	the end address of the mapped overlay memory block.  if the specified address does not end at a 2K-byte boundary, it is rounded up to the nearest boundary block.  If end-address is not specified, a 2k-byte block will default.

Target	is used to override default mapping (source or destination).  If specified, target memory is used as the source or destination.

allow reads and writes to region (default).  This allows unrestricted access to the region.

ram	allow reads and writes to region (default).  This allows unrestricted access to the region.

rombrk	allow reads, prevent writes, and break emulation if a write to the region is detected.

none	disallow reads, prevent writes, and break emulation if any access to the region is detected.

off	clear the defined map blocks.

space	is the memory access space (sp|up|sd|ud) options.  If none is specified, all options will default.

where:	SP is the supervisor program memory space
		UP is the user program memory access space
		SD is the supervisor data memory access space
		UD is the user data memory access space

Examples

Clear existing map.

>map clear

// All memory mapped to target

Map 64k region starting at address 0x1000 (rounded down).

>map 0x10010

// Mapped block #0: 0x00010000 0x000107FF RAM UP UD SP S

Display current map.

>map

// Mapped block #0: 0x00010000 0x000107FF RAM UP UD SP S

Map a second block that is 128k bytes long and write protected.

>map 0x60000 0x75000

// Mapped block #0: 0x00010000 0x000107FF RAM UP UD SP SD

// Mapped block #1: 0x00060000 0x000757FF RAM UP UD SP SD

Clear the 128k block and leave first block active.

> map clear 0x60000

// Mapped block #0: 0x00010000 0x000107FF RAM UP UD SP SD

See also

Mapping Memorymappingmemory

Map Dialog Boxmapdialogbox

Shell Syntax Conventionsshellsyntaxconventions

# $ # K On-line Help

To access on-line help:

-	Press F1.  This will give context sensitive help for the current window or dialog box.

-	Choose the Help menu item and pick a topic of interest.

# $ # K Symbol Entry Syntax

Symbols are specified as:

[#<module name>] [#<function name>] [#<variable name>] [#<column number>] [#<line number>]

A fully-qualified symbol contains the module name, function name, and variable name.  Column number specifies a statement breakpoint.  Line number specifies a line number.

If you do not specify a fully-qualified name MICEpack/SLD attempts to:

-	match the symbol in the current function

-	match the symbol in the current module

-	match the symbol to a global

Many symbol specifications are valid, including:

#<line number>

#<module name>

#<function name>

#<variable name>

#<column number>

#<module name>#<function name>

#<module name>#<variable name>

#<module name>#<column number>

#<function name>#<variable name>

#<function name>#<column number>

#<module name>#<function name>#<variable name>

#<module name>#<variable name>#<column number>

#<function name>#<variable name>#<column number>

#<module name>#<function name>#<variable name>#<column number>

# $ # K Version Shell Command

Display hardware and software version information.

Syntax

version

Parameters

none

Examples

Display version information.

>version

// System configuration is MICEpackÒ for MC68306

// Software version is 2.3M

// Firmware version is 0.0(0)

// Motherboard version is 1.0(1)

// Probe version is 0.0(0)

// Trace R version is 1.0(1)

// Trace S version is 1.0(1)

// Trace T version is 1.0(1)

// Dos version is 6.20

// Windows version is 3.10

See also

Reporting Problems to Microtekreportingproblemsto

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Swapfile

A portion of the hard disk is set aside to swap memory images when more memory is needed than actually exists.  To find the amount of swapfile space available:

-	Choose Program Manager's Main program group.

-	Choose the Control Panel.

-	Choose 386 Enhanced.

-	Choose Virtual Memory... button.

There are three settings:

-	Drive points to a hard disk partition (e.g. C:).

-	Size is the number of bytes in the swapfile (e.g. 6,000 KB is 6 Megabytes).

-	Type is permanent or temporary (e.g., type is Permanent if using BIOS).

MICEpack/SLD requires a minimum of 4 MB of permanent swapfile.

# $ # + K Exiting MICEpack/SLD

The ToolBar Window'stoolbar File/Exit closes all windows and terminates the MICEpack/SLD application.

Each individual window has an exit menu option.  This exit closes the window, but does not exit MICEpack/SLD.

# # $ K Peripheral Window

The Peripheral window enables the viewing and modifying of internal peripheral registers.

Screen Description

View the internal registers in compressed or expanded format.  The columns are (from left to right):

-	(-) or (+) indicates expanded or compressed, respectively.  Click to change format.

-	Register address.  Double-click anywhere on the line to edit.

-	Register value.

-	Register mnemonic.

-	Register Description.

Menu Selections

File

Exit	Close the Peripheral Window.

Edit

Registereditperiph...	Edit a peripheral register.

Go To Peripheralperiphgotoperiphdialog...	Scroll to a specified peripheral.

Go To Addressperiphgotoaddressdialog...	Scroll to a specified peripheral address.

View

Expand All	Expandexformperiph the register display.

Compress All	Compresscomformperiph the register display.

Refresh Display	Refreshupdateperiph the register display.

Windows	Move between windowsmovingbetweenwindows.

Help	On-line helponlinehelp.

Using the Peripheral Window

Editing a Peripheral Registereditperiph

Scrolling through the Peripheral Windowscrollperiph

# # $ K Edit Peripheral Register Dialog Box

Depending on the peripheral you are editing, MICEpack/SLD will display one of the following dialog boxes:

-	Edit Peripheral Register Dialog Boxeditperiphregdialog (for most peripherals).

-	Edit Peripheral Data Register Dialog Boxeditperiphregbufferdialog

# # $ K Expanded Format Peripheral Window

Shows the individual bit fields of a peripheral register, as well as the information shown in compressed formatcomformperiph.

# # $ K Compressed Format Peripheral Window

In this format, the columns in the Peripheral windowperiph are (from left to right):

-	Register address.

-	Register mnemonic

-	Register value (in red).

-	Register description.

# # $ K Update Peripheral Registers During Emulation

You can manually update the Peripheralperiph window if you suspect that the register values have changed but that change is not reflected in the window.

1.	If you want to be able to manually update memory during emulation then, from the Toolbartoolbar, choose Options/Enable Run Access before starting emulation.

2.	Choose View/Refresh Display.

# # $ K Scrolling the Peripheral Window

## If the Emulator is Halted:

-	Scrolling or paging updates the registers.

## To Scroll During Emulation:

1.	Toggle on Options/Run Accessrunaccessterm from the Toolbar before emulating.

2.	Scrolling or paging updates the registers.

# # $ K Receive and Transmit Buffers

Receive and transmit buffers are displayed in the Peripheralperiph window in their entirety.  Buffers are displayed in byte format.

-	Double click on a buffer line to display an edit dialog box that allows you to edit specific bytes in hex.

# # $ K Reset

Choose Configure/Reset from the Toolbartoolbar to do the following:

-	The processor reset pin is asserted.

-	The program counter and stack pointer values are read from memory.

-	All windows are updated with the following results:

-	The Stack display is invalid because the stack is reset.

-	The Source window points to the start of code.

See also

Resetting the Processorresettingtheprocessor

Reset CPU onlyresetcpuonly

# # $ K Reset CPU only

Resets only the processor and does not update the windows.  This command should be used only if the Reset command fails to reset the processor.

See also

Resetting the Processorresettingtheprocessor

Resetresettool

# # $ K Peripheral Go To Address Dialog Box

The window displays:

-	The peripheral at a valid hexadecimal address (prefixed by 0x).

To reposition the Peripheralperiph window display:

-	Enter an address.

See also

Go To Peripheral Dialog Boxperiphgotoperiphdialog

# # $ K Go To Peripheral Dialog Box

1.	Click the down-arrow on the drop-down box to display the list of peripheral groups.

2.	Enter a peripheral group for repositioning the Peripheralperiph window display.

See also

Peripheral Go To Address Dialog Boxperiphgotoaddressdialog

# # $ K Edit Peripheral Data Register Dialog Box

The dialog box displays:

-	A listbox that shows the data in all memory locations that comprise the buffer.

-	An edit field for the selected memory location.

To edit the buffers:

1.	Highlight the memory location you want to change

2.	Type the new value.

3.	When finished with all changes, write all locations of the buffer by choosing OK.

4.	Choose Cancel to exit the dialog box without making changes.

See also

Edit Peripheral Register Dialog Boxeditperiphregdialog

Receive and Transmit Buffersbufferperiph

# # $ K Edit Peripheral Register Dialog Box

The dialog box displays:

The register name in the title.

A list box with register fields.

The field to the right of the Field Value spin box.

Use the dialog box as follows:

1.	Edit the register value.

-or-

Use the list box to select a register field and edit it with the Field Value spin box.

2.	Choose  Write to write to the register.  This does not close the dialog box so you can make further changes and write again.

3.	When finished, choose Close to close the dialog box.  No writes take place when Close is chosen.

See also

Edit Peripheral Data Register Dialog Boxeditperiphregbufferdialog

# # $ K + Mask Interrupts for Step

If your target uses periodic interrupts, you can disable them.

-	Toggle on or off to mask or to not mask interrupts during single steps.

-	The default is off, i.e., interrupts are not masked.

Restrictions on use:

-	If mask is on, a single step restores the original contents of the SR register when complete.  If the stepped instruction modifies the register, then the modification may be lost when the register is restored by MICEpack/SLD.

The following instructions may operate incorrectly when stepped with the mask:

ANDI  <ea>,SR

ORI    <ea>,SR

EORI  <ea>,SR

MOVE <ea>,SR

MOVE SR,<ea>

RTE

LPSTOP

STOP

Most instructions which access memory can generate exceptions or traps due to bus or address errors, or as an expected result of the instruction.  If this exception or trap is generated, then the following sequence of events occurs:

1.	The value of SR/CCR (which is saved on the stack for the exception) is incorrect.

2.	When the exception returns the incorrect stack value is restored into the SR/CCR register.

Instructions which always or sometimes generate a trap include:

TRAP

TRAPcc

TRAPV

BKPT

CHK

DIVS

DIVSL

DIVU

DIVUL

ILLEGAL

LINE A

LINE F

# # $ K + RestoreMap Shell Command

Recalls the mapped overlay memory from a file.  You can also choose the ToolBar Mapmapdialogbox button.

Syntax

restoreMap "<filename> "

Parameters

filename	the file where the mapped memory is saved (using saveMapsavemapshell) and stored.

Examples

Recall mapped memory from file “abcd.map”.

>restoreMap abcd.map

// MAP 0x00000000 0x0007FFFF RAM UP UD SP SD

// Mapped block #0: 0x00000000 0x0007FFFF RAM UP UD SP SD

See also

Mapping Memorymappingmemory

Map Dialog Boxmapdialogbox

saveMapsavemapshell

Shell Syntax Conventionsshellsyntaxconventions

# # $ K + SaveMap Shell Command

Saves the mapped overlay memory to a file.  You can also choose the ToolBar Mapmapdialogbox button.

Syntax

saveMap "<filename> "

Parameters

filename	the file to save and store the mapped memory.

Examples

Save mapped memory to file “abcd.map”.

>saveMap abcd.map

See also

Mapping Memorymappingmemory

Map Dialog Boxmapdialogbox

RestoreMaprestoremapshell

Shell Syntax Conventionsshellsyntaxconventions

# toolbar

$ ToolBar Window

K toolbar;window;emulation

$ Minimizing PowerViews from the ToolBar

# minimizingpowerviewsfrom

K icons;minimizing;window

# toolbarshellcommands

$ Toolbar Shell Commands

K toolbar;shell

$ Moving Between Windows

# movingbetweenwindows

K window

$ RAM

# ramterm

K RAM;memory

$ ROM

# romterm

K ROM;memory

$ Mapping Memory

# mappingmemory

K map;memory;toolbar;overlay;target;

$ Map Dialog Box

# mapdialogbox

K map;Dialog Box;memory

$ Map Shell Command

# mapshell

K map;memory;load;

$ On-line Help

# onlinehelp

K help;window

$ Symbol Entry Syntax

# symbolentrysyntax

K symbols

$ Version Shell Command

# versionshell

K version;PowerViews;revision

$ Swapfile

# swapfileterm

K swapfile

$ Exiting PowerViews

# exitingpowerviews

+ gettingstarted:0130

K exit;PowerViews

# periph

$ Peripheral Window

K peripherals;registers

# editperiph

$ Edit Peripheral Register Dialog Box

K peripherals;registers;edit peripherals;

# exformperiph

$ Expanded Format Peripheral Window

K peripherals;registers;expanded format

# comformperiph

$ Compressed Format Peripheral Window

K peripherals;registers;compressed format

# updateperiph

$ Update Peripheral Registers During Emulation

K peripherals;registers;run access;update peripherals;

# scrollperiph

$ Scrolling the Peripheral Window

K peripherals;

# bufferperiph

$ Receive and Transmit Buffers

K peripherals;receive buffers;transmit buffers;edit buffers;

# resettool

$ Reset

K reset;processor reset

# resetcpuonly

$ Reset CPU only

K CPU;reset CPU;

# periphgotoaddressdialog

$ Peripheral Go To Address Dialog Box

K peripherals;go to address;dialog box;

# periphgotoperiphdialog

$ Go To Peripheral Dialog Box

K peripherals;go to peripheral;dialog box;

# editperiphregbufferdialog

$ Edit Peripheral Data Register Dialog Box

K peripherals;edit peripheral;dialog box;

# editperiphregdialog

$ Edit Peripheral Register Dialog Box

K peripherals;edit peripheral;dialog box;

# Mask_Interrupts_for_Step

$ Mask Interrupts for Step

K mask interrupts;step;emulation;

+ TOOLBAR

# restoremapshell

$ restoreMap Shell Command

K restoreMap Shell Command

+ TOOLBAR:000000

# savemapshell

$ saveMap Shell Command

K saveMap Shell Command

+ TOOLBAR:000000