# # $ K + CPU Window

The CPU Window displays registers and allows registers to be modified.

Screen Description

The registers are shown with their values.  Double click on the value field to edit.

Menu Selections

Options

Reset	Reset the processorresettingtheprocessor.

Help Index	MICEpack/SLD table of contents.

Help with Help	Using the help system.

Help with CPU	CPU Window table of contents.

Exit	Close the CPU Window

Using the CPU Window

Resetting the Processorresettingtheprocessor

Warning on Changing Certain CPU Registerswarningonchanging

See also

CPU Shell Commandscpushellcommands

# $ # + K Warning on Changing Certain CPU Registers

If the program counter, the stack pointer, or the supervisor/user bit of the status register is changed, the source and stack context may be invalid.

The program counter is changed by the Source Window'ssource Run/Go From Cursor, the CPU Windowcpu, and the register Shell commandregistershell.

The stack pointer and status register are changed from the CPU Windowcpu or by the register Shell commandregistershell.

See also

Resetting the Processorresettingtheprocessor

# $ # + K Resetting the Processor

To reset the processor, choose one of the following:

-	The CPU Window'scpu Options/Reset CPU menu option.

-	The Source window'ssource Run/Reset menu option.

-	The Toolbar’ssource Configure/Reset menu option.

-	The reset Shell commandresetshell.

A reset causes the following actions:

-	The processor reset pin is asserted.

-	The program counter and stack pointer values are read from memory.

-	All open windows are updated.

The consequences of reset are:

-	The stack is reset so the Stack Window contents are invalid.

-	The Source Windowsource points at the start of code which probably has no source.

If the reset command fails:

-	Initialize the program counter and the stack pointer to valid values using the CPU Windowcpu or the register Shell commandregistershell.

-	Reset the processor and update the windows by choosing one of the following:

-	CPU Window'scpu Options/Reset

OR

-	Source Window’source Run/Reset

OR

-	Toolbar’s.Configure/Reset

OR

-	Enter the reset Shell commandresetshell.

# # $ K Register Shell Command

Display and change CPU registers.

Syntax

register [<register name> [value]]...

Parameters

register name	The name of a register, such as SR, PC, D0, D1, ..., D7, A0, A1, ..., A7.

value	The new value for the register.  Be careful on changing certain registerswarningonchanging.

Examples

Display all registers.

> register

//     PC = 0x00000642

//     A7 = 0x00001DF6

//     A6 = 0x00000000

//     D0 = 0x00000020

//     D1 = 0x0000001E

//     D2 = 0xFFFFFFFF

//     D3 = 0xFFFFFFFF

//     D4 = 0x00008490

//     D5 = 0xFFFFFFFF

//     D6 = 0xFFFFFFFF

//     D7 = 0x00008490

//     A0 = 0x00000BA1

//     A1 = 0x00FFFD20

//     A2 = 0x00000B84

//     A3 = 0xFFFFFFFF

//     A4 = 0xFFFFFFFF

//     A5 = 0xFFFFFFFF

//     SR =     0x2700

//        = 0S7xnzvc

//    VBR = 0x00000000

Change a register value and display the new value.

> register d3 0x12345678; register d3

//     D3 = 0x12345678

See also

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Reset Shell Command

Reset the processorresettingtheprocessor.

The CPU Window'scpu Options/Reset is equivalent to this command.

Syntax

reset

See also

Resetting the Processorresettingtheprocessor

Shell Syntax Conventionsshellsyntaxconventions

# $ # K CPU Shell Commands

CPU related commands are:

registerregistershell [<register name> [value]]...

resetresetshell [cpuonly]resetshell

See also

Shell Command List (alphabetic)shellcommandlist

Shell Command List (topical)shellcommandlisttopical

Shell Syntax Conventionsshellsyntaxconventions

# cpu

$ Warning on Changing Certain CPU Registers

K cpu;registers;signal;reset;

+ POWERVIEWS:0040

$ Warning on Changing Certain CPU Registers

# warningonchanging

+ usingthecpu:0010

K cpu;register

$ Resetting the Processor

# resettingtheprocessor

+ usingthesource:0120

K reset;chip;initialization;cpu

# registershell

$ Reset Shell Command

K cpu;registers

$ Reset Shell Command

# resetshell

K cpu;reset;

$ CPU Shell Commands

# cpushellcommands

K cpu;shell