# $ # K Processor type not supported.

Cause:	The compiler specified an unsupported processor type.

Solution:	Check the flags in the IEEE compiler.

# $ # K Cannot find module.

Cause:	The module specified is not in the symbol table.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell lists modules.

See also

Source Windowsource

Load Shell Commandloadshell

# $ # K Cannot add public symbol to symbol table: <symbol name>.

Cause:	The CompilerUsed setting may be inconsistent with the loadfile, and this can cause symbol loading to fail with some symbols.

Solution:	Verify the Compiler Usedcompilerusedshell setting from the Shell window.  If you are using a non-supported C++ toolchain, please refer to the Teknotes files in your \MICEPACK\teknotes directory.

# $ # K Out of memory during load.

Cause:	No more memory available from Windows.

Solution:	Close other applications or MICEpack/SLD windows to free memory.

# $ # K Module time stamp is later than the original load file.

Cause:	Modifications to the load file were made.

Solution:	When the load Option.../On Demand Symbol Loading is choosen, symbols are loaded as needed.  The load file time stamp changed between the initial load and now.  To correct this inconsistency, load symbols again.  If code was re-compiled and re-linked, it needs to be loaded to match the symbols.

See also

Source Windowsource

# $ # K User abort requested.

Cause:	The Escape key aborted the load operation.

Solution:	The code and symbolic information may not be complete.

# $ # K Symbols already loaded, reload?

Cause:	Symbols from a load file are currently loaded.

Solution:	Symbols from one load file supported.  Delete symbols when queried or use the removeSymbols Shell commandremovesymbolsshell.

See also

ToolBar Windowtoolbar

Source Windowsource

# $ # K No starting PC available.

Cause:	The load file does not contain starting program counter information.

Solution:	The PC is not set.  Use the CPU Windowcpu or register Shell commandregistershell to set PC and stack before starting emulationemulationterm.

# $ # K Register translation table contains invalid value.

Cause:	The loader configuration file ldrcpu16.cfg or ldrcpu32.cfg is corrupt.

Solution:	Re-install the MICEpack/SLD software into a new directory and copy these files to the existing directory.  Or, use the new MICEpack/SLD installation.

# $ # K User abort requested.

Cause:	The Escape key aborted the load operation.

Solution:	The code and symbolic information may not be complete.

# $ # K Cannot load code while processor running.

Cause:	Code cannot be loaded while emulatingemulationterm unless run accessrunaccessterm is enabled.

Solution:	Stop emulation if loading code that is currently executing.

# $ # K Cannot update PC, SP while processor running.

Cause:	The registers cannot be written while emulatingemulationterm.

Solution:	Stop emulation to set register values.  The PC and SP are set if emulation is not running when loading.

# $ # K Symbols already loaded, specify 'reload' option.

Cause:	Symbols from a load file are currently loaded.

Solution:	Symbols from one load file supported.  Delete symbols using the removeSymbols Shell commandremovesymbolsshell or specified the reload option in the load Shell commandloadshell.

See also

ToolBar Windowtoolbar

Source Windowsource

# $ # K Loading is in progress, cannot perform the requested operation.

Cause:	Only one load can be active at a time.

Solution:	MICEpack/SLD is not currently multi-tasking.

# $ # K Cannot find Windows directory.

Cause:	The environment variable, windir, is missing.

Solution:	Set the environment variable to the location of Windows.  The Windows set up takes care of this typically.  There may be further problems with the Windows set up.

# $ # K Cannot find initialization file.

Cause:	The initialization file, micepack.ini, is missing.

Solution:	The initialization file must reside in the Windows directory.  The MICEpack/SLD installation places it there.  Re-install the MICEpack/SLD software into a new directory to get this file.  Use the existing or new MICEpack/SLD installation.

# $ # K Tool not defined in initialization file.

Cause:	The initialization file, micepack.ini, is missing the loader information.

Solution:	The initialization file may be missing or the [ToolChain] section in the file is missing.  Re-install the MICEpack/SLD software into a new directory to get this file.  Use the existing or new MICEpack/SLD installation.

# $ # K Invalid section name.

Cause:	An entry in the initialization file, micepack.ini, under the [ToolChain] section is incorrect.

Solution:	Check any editing changes.  Re-install MICEpack/SLD software into a new directory to get the original initialization file.  Use the existing or new MICEpack/SLD installation.

# $ # K Missing section name information.

Cause:	An entry in the initialization file, micepack.ini, under the [ToolChain] section is incorrect.

Solution:	Check any editing changes.  Re-install MICEpack/SLD software into a new directory to get the original initialization file.  Use the existing or new MICEpack/SLD installation.

# $ # K No compilers defined in initialization file.

Cause:	In the initialization file, micepack.ini, the [ToolChain] section is missing the Compilers= line.

Solution:	Check any editing changes.  Re-install MICEpack/SLD software into a new directory to get the original initialization file.  Use the existing or new MICEpack/SLD installation.

# $ # K Cannot get compiler defined in initialization file.

Cause:	In the initialization file, micepack.ini, the [ToolChain] section member Compilers= is corrupted.

Solution:	Check any editing changes.  Re-install MICEpack/SLD software into a new directory to get the original initialization file.  Use the existing or new MICEpack/SLD installation.

# $ # K Missing compiler information in initialization file.

Cause:	In the initialization file, micepack.ini, the [ToolChain] compiler information is corrupted.

Solution:	One of the compiler lines (e.g. MRI=) is bad.  Check any editing changes.  Re-install MICEpack/SLD software into a new directory to get the original initialization file.  Use the existing or new MICEpack/SLD installation.

# $ # K Compiler name too long.

Cause:	In the initialization file, micepack.ini, the [ToolChain] compiler information contains a compiler line with a name greater than 30 characters.

Solution:	Check any editing changes and reduce the number of characters on a compile line (e.g. MRI= is a compiler line).  Re-install MICEpack/SLD software into a new directory to get the original initialization file.  Use the existing or new MICEpack/SLD installation.

# $ # K Cannot write to initialization file.

Cause:	In the initialization file, micepack.ini, cannot be written.

Solution:	Check that the initialization file in the Windows directory (e.g. c:\windows) is writeable.

# $ # K Maximum number of sections reached; the rest will be ignored.

Cause:	The number of sections in the load file exceeds the maximum of 32 supported.

Solution:	Use the mergeSections Shell commandmergesectionsshell.

# $ # K Incompatible load file processor type: <processor>.

Cause:	The compiler specified an unsupported processor type.

Solution:	Check the flags in the IEEE compiler.

# $ # K Cannot find section defined in initialization file: <section name>.

Cause:	In the initialization file, micepack.ini, the [ToolChain] compiler information is bad.

Solution:	The compiler line (e.g. MRI=) has a bad section.  There should be two entries, one for the code section and one for the data section separated by a comma.

Check any editing changes.  Re-install MICEpack/SLD software into a new directory to get the original initialization file.  Use the existing or new MICEpack/SLD installation.

# $ # K No line numbers in module: <module name>

Cause:	A module has no line number debug information.

Solution:	Source debugging is not possible without line number information.  Check compiler switches to enable line number debug information.

# $ # K Loader configuration file corrupted.

Cause:	The loader configuration file ldrcpu16.cfg or ldrcpu32.cfg is corrupt.

Solution:	Re-install the MICEpack/SLD software into a new directory and copy these files to the existing directory.  Or, use the new MICEpack/SLD installation.

# $ # K Invalid frame number.

Cause:	The frame number is not valid.

Solution:	Enter a number (1234, 0x4444, 0177) for the frame number.

See also

Trace Windowtrace

# $ # K Too many template fields for one line.

Cause:	The trace configuration file is corrupted in MICEpack/SLD default directory (e.g. c:\MICEPACK).

Solution:	An event configuration file (evXXX.cfg) has too many entries on a line.  Re-install the MICEpack/SLD software into a new directory and copy these files to the existing directory.  Or, use the new MICEpack/SLD installation.

See also

Trace Windowtrace

# $ # K Invalid search event name.

Cause:	The event name given does not exist.

Solution:	The Event Window Active Event list has no entry for event.  If it is saved on disk, use the Event Window'sevent File/Restore Events... to bring into the Active Event list.

See also

Trace Windowtrace

# $ # K Timed out waiting for hardware to respond.

Cause:	The communications between the host PC and the hardware failed.

Solution:	The software retries many times before reporting this error.  Check the parallel connection and try operation again.  If it still fails, exit MICEpack/SLD and start again.  Success this time indicates a possible software bug to report to Microtekcontactingmicrotek.

# $ # K Cannot open the connection.

Cause:	The communications port cannot be opened.

Solution:	The wrong comm port is specified or another application is using the comm port.  Close the other application.  If this is not the problem, exit Windows and re-boot the PC to re-initialize the comm port hardware.

See also

Changing Comm Portcommportswitching

# $ # K Session terminated.

Cause:	This is an informational message.

Solution:	Read the MICEpack Up and Running manual for information on getting started with MICEpack/SLD.

See also

Solutions to Common Problemssolutionstocommon

# $ # K Communication time-out sending to box.

Cause:	The communications between the host PC and the hardware failed.

Solution:	The software retries many times before reporting this error.  Check the parallel connection and try operation again.  If it still fails, exit MICEpack/SLD and start again.  Success this time indicates a possible software bug to report to Microtekcontactingmicrotek.

# $ # K Cannot establish communications with MICEpack

Cause:	The communications between the host PC and the hardware failed.

Solution:	Check the following:

1.	Verify that all external cable connectors are securely connected.

2.	Make sure the MICEpack is turned on and the Power LED is lit.

3.	Make sure that the communication link setup selection corresponds with 	the type of COM port in use.

# $ # K Cannot initialize probe.

Cause:	Communications with the probe cannot be established for initialization.

Solution:	Check the following:

1.	The cable from MICEpack chassis to ICE probe has no bent pins and is secured in the connectors.

2.	The power cord from null-target board to the ICE probe is not connected.  The LED on the probe tip should be lit.

Re-seat all cables in case of a bad connection.  If probe still does not initialize, contact Microtekcontactingmicrotek technical support.

# $ # K MICEpack version not compatible with the required ROM breakpoints.

Cause:	The MICEpack version does not support simulated ROM breakpoints.

Solution:	Contact Microtekcontactingmicrotek for upgrade information.

# $ # K MICEpack version not compatible with the required internal termination.

Cause:	The MICEpack version does not support automatic bus cycle termination.

Solution:	Contact Microtekcontactingmicrotek for upgrade information.

# $ # K Cannot read mapped memory type.

Cause:	A hardware error with the mapped memory was detected.

Solution:	Contact Microtekcontactingmicrotek technical support for help.

# $ # K ICE Probe transmission error.

Cause:	A communications error occurred between the main module and the ICE probe module.

Solution:	Check the following:

1.	The ribbon cables from the main module to ICE probe are nor properly locked in place.

2.	The two wire power cord from the ICE probe to the null target board is not properly connected.  The LED on the probe tip should be lit.

3.	Reset the processorresettingtheprocessor since it may be hung.

Re-seat all cables in case of a bad connection.  If MICEpack still does not initialize, contact Microtekcontactingmicrotek technical support.

# $ # K Unterminated memory cycle; internal termination failed.

Cause:	Memory at an invalid address was accessed and MICEpack's attempt to terminate the memory failed.

Solution:	Reset the processorresettingtheprocessor.

# $ # K Processor reset during BDM exchange.

Cause:	The processor reset while communicating between the main module and the probe module.

Solution:	A possible hardware problem exists with the target.

# $ # K Processor clock speed is slower than 131 KHz.

Cause:	The clock is too slow for MICEpack.

Solution:	Increase the clock frequency.

# $ # K Memory verification failed.

Cause:	Memory verification is enabled.  A written memory location failed to read back the proper data.

Solution:	Check your target memory map to verify memory at the location being accessed.

See also

Verify Shell Commandverifyshell

Memory Windowmemory

# $ # K Timed out waiting for emulation to stop.

Cause:	A communications failure between the main module and the probe module tip occurred while halting emulationemulationterm.

Solution:	Reset the processorresettingtheprocessor.

# $ # K Timed out waiting for single step.

Cause:	A communications failure between the main module and the ICE probe module occurred while steppingemulationterm.

Solution:	Reset the processorresettingtheprocessor.

# $ # K Fill command aborted prior to completion.

Cause:	The Escape key aborted the memory fill in progressemulationterm.

Solution:	Check the range for the fill and reduce the range to speed operation.

# $ # K Violate system map

Cause:	0xFFFFF000~0xFFFFFFFF(SD space) has been occupied by MICEpack.

Solution:	Avoid map setting at this area.

# $ # K BKPT pin was asserted spuriously.

Cause:	Emulation was halted because the BKPT pin was asserted.

Solution:	When breakpoints are being used, the BKPT is asserted by the MICEpack hardware to stop emulation.  A BKPT occurred, but MICEpack did not generate it.  Check the target hardware for spurious BKPT assertion.

# $ # K Attempted halt failed.

Cause:	The probe timed out responding to the halt request.

Solution:	The processor may be in a STOP state.

# $ # K SIM register bank cannot be located.

Cause:	The MBAR register has not been set to valid (330, 340, 360).

Solution:	Enable internal registers by writing to MBAR.

Cause:	Cannot read memory at either of the two possible locations for the SIM.

Solution:	Check for overlapping internal RAM, slow processor clock, reset assertion.

See also

Accessing Memorymemoryaccess

# $ # K Trace instruction processing aborted.

Cause:	The Escape key aborted the trace instruction calculation in progressemulationterm.

Solution:	This is an informational message.

See also

Trace Windowtrace

# $ # K No bus cycles collected in trace buffer.

Cause:	The current trace buffer does not contain any bus cycles for trace instruction processingemulationterm.

Solution:	This is an informational message.

See also

Trace Windowtrace

# $ # K Trigger programming aborted.

Cause:	The Escape key aborted the programming in progressemulationterm.  Emulationemulationterm is not started.

Solution:	This is an informational message.

# $ # K Invalid trace buffer.

Cause:	The trace buffer has no data collected in it yet.

Solution:	Check the sequential action in the trigger programming.  A trigger may not have been reached yet.  Check the Status Windowstatus to see which buffer is currently collecting trace.

# $ # K Unterminated memory access; internal termination used.

Cause:	The target hardware did not terminate the memory cycle properly.  MICEpack asserted to terminate the cycle.

Solution:	This is an informational message.

# $ # K Memory copy aborted.

Cause:	The Escape key aborted the memory copy in progressemulationterm.

Solution:	Only a portion of the memory was copied.

# $ # K Memory search aborted.

Cause:	The Escape key aborted the memory search in progressemulationterm.

Solution:	The search was terminated.

# $ # K Search found at <address>.

Cause:	The search pattern was found at the address shown.emulationterm

Solution:	This is an informational message.

See also

Memory Windowmemory

# $ # K Search not found.

Cause:	The search pattern was not found in the memory range specified.emulationterm

Solution:	This is an informational message.

See also

Memory Windowmemory

# $ # K Load Aborted.

Cause:	The Escape key was pressed to terminate the load.

Solution:	The code was not completely loaded.

See also

Load Shell Commandloadshell

Source Windowsource

# $ # K ICE probe version incompatible with MICEpack/SLD.

Cause:	MICEpack/SLD of older version is used with ICE probe module of newer version.

Solution:	Contact Microtekcontactingmicrotek for upgrade information.

# $ # K Abort from Escape.

Cause:	The Escape key was pressed to terminate a lengthy operation.

Solution:	The operation was not completed.

# $ # K Expected ATN record type.

Cause:	An ATN record was expected but not found in the load file.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

compilers supportedtoolchains

changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Unrecognized miscellaneous record skipped.

Cause:	A record of unknown type is in the load file, and was skipped during loading.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Attribute type not supported.

Cause:	An attribute type in the load file is not supported by MICEpack/SLD.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Cannot read file.

Cause:	Can't read the load file.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Invalid or unsupported ID.

Cause:	An ID in the load file is invalid or not supported.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Invalid module id.

Cause:	A module ID in the load file is invalid.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Bad ATI record in Loadfile.

Cause:	An ATI record in the loadfile is corrupted.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Corrupted file.

Cause:	The load file has a format error.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Invalid 695 file header.

Cause:	The header of the load file contains an error.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Invalid line number block in file.

Cause:	There is an invalid line number block in the load file.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Nested block is not a procedure.

Cause:	A nested block in the load file is not a procedure.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Invalid part of file.

Cause:	A section of the load file is unrecognized or invalid.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Invalid register index.

Cause:	A register index in the load file is invalid.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Invalid section record.

Cause:	A section record in the load file is invalid.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Cannot seek load file.

Cause:	Read operations on the load file are not working.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Expected begin-block function record.

Cause:	The load file cannot be read.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Current function has no call or return instructions.

Cause:	This may be caused by normal compiler optimization, in some cases.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K PC-NFS is not installed.

Cause:	PC-NFS is not installed, but is needed.

Solution:	Install PC-NFS or reboot your computer if needed.

# $ # K Problems on mapping boundary, mapping resolution, or memory spaces

Cause:	The problem will occur only while setting a map with NONE type. Map settings of one or all of the following are in error:

·1. start address is set outside the 128Kb-block boundary

·2. mapped size is not set in 128Kb-block resolution

·3. one or more address space settings is omitted

Solution: ·1. set start address at 128Kb-block boundary

·2. set map size to a multiple of 128Kb-block resolution

·3. ensure that all address spaces are set

# $ # K Must define CompilerUsed information, specify 'CompilerUsed <compiler>'

Cause:	No compiler has been specified.

Solution:	Check Compiler Used.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Cannot add module reference to symbol table.

Cause:	An internal module name can't be added.  This is usually not a serious problem.

Solution:	Check the switches on the compiler command line.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

# $ # K Breakpoint already set.

Cause:	A breakpoint has already been set at the current location.

Solution:	Re-check the address of the breakpoint.

# $ # K Invalid number of parameters.

Cause:	The Shell command you used takes a different number of parameters.

Solution:	Double-check the syntax of the Shell command.

See also

Shell Commandsshellcommands.

# $ # K Operation not allowed while emulating.

Cause:	An operation cannot be done while emulation is running.

Solution:	Enable runaccessrunaccessshell from the ToolBar/Configure menu or by using the RunAccess Shell Commandrunaccessshell.  This will allow memory commands to occur during emulation.

See also

RunAccess Shell Commandrunaccessshell

ToolBar Windowtoolbar

# $ # K CPU configuration file missing.

Cause:	The CPU configuration file is missing.

Solution:	The CPU configuration files should have been installed during MICEpack/SLD installation.  If you have changed the directory, please copy the appropriate ".cfg" file to the new location, or else try re-installing MICEpack/SLD.

# $ # K Optimized code detected: source view and variable access may be incorrect.

Cause:	The loadfile used contains optimized code.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell

Compilers supportedtoolchains

Changing compilerschangingcompilers

Recommended Readingrecommendedreading

# $ # K Invalid tab width.

Cause:	The tab width used is invalid.

Solution:	Choose a different tab width, between 1 and 32 spaces.

# $ # K Stack high water measurement is disabled.

Cause:	MICEpack/SLD has failed to enable the HighWaterMark.

Solution:	Reset/Reinitialize stack area for the HighWaterMark.  Use the fillStackPatternfillstackpatternshell Shell command.

See also

HighWaterMarkhighwatermark

# $ # K Cannot open a stack session.

Cause:	This should only occur if you run out of memory, or some other fairly catastrophic system failure has occurred.

Solution:	Try restarting Windows and MICEpack/SLD -- if you get this error again, please contact Microtek.

See also

Contacting Microtekcontactingmicrotek

# # $ K Unable to access communication type.

Cause:	The communication type was not specified in the MICEPACK.INI file.

Solution:	In MICEPACK.INI, check the communication type in the "[Comm]" section; the line "type = serial" sets the communication type to serial, and "type = network" sets the communication type to network.

# # $ K Network communication style not selected.

Cause:	Although network system code in MICEpack/SLD is active, the indicated communication type is not network.

Solution:	In MICEPACK.INI, check the communication type in the "[Comm]" section; network communication must be specified as "type = network".

# # $ K Unable to find network emulator host names.

Cause:	The list of available emulators was not found in the MICEPACK.INI file.

Solution:	In MICEPACK.INI, check the emulator names in the "[Network]" section; the line starting with "emulators =" must contain a comma-separated list of one or more emulators available on the network.

# # $ K Unable to create host selection dialog.

Cause:	Internal error.

Solution:	Contact Microtekcontactingmicrotek technical support for help.

# # $ K PC-NFS is not running.

Cause:	PC-NFS must be resident for network communications to occur.  PC-NFS either hasn't been installed properly, or was not resident when MICEpack/SLD initialized.

Solution:	Install PC-NFS if necessary.  Activate PC-NFS prior to starting Windows.

# # $ K PC-NFS RTM is not resident.

Cause:	PC-NFS must be resident for network communications to occur.  PC-NFS either hasn't been installed properly, or was not resident when MICEpack/SLD initialized.

Solution:	Install PC-NFS if necessary.  Activate PC-NFS prior to starting Windows.

# # $ K PC-NFS TKLIB.DLL is not accessible.

Cause:	PC-NFS must be resident for network communications to occur.  PC-NFS either hasn't been installed properly, or was not resident when MICEpack/SLD initialized.

Solution:	Install PC-NFS if necessary.  Activate PC-NFS prior to starting Windows.

# # $ K PC_NFS RNM.DLL is not accessible.

Cause:	PC-NFS must be resident for network communications to occur.  PC-NFS either hasn't been installed properly, or was not resident when MICEpack/SLD initialized.

Solution:	Install PC-NFS if necessary.  Activate PC-NFS prior to starting Windows.

# # $ K Failed to establish communication endpoint.

Cause:	Internal error.

Solution:	Contact Microtekcontactingmicrotek technical support for help.

# # $ K Unable to access emulator address information.

Cause:	Internal error.

Solution:	Contact Microtekcontactingmicrotek technical support for help.

# # $ K Unable to access host address information.

Cause:	Internal error.

Solution:	Contact Microtekcontactingmicrotek technical support for help.

# # $ K Unable to establish network connection.

Cause:	The network communications failed to establish a connection.

Solution:

-	Verify the PC's and emulator's physical network connections.

-	Verify that the emulator is ready to receive connections (i.e., powered on and selftest completed).

# # $ K Network connection timed out.

Cause:	The network communications failed to establish a connection within the time-out period.

Solution:

-	Retry the operation.

-	Verify the PC's and emulator's physical network connections.

-	Verify that the emulator is ready to receive connections (i.e., powered on and selftest completed).

# # $ K The emulator is currently in use.

Cause:	The emulator is currently in use by another user.

Solution:	Only one user can use the emulator at a time.  You must wait for the current emulator user to finish, or bump the current user from the emulator.

Note that, if a user terminates a previous MICEpack/SLD session abnormally, the emulator may be still connected.  This spurious connection can be broken by physically resetting the emulator, or by overriding the connection.

Note that, if you override the current connection while the emulator is in use, the other user's MICEpack/SLD session is terminated.

# # $ K Network read operation failed.

Cause:	The network read operation failed.

Solution:

-	Retry the operation.

-	Verify the PC's and emulator's physical network connections.

-	Verify that the emulator is ready to communicate.

-	Verify network integrity and check network load.

-	Contact Microtek.

# # $ K Network write operation failed.

Cause:	The network write operation failed.

Solution:

-	Retry the operation.

-	Verify the PC's and emulator's physical network connections.

-	Verify that the emulator is ready to communicate.

-	Verify network integrity and check network load.

-	Contact Microtek.

# # $ K Network close operation failed.

Cause:	The network close operation failed.

Solution:

-	Retry the operation.

-	Verify the PC's and emulator's physical network connections.

-	Verify that the emulator is ready to communicate.

-	Verify network integrity and check network load.

-	Contact Microtek.

# # $ K Network read operation timed out.

Cause:	The network read operation did not complete within the time limit.

Solution:

-	Retry the operation.

-	Verify the PC's and emulator's physical network connections.

-	Verify that the emulator is ready to communicate.

-	Verify network integrity and check network load.

-	Contact Microtek.

# # $ K OS/2 LAN Server is not running.

Cause:	LAN Server must be resident for network communications to occur.  LAN Server either hasn't been installed properly, or was not resident when MICEpack/SLD initialized.

Solution:	Install LAN Server if necessary.  Activate LAN Server prior to starting Windows.

$ Processor type not supported.

# HE_NOT_CPU

K error

$ Cannot find module.

# HE_MODULE_NOT_FOUND

K error

$ Cannot add public symbol to symbol table: <symbol name>.

# HE_CANNOT_ADD_PUBLIC

K error

$ Out of memory during load.

# HE_NO_MEMORY

K error

$ Module time stamp is later than the original load file.

# HE_MODTIME

K error

$ User abort requested.

# HE_USER_ABORT

K error

$ Symbols already loaded, reload?

# HE_SYM_ALREADY

K error

$ No starting PC available.

# HE_NO_PC

K error

$ Register translation table contains invalid value.

# HE_REGTABLE

K error

$ User abort requested.

# HE_LDR_ABORT

K error

$ Cannot load code while processor running.

# HE_NORUNACCESS

K error

$ Cannot update PC, SP while processor running.

# HE_NOREG_UPDATE

K error

$ Symbols already loaded, specify 'reload' option.

# HE_SYM_ALREADY_CLI

K error

$ Loading is in progress, cannot perform the requested operation.

# HE_IN_PROGRESS

K error

$ Cannot find Windows directory.

# HE_CANNOT_FIND_WDIR

K error

$ Cannot find initialization file.

# HE_CANNOT_FIND_PFILE

K error

$ Tool not defined in initialization file.

# HE_NO_SECTION_NAME_DEFINED

K error

$ Invalid section name.

# HE_INVALID_SECTION_NAME

K error

$ Missing section name information.

# HE_MISSING_SECTION_NAME

K error

$ No compilers defined in initialization file.

# HE_NO_TOOLS_SUPPORT_DEFINED

K error

$ Cannot get compiler defined in initialization file.

# HE_CANNOT_GET_TOOL_NAME

K error

$ Missing compiler information in initialization file.

# HE_MISSING_TOOLS_SUPPORT

K error

$ Compiler name too long.

# HE_TOOL_NAME_TOO_LONG

K error

$ Cannot write to initialization file.

# HE_CANNOT_WRITE_TO_FILE

K error

$ Maximum number of sections reached; the rest will be ignored.

# HE_TOOMANY_SEC

K error

$ Incompatible load file processor type: <processor>.

# HE_INCOMPATIBLE_CPU

K error

$ Cannot find section defined in initialization file: <section name>.

# HE_DEFSEC_NOT_FOUND

K error

$ No line numbers in module: <module name>

# HE_NO_LINENUMS_ADDED

K error

$ Loader configuration file corrupted.

# HE_CONFIG_FILE_FORMAT

K error

$ Invalid frame number.

# HE_INVALID_FRAME

K error

$ Too many template fields for one line.

# HE_TOO_MANY_FIELDS

K error

$ Invalid search event name.

# HE_NO_SUCH_EVENT

K error

$ Timed out waiting for hardware to respond.

# HE_TIMED_OUT

K error

$ Cannot open the connection.

# HE_COM_OPEN

K error

$ Session terminated.

# HE_ABORT_SESSION

K error

$ Communication timeout sending to box.

# HE_COMM_PACKET_TIMEOUT

K error

$ Cannot establish communications with MICEpack

# HE_COMM_RETRY_EXHAUSTED

K error

$ Cannot initialize probe.

# HE_CANT_INIT_PROBE

K error

$ PowerScope probe version 1.4 or higher required for ROM breakpoints.

# HE_PROBE_VERSION_NO_XBRKS

K error

$ PowerScope probe version 1.4 or higher required for internal termination.

# HE_PROBE_VERSION_NO_RETRY

K error

$ Cannot read mapped memory type.

# HE_OVERLAY_HARDWARE

K error

$ Probe transmission error.

# HE_ILLEGAL_CMD

K error

$ Unterminated memory cycle; internal termination failed.

# HE_BUS_TIMEOUT

K error

$ Processor reset during BDM exchange.

# HE_RESET_DURING_BDM

K error

$ Processor clock speed is slower than 1 MHz.

# HE_SLOW_CLOCK

K error

$ Memory verification failed.

# HE_MEMORY_VERIFY

K error

$ Timed out waiting for emulation to stop.

# HE_WAIT_EMOFF_TIMEOUT

K error

$ Timed out waiting for single step.

# HE_STEP_TIMEOUT

K error

$ Fill command aborted prior to completion.

# HE_FILL_ABORT

K error

$ Timed out on reset entry to BDM.

# HE_WAIT_BDM_TIMEOUT

K error

$ BKPT pin was asserted spuriously.

# HE_CAUSE_STATUS

K error

$ Attempted halt failed.

# HE_HALT_TIMEOUT

K error

$ SIM register bank cannot be located.

# HE_SIM_UNREADABLE

K error

$ Trace instruction processing aborted.

# HE_DQ_ABORTED

K error

$ No bus cycles collected in trace buffer.

# HE_DQ_NO_FRAMES_AVAILABLE

K error

$ Trigger programming aborted.

# HE_TRIGGER_ABORTED

K error

$ Invalid trace buffer.

# HE_INVALID_BUFFER

K error

$ Unterminated memory access; internal termination used.

# HE_INTERNAL_TERMINATION

K error

$ Memory copy aborted.

# HE_COPY_ABORT

K error

$ Memory search aborted.

# HE_SEARCH_ABORT

K error

$ Search found at <address>.

# HE_SEARCH_FOUND

K error

$ Search not found.

# HE_SEARCH_NOT_FOUND

K error

$ Load Aborted.

# HE_LOAD_ABORT

K error

$ Probe firmware version incompatible with MICEpack/SLD.

# HE_PROBE_FW_VERSION

K error

$ Abort from Escape

# HE_ABORT_FROM_ESC

K error

$ Expected ATN record type.

# HE_ATN_EXPT

K error

$ Unrecognized miscellaneous record skipped

# HE_ATN_NOT_HANDLED

K error

$ Attribute type not supported.

# HE_ATTR_NOT_HANDLED

K error

$ Cannot read file.

# HE_BADGET

K error

$ Invalid or unsupported ID.

# HE_BADID

K error

$ Invalid module id.

# HE_BADMODULE

K error

$ Bad ATI record in Loadfile.

# HE_BAD_ATI_RECORD_IN_FILE

K error

$ Corrupted file.

# HE_BAD_FILE

K error

$ Invalid 695 file header.

# HE_BAD_HDR

K error

$ Invalid line number block in file.

# HE_BAD_LNUM_BLOCK_IN_FILE

K error

$ Nested block is not a procedure.

# HE_BAD_NEST

K error

$ Invalid part of file.

# HE_BAD_PART

K error

$ Invalid register index.

# HE_BAD_REG_INDEX

K error

$ Invalid section record.

# HE_BAD_SECREC

K error

$ Cannot seek load file.

# HE_BAD_SEEK

K error

$ Expected begin-block function record.

# HE_BBFUNCTION_EXPT

K error

$ Current function has no call or return instructions.

# HE_BX_NO_CALL_RETURN

K error

$ PC-NFS is not installed.

# HE_CLIULIB_PCNFS_NOT_INSTALLED

K error

$ The probe transmission is faster than BDM.

# HE_INPUT_OVERRUN

K error

$ Must define CompilerUsed information, specify 'CompilerUsed <compiler>'

# HE_MUST_DEFINE_TOOLUSE

K error

$ Cannot add module reference to symbol table.

# HE_NAME_NOT_ADDED

K error

$ Breakpoint already set.

# HE_BX_ALREADY_HAS_BKPT

K error

$ Invalid number of parameters.

# HE_DAD_BAD_NUM_PARM

K error

$ Operation not allowed while emulating.

# HE_ILLEGAL_WHILE_EMON

K error

$ CPU configuration file missing.

# HE_NO_CONFIG

K error

$ Optimized code detected: source view and variable access may be incorrect.

# HE_OPTIMIZED

K error

$ Invalid tab width.

# HE_SRC_INVALID_INPUT_TAB

K error

$ Stack high water measurement is disabled.

# HE_STACK_HW_DISABLED

K error

$ Cannot open a stack session.

# HE_STACK_NO_SESSION

K error

# HE_NET_PROFILE_MISSING

$ Unable to access communication type.

K error

# HE_NET_NOT_NETWORK

$ Network communication style not selected.

K error

# HE_NET_HOST_NAMES_NOT_FOUND

$ Unable to find network emulator host names.

K error

# HE_NET_HOST_SELECT_DIALOG

$ Unable to create host selection dialog.

K error

# HE_PCNFS_NOT_RUNNING

$ PC-NFS is not running.

K error

# HE_PCNFS_RTM_NOT_RESIDENT

$ PC-NFS RTM is not resident.

K error

# HE_PCNFS_TKDLL_NOT_FOUND

$ PC-NFS TKLIB.DLL is not accessible.

K error

# HE_PCNFS_RNM_NOT_FOUND

$ PC_NFS RNM.DLL is not accessible.

K error

# HE_NET_COMM_ENDPOINT

$ Failed to establish communication endpoint.

K error

# HE_NET_EMULATOR_INFO

$ Unable to access emulator address information.

K error

# HE_NET_HOST_INFO

$ Unable to access host address information.

K error

# HE_NET_CONNECT

$ Unable to establish network connection.

K error

# HE_NET_CONNECT_TIMEOUT

$ Network connection timed out.

K error

# HE_NET_EMULATOR_IN_USE

$ The emulator is currently in use.

K error

# HE_NET_READ

$ Network read operation failed.

K error

# HE_NET_WRITE

$ Network write operation failed.

K error

# HE_NET_CLOSE

$ Network close operation failed.

K error

# HE_NET_READ_TIMEOUT

$ Network read operation timed out.

K error

# HE_OS2_LAN_SERVER_NOT_RUNNING

$ OS/2 LAN Server is not running.

K error