# $ # K Stack Size is 0.  It will be reset to 1K.

Cause:	The stack size is set to 0, which is invalid.

Solution:	No action is necessary unless you want the stack to be larger than 1 KB.

See also

setStackArea Shell Commandsetstackareashell

# $ # K Stack area will be filled with a pattern to enable HighWaterMark.

Cause:	You have enabled the high water mark and MICEpack/SLD must fill the stack area in order to determine where the HighWaterMark is.

Solution:	This is information only.

See also

HighWaterMarkhighwatermark

# $ # K No stack frames available.

Cause:	The stack pointer may not be within the stack area.

Solution:	Check that the stack frame pointer and stack pointer are within the current stack area.

See also

setStackArea Shell Commandsetstackareashell

# $ # K Stack session name not found.

Cause:	The specified name is invalid.

Solution:	Specify a valid stack session name.

# $ # K SWAT operation aborted to change mode.

Cause:

Solution:	.

# $ # K SWAT hardware overflowed, information is missing.

Cause:

Solution:	.

# $ # K SWAT not responding to commands.

Cause:

Solution:	.

# $ # K The number of hardware breakpoints exceeds capability.

Cause:	The number of hardware breakpoints is already at the maximum.

Solution:	Remove one or more of the breakpoints.

# $ # K Cannot configure the probe hardware.

Cause:	PowerPack hardware failure.

Solution:	Contact Microtek.

See also

Contacting Microtekcontactingmicrotek

# $ # K Ethernet is not yet available.

Cause:	Internal error

Solution:	Contact Microtek.

See also

Contacting Microtekcontactingmicrotek

# $ # K Cannot add line number symbols.

Cause:	Line number records in this loadfile are not in a standard format.

Solution:	Check the compilerUsed setting; verify that you are compiling your program with full debug symbols

# $ # K Specified line number is out of range.

Cause:	(Internal error) Line number records have been corrupted.

Solution:	Try reloading symbols.  If this does not correct the problem contact Microtek.

See also

Contacting Microtekcontactingmicrotek

# $ # K Cannot create symbol type.

Cause:	Can't create a symbol type during load.  Possibly a module from a non-supported language is being linked in - either from your code, or libraries you've linked in (Pascal, Ada, and Fortran debug record types are not currently supported).

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell, compiler switches, compilers supportedtoolchains, changing compilerschangingcompilers.

Recommended Readingrecommendedreading

# $ # K Symbol not located in table - base index too large.

Cause:	If you get this error as a result of selecting a symbol from the Source window which is of the form:  "classname.memberFunction", you can safely ignore it.  Our Source tokenizer has not been completely converted to recognize C++ syntax, and misinterprets this token as a structure.member data item.

Solution:	You can perform operations on C++ class member functions.  To set a breakpoint, bring up the Set Breakpoint dialog for the module where the function is defined.  To inspect its source, you can select the GoTo Address menu item from the File menu in the Source window, and enter the symbolic address in the format: "#memberFunction".

There may be other reasons you would get this error - if you are not debugging a C++ program and get this error, please report it to Microtek.

# $ # K C++ duplicate name detected.

Cause:	C++ modules that contain virtual or overloaded functions may cause duplicate names to be generated as a result of demangling during symbol loading.  The mangled names for the second and successive instances of duplicates are stored in the symbol table.

Solution:	If you define one class per module to avoid this problem.  Also, you can shut off this warning by turning off warnings during load.

See also

Shell Load commandloadshellTekNotestechnicalnotes

# $ # K C++ type not handled: mangled names may result.

Cause:	A C++ type is not handled by MICEpack/SLD.  A "mangled" name may result, which is different than the source name of the symbol.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell, compiler switches, compilers supportedtoolchains, changing compilerschangingcompilers.

Recommended Readingrecommendedreading

# $ # K Loadfile compiled using <compiler> tool is incompatible with current Compiler Used.

Cause:	The current loadfile was compiled using a different compiler than the one selected.

Solution:	Choose Source/Options/ Compiler Used.  The resulting dialog box displays the current compiler toolchain.

Also, make sure that you used the correct compiler and linker switches.

This error can also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell, compiler switches, compilers supportedtoolchains, changing compilerschangingcompilers.

# $ # K Invalid register ID.

Cause:	A register ID in the current loadfile is invalid.

Solution:	Check Compiler Used and the switches on the compiler and linker command lines.  This could also be caused by a corrupted file, or a file not in IEEE695 format.

See also

Compiler Usedcompilerusedshell, compiler switches, compilers supportedtoolchains, changing compilerschangingcompilers.

# $ # K The NOT feature is not supported for this field.

Cause:	The NOT feature (BOOLEAN negation) is not allowed for a field.

Solution:	You must negate the value before entering it.

See also

# $ # K No active trigger found.

Cause:	There is no active trigger.

Solution:	.

See also

# $ # K Invalid tracing mode.

Cause:	The tracing mode selected is invalid.

Solution:	.

See also

# $ # K The variable was not found in Shell.

Cause:	The variable was not found.

Solution:	Check the spelling carefully, and remember to completely specify the variable.

See also

Shell variable commandsvariableshellcommands

# $ # K Cannot find address in symbol table.

Cause:	Attempted lookup of an address in the symbol table failed to find a matching symbol at this address.  Symbols may not have been loaded, or this address is out of the range of symbol addresses.

Solution:	Verify your program has been compiled and linked with debug symbols; verify the compilerUsed setting from the Shell.

# $ # K The attempt to edit the value has failed.

Cause:	The variable name, module name, function name, or value were specified incorrectly, so the variable value edit failed.

Solution:	Check the spelling carefully, and remember to completely specify the variable.

# $ # K Public symbol updated.

Cause:	Debug information for variables can occur several times in a loadfile.  When the first occurrence is an 'external' reference, the symbol table creates an initial entry for the symbol.  The later processing of debug records in the Public Part of an IEEE-695 loadfile enhances the symbol record with additional information.

Solution:	Not an error, an informational message.

# $ # K Cannot set end address smaller than start address.

Cause:	Obsolete error.

Solution:	Please report this problem to Microtekcontactingmicrotek.

# $ # K No symbols are loaded.

Cause:	Full symbolic information has not been loaded (enabling source-level debugging).

Solution:	If you have an assembly language program, only public symbols are loaded, resulting in this message.  If you have high-level language modules in your program (C/C++ source files), be sure you have specified debug symbols in your compilation switches.  Also, enter the Shell command compilerUsed to verify that your toolchain is specified.

# $ # K Invalid trigger event index.

Cause:	Internal error.  No user error.

Solution:	Please report this problem to Microtekcontactingmicrotek.

# $ # K Cannot initialize stack area for HighWaterMark.

Cause:	Internal error.  No user error

Solution:	Please report this problem to Microtekcontactingmicrotek.

# $ # K Cannot set section base in symbol table.

Cause:	Section bases may already be defined, or, your application may be defining more than the maximum number of sections supported (in which  case the remainder are discarded).

Solution:	To explicitly clear out sections defined by previous loadfiles (which should automatically happen when you load a new program), you can use the removesymbols Shell command.  If you have more than 32 sections defined in your application, you can merge sections by the mergeSection Shell command (which is adequate for source-level debugging, but may not be adequate for section relocation).

# # $ K No module to view since no file is loaded.

Cause:	The source and the symbol table are out of synchronization.

Solution:	Reload the symbols, then close/reopen the Source window.

# $ # K No module to view since no file is loaded.

Cause:	No loadfile has been loaded, so there is no source module available to browse.

Solution:	Load a file, then try again.

# $ # K Module does not have a valid address range.

Cause:	Usually, a module doesn't have a valid address range if it is a data module (i.e. if the module contains only data definitions).

Solution:	Add a dummy function to the data module, and recompile it, to force the compiler to generate an address range for the module and allow you to view the module in the Source window.

# $ # K Invalid symbol address type.

Cause:	The text you entered does not correspond to any recognized form of symbolic address.  The input conventions for the types of address are:

1.	Hex values, which must begin with 0x -- such as 0x11.

2.	Decimal values, using only the digits 0..9.

3.	Symbolic addresses, which must begin with a '#'.

Solution:	Try re-entering the address using these conventions.

See also

Symbol Entry Syntaxsymbolentrysyntax

# $ # K No source module(s) loaded.

Cause:	No source modules have been loaded.

Solution:	Try reloading, and if you have high-level language modules in your program (C/C++ source files), be sure you have specified debug symbols in your compilation switches.  Also, verify that compilerUsed reflects the toolchain you are using (use the Shell compilerUsed command).

# $ # K Peripheral configuration file not found.

Cause:	The peripheral configuration file is not in the specified directory.

Solution:	Be sure you specified the correct filename and path.  (If you did not specify a path, the MICEpack/SLD directory was searched for this file.)

# $ # K Address addition overflowed.

Cause:	One situation that can cause this error to be generated is dereferencing a structure pointer that already points near the end of memory.

Solution:	Be sure data values for your local variables are properly initialized before dereferencing their values.

# $ # K Peripheral register is unknown.

Cause:	The register name does not conform to the Motorola 68000 mnemonics.

Solution:	Check the register name.

# $ # K Corrupted peripheral configuration file found.

Cause:	File is corrupted.

Solution:	Edit the file or re-create the file.

# $ # K Unable to close peripheral configuration file.

Cause:	The file for saving or restoring peripherals cannot be closed.

Solution:	A DOS error has occurred.  Check the file and try again.

# $ # Unexpected end of peripheral configuration file found.

Cause:	The peripheral configuration file is corrupted in the MICEpack/SLD default directory (e.g. c:\MICEPACK).

Solution:	The peripheral configuration file has been truncated.  Re-install the MICEpack/SLD software.

# $ # K Register found with unknown peripheral group.

Cause:

Solution:

# $ # K Attempted to edit register with an invalid value.

Cause:	You entered an invalid register value.

Solution:	Re-edit the register with the correct value.

# $ # K Encountered peripheral configuration EOF.

Cause:

Solution:

# $ # K TRegister object failed to construct during initialization.

Cause:

Solution:

# $ # K TField object failed to construct during initialization.

Cause:

Solution:

# $ # K TGroup object failed to construct during initialization.

Cause:

Solution:

# $ # K Peripheral window creation failed.

Cause:

Solution:

# $ # K Unable to create peripheral display cache.

Cause:

Solution:

# $ # K Attempted to add a display cache entry out of sequence.

Cause:

Solution:

# $ # K Display cache entry found has no match.

Cause:

Solution:

# $ # K Reached end of available cache.

Cause:

Solution:

# # $ K + Expected line number block.

Cause:	Module contains only data definitions, which causes the compiler not to generate a line number block for that module.  Usually, you cannot browse the module in the Source window.

Solution:	Add a dummy function to the module and recompile it to force the compiler to generate a line number block for the module.  For example, add the following dummy function: void dummy Function() {/* do nothing */}

# # $ K Unable to open peripheral configuration file.

Cause:	A peripheral configuration file is missing or invalid.

Solution:	If you have moved MICEpack/SLD after your initial installation, make sure to copy the peri????.cfg and *.pcf files to the new location.

# # $ K Location of peripheral block is indeterminate.

Cause:	The peripheral registers of the 68330, 340, and 360 are inaccessible after the processor is reset.

Solution:	To enable their access, the MBAR  (Module Base Address Register) bit 0 (Valid bit) must be set to 1.

This register also programs the starting address of the peripheral registers - on 4K boundary for the 330 and 340, and 8K boundary for the 360.  This register is only accessible in cpu space.  MICEpack/SLD can only access cpu space with the Shell write command.  Assuming that  you want the peripheral registers to start at address 0xFFFF0000, the Shell command would be:

write 0x3FF00 0xFFFF0001 long cpu;

where the 0x3FF00 is the address of the MBAR register, long does a 32 bit

write, and cpu specifies the cpu space.

You can create an include file that you can then call after the processor has been reset that will set up the MBAR register.  Simply edit a file with the above command line, then issue the Shell command:

include "<path\filename>";

For example, suppose the command is in the file "reset.inc" in the MICEPACK directory.  You would type:

include "c:\MICEPACK\reset.inc";

# # $ K + Module %s does not contain line number information.

Cause:	The module was compiled without debug information.  You cannot browse the module in the Source window.

Solution:	Recompile with the debug information switches turned on.

# # $ K + No CompilerUsed defined in MICEPACK.INI file.

Cause:	Either CompilerUsed was accidentally deleted, or MICEPACK.INI is corrupted.

Solution:	Edit the MICEPACK.INI. file and copy the following text under the [ToolChain section].

[ToolChain]

MergeSections=0

compilers=Unknown, MRI,Intermetrict,Whitesmiths,Sierra,Introl

CompilerUsed=MRI

MRI=code,vars

# # $ K + Peripheral configuration file version is newer than one this version of MICEpack/SLD can read.

Cause:	This version of the configuration file data format is higher than one which this version of MICEpack/SLD can read.

Solution:	Either use the older peripheral configuration files that came with this version of MICEpack/SLD, or upgrade the version of MICEpack/SLD to the latest version.

# # $ K + Unexpected type: %s.

Cause:	The loader found a data type that is not defined by the IEEE-695 specification; therefore, it is invalid.

Solution:	Contact Microtek customer servicereportingproblemsto>main.

# # $ K + Memory Window Count Exceeded

Cause:	Since the maximum number of Memory windows are already open, you cannot open another new Memory window.

Solution:	Use one of the open Memory windows.

# # $ K + Unable to open Memory Window.

Cause:	Opening another Memory window failed because there are insufficient system resources.

Solution:	Close othher MICEpack/SLD windows or other Windows applications to free system resources, and then try again to open a new Memory window.

# # $ K Space designator is incompatible with current processor.

Cause:  Space designators are processor specific.

Solution:	Motorola space designators are SP, SD, UP, and UD.

# # $ K Address mode does not conform to processor standards.

Cause:	Address mode is processor specific.

Solution:	Intel modes are SMM, Real, Protected16, Protected32, and Virtual86.

# # $ K Address offset exceeds processor's addressing limit.

Cause:	Address is out of bounds.

Solution:	Use the Limit Checking Tablelimitcheck to verify the address.

# # $ K Limit Checking Table

| Type | Pmode | Processor | Limits |
| --- | --- | --- | --- |
| Virtual | SMM | all | 0:0 - FFFF:FFFFFFFF |
|  | Real | all | 0:0 - FFFF:FFFF |
|  | Virtual-86 | all | 0:0 - FFFF:FFFF |
|  | Protect16, 32 | all | selector £ table limit; offset within segment limit |
| Linear | all | all | 0 - FFFFFFFF |
| Physical | all | 386DX | 0 - FFFFFFFF |
|  | all | 386SX | 0 - FFFFFF |
|  | all | 386CX | 0 - 3FFFFFF |
|  | all | 386EX | 0 - 3FFFFFF |

# # $ K Address offset exceeds processor's segment limit.

Cause:	Address offset is out of bounds for the given segment.

Solution:	Use the Limit Checking Tablelimitcheck to verify the address.

# # $ K CPU stopped due to accessing NONE memory.

Cause:	NONE memory doesn’t allow any read/write action on it.

Solution:	Make sure the map setting is what you expect

# # $ K Unable to translate physical address to linear address.

The address server does not support physical to linear address translation.

Internal error.  Contact Microtekcontactingmicrotek.

# # $ K Unable to translate linear address to virtual address.

Cause:	There is no information available for the address translation.

Solution:	Update symbolic information.

# # $ K Unable to compare addresses of differing segments.

Cause:	Addresses compared do not have the same segment.

Solution:	Specify addresses of the same segment for comparison purposes.

# # $ K Unable to compare addresses of differing types.

Cause:	Addresses compared do not have the same type.

Solution:	Specify addresses of the same type for comparison purposes.

# # $ K Unable to compare addresses of differing spaces.

Cause:	Addresses compared do not have the same space.

Solution:	Specify addresses of the same space for comparison purposes.

# # $ K Requested operation is incompatible with processor

Cause:	A processor-specific routine was accessed while operating with the wrong processor family.

Solution:	Internal error.  Contact Microtekcontactingmicrotek.

# # $ K CPU stopped because of writing data to ROM memory.

Cause:	Writing data on ROM memory area.

Solution:	Check map setting.

# # $ K Requested offset in the descriptor table exceeds the physical limits.

Cause:	Indexing into a descriptor table with the specified segment selector resulted in an offset exceeding the physical limits of the table.

Solution:	The upper 13 bits of the segment selector contains the descriptor table index.  Specify an index inside the descriptor table limit.  Specify an index into the descriptor table which is less than the limit.

# # $ K Specified segment is no longer valid or present in memory.

Cause:	Specified segment contains a cleared P-bit.

Solution:	The system deallocated this segment and it needs to be loaded into memory before use.  A segment not present exception will be generated if the descriptor selector is loaded into a segment register.  Do not use the segment.

# # $ K Segment specified is not a valid type for requested use.

Cause:	The segment specified may be a system segment and not a valid code or data segment.

Solution:	Use valid code and data segments for addresses.  When the S bit is set in the segment descriptor it is a valid code or data segment.

# # $ K The page table entry is not present during page translation.

Cause:	The present bit in the page table entry is 0 and the page is not in memory.  During address translation, the accessed page table entry had a cleared present bit.  The page table entry cannot be used for physical address translation.

Solution:	Update the page tables.

# # $ K The pmode is uninitialized and prevents address translation.

Cause:	Address translation depends on current processor mode.  The mode is uninitialized and the address can not be translated.

Solution:	Initialize processor mode prior to address translation requirements.  To initialize the processor, operate it in one of the following legal modes:  Real, V86, Prot16, Prot32, SMM.

# # $ K Shell command specified is not supported for the current processor.

Cause:	The shell command is processor-specific and is not supported on the current processor.

Solution:	Search Help index for "shell" for legal commands for the current processor.

# # $ K MBAR is invalid; no other SIM registers will be processed.

Cause:

Solution:

# # $ K Invalid pod version.

Cause:

Solution:

# # $ K Timeout waiting for processor to enter ice mode.

Cause:

Solution:

# # $ K Timeout waiting for processor to respond to commands.

Cause:

Solution:

# # $ K Timeout waiting for processor to process breakpoint.

Cause:

Solution:

# # $ K Insufficient overlay memory.

Cause:

Solution:

# # $ K Emulation adapter is not connected properly to target.

Cause:

Solution:

# # $ K Cannot drive target processor FLT# pin to low State."

Cause:

Solution:

# # $ K Target power is off.

Cause:

Solution:

# # $ K Cannot drive emulation processor FLT# pin to high state.

Cause:

Solution:

# # $ K No clock input from target.

Cause:

Solution:

# # $ K Processor reset during operation.

Cause:

Solution:

# # $ K Hold is continuously asserted.

Cause:

Solution:

# # $ K Read operation aborted prior to completion.

Cause:

Solution:

# # $ K Test operation aborted prior to completion.

Cause:

Solution:

# # $ K Unknown test command received by emulator.

Cause:

Solution:

# # $ K Unsupported event recognizer type.

Cause:

Solution:

$ Stack Size is 0.  It will be reset to 1K.

# HE_STACK_SIZE_0

K error

$ Fill stack area to enable HighWaterMark.

# HE_STK_FILL_STACK_AREA

K error

$ No stack frames available.

# HE_STK_NO_FRAMES

K error

$ Stack session name not found.

# HE_STK_SESSION_NAME_NOT_FOUND

K error

$ SWAT operation aborted to change mode.

# HE_SWAT_ABORT

K error

$ SWAT hardware overflowed, information is missing.

# HE_SWAT_PA_OVERFLOW

K error

$ SWAT not responding to commands.

# HE_SWAT_TIMEOUT

K error

$ The number of hardware breakpoints exceeds capability.

# HE_TOO_MANY_XBRKS

K error

$ Cannot configure the probe hardware.

# HE_XILINX_PROGRAMMING

K error

$ Ethernet is not yet available.

# HE_ETHERNET_NOT_SUPPORTED

K error

$ Cannot add line number symbols.

# HE_LINENUM_BAD_STATE

K error

$ Specified line number is out of range.

# HE_VS_RANGE

K error

$ Cannot create symbol type.

# HE_SYM_TYPE

K error

$ Symbol not located in table - base index too large.

# HE_TABLE_SMALLER_THAN_INDEX

K error

$ C++ duplicate name detected.

# HE_CPP_DUPLICATE

K error

$ C++ type not handled: mangled names may result.

# HE_CPP_NOT_SUPPORTED

K error

$ Loadfile compiled using <compiler> tool is incompatible with current Compiler Used.

# HE_LOADFILE_HAD_INCOMPATIBLE_TOOLCHAIN

K error

$ Invalid register ID

# HE_INVALID_REGID

K error

$ The NOT feature is not supported for this field.

# HE_NOT_NOT_ALLOWED

K error

$ No active trigger found.

# HE_NO_TRIG_DEFINED

K error

$ Invalid tracing mode.

# HE_INVALID_MODE

K error

$ The variable was not found in Shell.

# HE_VS_CLI_VAR_NOT_FOUND

K error

$ Cannot find address in symbol table.

# HE_ADDRESS_NOT_FOUND

K error

$ The attempt to edit the value has failed.

# HE_VS_VALU_EDIT_FAIL

K error

$ Public symbol updated.

# HE_ADDRESS_UPDATED

K error

$ Cannot set end address smaller than start address.

# HE_ADR_END_ADDR_TOO_SMALL

K error

$ No symbols are loaded.

# HE_SYMBOL_NOT_A_MODULE

K error

$ Invalid trigger event index.

# HE_BAD_EVENT

K error

$ Cannot initialize stack area for HighWaterMark.

# HE_STK_INIT_STK_FAIL

K error

$ Cannot set section base in symbol table.

# HE_SYMBASE

K error

# HE_SRC_FIND_ADDRESS

$ No module to view since no file is loaded.

K error

$ No module to view since no file is loaded.

# HE_SRC_NO_LOADFILE

K error

$ Module does not have a valid address range.

# HE_SRC_MODULE_HAS_NO_ADDR_RANGE

K error

$ Invalid symbol address type.

# HE_SYM_SYM_START_NOT_SHARP

K error

$ No source module(s) loaded.

# HE_SYM_NO_MODULES_LOADED

K error

$ Peripheral configuration file not found.

# PP_NO_CFG_STR

K error

$ Address addition overflowed.

# HE_ADR_RESULT_OVERFLOW

K error

$ Peripheral register is unknown.

# PP_UNK_REG_STR

K error

$ Corrupted peripheral configuration file found.

# PP_CORR_CFG_STR

K error

$ Unable to close peripheral configuration file.

# PP_CLOSE_CFG_STR

K error

$ Unexpected end of peripheral configuration file found.

# PP_UNEX_CFG_STR

$ Register found with unknown peripheral group.

# PP_UNK_GRP_STR

K error

$ Attempted to edit register with an invalid value.

# PP_INV_VALUE_STR

K error

$ Encountered peripheral configuration EOF.

# PP_CFG_EOF_STR

K error

$ TRegister object failed to construct during initialization.

# PP_TREG_FAIL_STR

K error

$ TField object failed to construct during initialization.

# PP_TFLD_FAIL_STR

K error

$ TGroup object failed to construct during initialization.

# PP_TGRP_FAIL_STR

K error

$ Peripheral window creation failed.

# PP_WIN_FAIL_STR

K error

$ Unable to create peripheral display cache.

# PP_NO_CACHE_STR

K error

$ Attempted to add a display cache entry out of sequence.

# PP_INV_CACHE_ENT_STR

K error

$ Display cache entry found has no match.

# PP_UNK_CACHE_ENT_STR

K error

$ Reached end of available cache.

# PP_END_CACHE_STR

K error

# HE_LINE_EXPT

$ Expected line number block.

K error;

+ ERRORS4

# HE_PP_OPEN_CFG_FAIL

$ Unable to open peripheral configuration file.

K error;

# HE_PP_INVALID_BASE

$ Location of peripheral block is indeterminate.

K error

# HE_NO_LINEBLK_IN_MODULE

$ Module %s does not contain line number information.

K error;

+ ERRORS4

# HE_NO_TOOLUSE_DEFINED

$ No CompilerUsed defined in PWRVIEWS.INI file.

K error;

+ ERRORS4

# HE_PP_OLD_VERSION

$ Peripheral configuration file version is newer than one this version of MICEpack/SLD can read.

K error;

+ ERRORS4

# HE_UNEXPECTED_TYPE

$ Unexpected type: %s.

K error;

+ ERRORS4

# HE_MEM_COUNT_EXCEEDED

$ Memory Window Count Exceeded

K error

+ ERRORS4

# HE_CANT_OPEN_MEM_WINDOW

$ Unable to open Memory Window.

K error

+ ERRORS4

# HE_ADR_SPACE_INVALID

$ Space designator is incompatible with current processor.

K error;

# HE_ADR_MODE_INVALID

$ Address mode does not conform to processor standards.

K error;

# HE_ADR_EXCEEDS_ADR_LIMIT

$ Address offset exceeds processor's addressing limit.

K error;

# limitcheck

$ Limit Checking Table

K addresses;limit checking;

# HE_ADR_EXCEEDS_SEG_LIMIT

$ Address offset exceeds processor's segment limit.

K error;

# HE_ADR_LDT_INVALID

$ LDT specified does not exist.

K error;

# HE_ADR_PHYS_TO_LINEAR_NOT_SUPPORTED

$ Unable to translate physical address to linear address.

K error;

# HE_ADR_LINEAR_TO_VIRTUAL_INVALID

$ Unable to translate linear address to virtual address.

K error;

# HE_ADR_DIFFER_SEGMENTS

$ Unable to compare addresses of differing segments.

K error;

# HE_ADR_DIFFER_TYPES

$ Unable to compare addresses of differing types.

K error;

# HE_ADR_DIFFER_SPACES

$ Unable to compare addresses of differing spaces.

K error;

# HE_PRC_OPER_INCOMPAT

$ Requested operation is incompatible with processor

K error;

# HE_ADR_NULL_GDT_DESCRIPTOR

$ The GDT null descriptor is invalid for use in an address.

K error;

# HE_ADR_DESCTBL_LIMIT_EXCEEDED

$ Requested offset in the descriptor table exceeds the physical limits.

K error;

# HE_ADR_SEGMENT_INVALID

$ Specified segment is no longer valid or present in memory.

K error;

# HE_ADR_SEGMENT_USE_INVALID

$ Segment specified is not a valid type for requested use.

K error;

# HE_ADR_PAGE_UNAVAILABLE

$ The page table entry is not present during page translation.

K error;

# HE_ADR_PMODE_UNAVAILABLE

$ The pmode is uninitialized and prevents address translation.

K error;

# HE_ADR_SHELL_NOT_SUPPORTED

$ Shell command specified is not supported for the current processor.

K error;

# HE_SAVECS_MBAR_INVALID

$ MBAR is invalid; no other SIM registers will be processed.

K error;

# HE_POD_VERSION

$ Invalid pod version.

K error;

# HE_ICE_MODE_TIMEOUT

$ Timeout waiting for processor to enter ice mode.

K error;

# HE_ICE_CMD_TIMEOUT

$ Timeout waiting for processor to respond to commands.

K error;

# HE_ICE_BKPT_TIMEOUT

$ Timeout waiting for processor to process breakpoint.

K error;

# HE_INSUF_MAP_BLOCKS

$ Insufficient overlay memory.

K error;

# HE_TARGET_CONNECT

$ Emulation adapter is not connected properly to target.

K error;

# HE_TARGET_FLT

$ Cannot drive target processor FLT# pin to low State."

K error;

# HE_TARGET_VCC

$ Target power is off.

K error;

# HE_BONDOUT_FLT

$ Cannot drive emulation processor FLT# pin to high state.

K error;

# HE_NO_CLOCK

$ No clock input from target.

K error;

# HE_RESET_OCCURRED

$ Processor reset during operation.

K error;

# HE_HOLD_STUCK

$ Hold is continuously asserted.

K error;

# HE_READ_ABORT

$ Read operation aborted prior to completion.

K error;

# HE_TEST_ABORT

$ Test operation aborted prior to completion.

K error;

# HE_UNKNOWN_TEST_CMD

$ Unknown test command received by emulator.

K error;

# HE_EVTREC_TRACE

$ Unsupported event recognizer type.

K error;