# $ # + K Trigger Window

The Trigger Window allows bus eventseventterm and PC Breakpoint to cause system break.

Menu Selections

File

Exit	Close the Trigger window.

Edit

Events...	Go to the Event Windowevent.

PC Breakpoint...	Define the PC Breakpoint.

Windows	Move between windowsmovingbetweenwindows

Help	On-line helponlinehelp

Using the Trigger Window

Trace and Trigger Fundamentalstraceandtriggerfundamentals

Creating Eventscreatingevents

See also

Trigger Shell Commandstriggershellcommands

# $ # + K Trace and Trigger Fundamentals

The trace features are:

-	32k bytes trace buffer.

-	Trace is collected on each execution instruction.

-	Synchronize trace frames with source using the linked cursorlinkedcursor.

The trigger features are:

-	The bus eventseventterm can be specified with either range address, mask data, or processor status, or their combination thereof.

-	Single or double-level triggers.

Use trace to:

-	Record the real-time execution of code.

-	Linklinkedcursor the trace execution to the source.

Use triggers to:

-	Stop emulation on an eventevent.

-	Control collection of trace.

-	Detect complex sequences of events.

-	Detect a subroutine that is taking too much timeusingcountersand.

See also

Event Windowevent

Trace Windowtrace

Trigger Windowtrigger

# $ # + K Triggering on Access to a Variable

To trigger on a variable:

1.	In the Event Windowevent, create an eventcreatingevents:

a.	Enter the symbolic name in the address start field.

b.	Enter a specific data value or leave the data fields blank to trigger on any value.

c.	Select 1 in the r/w- field for trigger on read, 0 for write, or leave as X to trigger on a read or a write.

2.	In the Trigger Windowtrigger, set the trigger:

a.	Choose the event name from the event name list.

b.	Click on enable to activate the event.

3.	Start emulationemulationterm:

a.	Emulation stops when the trigger is detected.

See also

Trace and Trigger Fundamentalstraceandtriggerfundamentals

Symbol Entry Syntaxsymbolentrysyntax

# $ # + K Trigger Examples

Many of the trigger subjects have examples.  Some additional ones are:

Stopping Emulation When Invalid Stack Area Accessedstoppingemulationwhen

Triggering on Access to a C Variabletriggeringonaccess

See also

Trigger Windowtrigger  for all the trigger subjects.

# $ # K Trigger Shell Commands

There are no Shell commands for trigger.  Use the Trigger Windowtrigger graphical interface.

# $ # K Trigger

A trigger causes emulation break when eventseventterm are matched on specified trigger levels.  Triggers allow the detection of real-time bus activity and the making of a real-time decision.

For example, "halt emulation when a variable is written" is a trigger.

$ Trigger Window

# trigger

+ powerviews:0120

K trigger;window

$ Trace and Trigger Fundamentals

# traceandtriggerfundamentals

+ technical:0250

K trace;trigger;event

$ Triggering on Access to a C Variable

# triggeringonaccess

+ technical:0260

K trigger;C and C++;variable

$ Trigger Examples

# triggerexamples

+ usingthetrigger:0080

K trigger

$ Trigger Shell Commands

# triggershellcommands

K trigger;shell

$ Trigger

# triggerterm

K trigger;event trigger