# $ # + K Stack Window

The Stack Window displays the function call stack.

Screen Description

The upper-left pane shows the functions in the call stack.  The lower pane shows the variables for the highlighted stack frame.  Variables which are dead are shown in black.  The meter shows the amount of stack usage.

If the current stack frame shows an assembly language function (for which there is no stack information), then the frames below may be inaccurate.

Menu Selections

File

Refresh Display	Updateupdatestack the call stack, variable information, and meter.

Exit	Close the Stack Window.

Options

Stack Areasettingstackarea...	Set the upper memory boundary of the stack and the number of bytes in the stack.

Alarm Limitstackalarm...	Set a percentage of stack meter.

Include Stack Address	Show stack addresses in stack display.

Include Code Address	Show code addresses in stack display.

Enable High-Water Markhighwatermark

An arrow points to the meter to indicate the highest address reached on the stack.

Enable Alarm Limit	Check for stack overflow.  Show alarm on stack as a red line across the meter.

Inspect Sourceinspectsource	Scroll the Source Window to the selected function.

Windows	Move between windowsmovingbetweenwindows

Help	On-line helponlinehelp

Using the Stack Window

High-Water Mark in Stackhighwatermark

Navigating the Stacknavigatingthestack

Setting the Stack Areasettingstackarea

Stack Alarmstackalarm

Stack Overflow Messagestackoverflowmessage

When is Stack Valid?whenisstack

See also

Dereferencing Pointersdereferencingpointers

Stack Shell Commandsstackshellcommands

Stopping Emulation When Invalid Stack Area Accessedstoppingemulationwhen

# $ # + K Setting Stack Area

The supported compilerstoolchains automatically set the stack base and the stack size.  When using an unsupported compiler:

-	Determine the initial stack address and the number of bytes in the stack.

-	Use Options/Stack Area... to set the stack base and the number of bytes in the stack.  The stack base is the upper memory boundary of the stack.

The stack's memory range is from stack base to stack base minus stack size plus 1.  For example, if the stack base is 0x5000 and the stack size is 0x1000, stack memory ranges from 0x4FFF to 0x4000 (the stack pointer is decremented when pushing).

See also

Compiler Supporttoolchains

# $ # + K Navigating the Stack

The upper pane displays the function call stack.  Selecting a function highlights the line and displays the function's Parameters & Local Variables in the bottom pane.

Some of the editing featuresviewingeditingand of the Variable Windowvariable are available in the Parameter & Local Variables pane, including:

-	Dereferencing.

-	Value editing.

See also

High-Water Mark in Stackhighwatermark

Stack Alarmstackalarm

# $ # + K Stack Alarm

The stack alarmstackalarmterm displays a warning when emulationemulationterm stops if the stack exceeds a preset value.  This value is the percentage of stack currently used.

To set the alarm:

-	Set the percentage of stack using Options/Alarm Limit.  If the stack exceeds this point, an alarm message is displayed when emulation breaks.  The initial default is 95%.  Setting the alarm limit automatically enables the alarm.

-	Choose Options/Enable Alarm Limit to enable or disable the alarm.

The meter displays a red line at the alarm limit.

The alarm is active when emulation breaks.  The alarm is not active during emulation.  If the alarm is exceeded during emulation, no error message is displayed.  Use the high-water markhighwatermark feature or a triggertrigger to detect this condition.

In addition to the stack alarm and high-water mark features, a stack overflow messagestackoverflowmessage is always displayed if the stack pointer is outside the stack range when emulation halts.

See also

High-Water Mark in Stackhighwatermark

Stack Overflow Messagestackoverflowmessage

Trigger Windowtrigger

# $ # + K High-Water Mark in Stack

When code is loaded, the stack is filled with a pattern.

-	The high-water markhighwatermarkterm checks the stack memory when emulationemulationterm halts.

-	The memory location furthest from the stack's upper memory boundary is the high-water mark.

-	An arrow at the right side of the meter points to the high-water mark.

To use the high-water mark:

-	Choose Options/Enable High-Water Mark.

The high-water mark is checked when emulation halts.  To detect a stack overflow in real time, use a triggertrigger.

Note, in addition to the stack alarmstackalarm and high-water mark features, a stack overflow messagestackoverflowmessage is always displayed if the stack pointer is outside the stack range when emulation halts.

See also

Stack Alarmstackalarm

Stack Overflow Messagestackoverflowmessage

Trigger Windowtrigger

# $ # + K Stack Overflow Message

A stack overflow message is displayed whenever the stack pointer is outside the stack range.  The range is computed when code is loaded from Options/Base Address... and Options/Stack Size....

This message usually appears on resetresettingtheprocessor, since the stack pointer is reset.

See also

High-Water Mark in Stackhighwatermark

Setting Stack Areasettingstackarea

Stack Alarmstackalarm

# $ # + K When is Stack Valid?

The stack is not valid until the stack pointer is initialized, which is usually one of the first initialization steps in a program.  The Stack Window does not have valid information until the stack pointer is initialized.

See also

Setting Stack Areasettingstackarea

# $ # K Stack Shell Commands

Stack related commands are:

disableAlarmLimitdisablealarmlimitshell

disableHighWaterMarkdisablehighwatermarkshell

displayStackdisplaystackshell [locals | hex]

enableAlarmLimitenablealarmlimitshell

enableHighWaterMarkenablehighwatermarkshell

fillStackPatternfillstackpatternshell

setStackAlarmsetstackalarmshell <% of stack>

setStackAreasetstackareashell [<address> <stack size> [fillArea]]

setStackBasesetstackbaseshell <address>

setStackSizesetstacksizeshell <stack size>

stackInfostackinfoshell

See also

Shell Command List (alphabetic)shellcommandlist

Shell Command List (topical)shellcommandlisttopical

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Alarm Limit

A percentage of the stack which can cause a stack alarmstackalarmterm to be displayed if exceeded.

The percentage ranges from the stack base (0%) to the stack base plus stack size (100%).

# # $ K Stack Alarm Definition

When emulationemulationterm stops, the current stack pointer is compared to a percentage of the total stack.  The stack alarm displays an informational box if the percentage is exceeded.

To set the alarm, use the Stack Window'sstack Options/Alarm Limit or Options/Enable Alarm Limit.

The alarm limit appears as a red line in Stack Window's meter.

# $ # K High-Water Mark

The arrow at the right side of the Stack Window's meter, which indicates how much of the stack memory has been used.

When code is loaded, the stack memory is filled with 0xAA55.  Each time emulationemulationterm halts, the memory is searched.  Stack pushes and pops overwrite the 0xAA55.  MICEpack/SLD searches for the last memory location containing 0xAA55 to determine how high the stack got.

Choose Options/Enable High-Water Mark to enable this feature.

See also

FillStackPattern Shell Commandfillstackpatternshell

# # $ K Stack Area Dialog Box

If the compiler does not automatically provide these values, use this dialog box to set:

-	The base address, or upper memory boundary of the stack.

-	The size of the stack area.

Prefix a hexadecimal value with 0x (e.g., 0x4F).

See also

Setting the Stack Areasettingstackarea

# # $ K Alarm Limit Dialog Box

Enter a percentage of the stack.  When emulation stops, an informational message is displayed if the stack pointer exceeded this percentage.

See also

Setting Stack Areasettingstackarea

Stack Alarmstackalarm

# # $ K DisableAlarmLimit Shell Command

Disable the stack alarmstackalarmterm.

The Stack Window'sstack Options/Enable Alarm Limit is equivalent.

Syntax

disableAlarmLimit

Parameters

none

Examples

Disable stack alarm.

> disablealarmlimit

Show current alarm setting.

> stackinfo

// stack base = 0x001E1A

// size = 1000

// current stack pointer = 0x001DE6

// alarm limit = 79%, DISABLED

// high water mark = 0x001DE6

// stack type = high to low

See also

DisableHighWaterMark Shell Commanddisablehighwatermarkshell

EnableAlarmLimit Shell Commandenablealarmlimitshell

EnableHighWaterMark Shell Commandenablehighwatermarkshell

SetStackAlarm Shell Commandsetstackalarmshell

Shell Syntax Conventionsshellsyntaxconventions

StackInfo Shell Commandstackinfoshell

# # $ K DisableHighWaterMark Shell Command

Disable high-water markhighwatermarkterm.

The Stack Window'sstack Options/Enable High-Water Mark is equivalent.

Syntax

disableHighWaterMark

Parameters

none

Examples

Disable high-water mark.

> disablehighwatermark

Show current setting.

> stackinfo

// stack base = 0x001E1A

// size = 1000

// current stack pointer = 0x001DE6

// alarm limit = 79%, DISABLED

// high water mark = disabled

// stack type = high to low

See also

DisableAlarmLimit Shell Commanddisablealarmlimitshell

EnableAlarmLimit Shell Commandenablealarmlimitshell

EnableHighWaterMark Shell Commandenablehighwatermarkshell

Shell Syntax Conventionsshellsyntaxconventions

StackInfo Shell Commandstackinfoshell

# # $ K DisplayStack Shell Command

Display the current function call stack.  The Stack Window must be open to display the stack.

The Stack Windowstack is equivalent.

Syntax

displayStack [locals | hex]

Parameters

locals	Display local variables and parameters for each function.

hex	Dump stack in hexadecimal.

Examples

Display the stack.

> displaystack;

// 001DF6 0004A6 QMCR_Config

// 001DFA 000468 QSM_setup

// 001E02 0007BE main

Display the stack with locals.

> displaystack locals;

// 001DF6 0004A6 QMCR_Config

// int QMCR_Config#FRZ1_MASK = 0xFFFFFFFF = -1;

// int QMCR_Config#IARB_MASK = 0xFFFFFFFF = -1;

// int QMCR_Config#STOP_MASK = 0xFFFFFFFF = -1;

// int QMCR_Config#SUPV_MASK = 0xFFFFFFFF = -1;

// unsigned short int QMCR_Config#QMCR_REG = 0xFFFF = 65535;

// unsigned short int QMCR_Config#QMCR_LOC = 0xE14;

// 001DFA 000468 QSM_setup

// 001E02 0007BE main

// char *main#MsgTx = no value;

Display the stack in hex.

> displaystack hex;

// 0x001DE6 00 00 00 ff 00 00 00 ff 00 00 00 ff 00 00 00 ff

// 0x001DF6 00 00 04 a6 00 00 04 68 00 00 0c ac 00 00 07 be

// 0x001E06 00 00 00 00 00 00 0b b4 00 00 00 00 00 00 00 ff

// 0x001E16 00 00 04 4a

//

See also

Shell Syntax Conventionsshellsyntaxconventions

StackInfo Shell Commandstackinfoshell

# # $ K EnableAlarmLimit Shell Command

Enable the alarm limitalarmlimitterm.

The Stack Window'sstack Options/Enable Alarm Limit is equivalent.

Syntax

enableAlarmLimit

Parameters

none

Examples

Set stack alarm to 75% and enable alarm limit.

> setstackalarm 75%; enablealarmlimit

Show current setting.

> stackinfo

// stack base = 0x001E1A

// size = 1000

// current stack pointer = 0x001DE6

// alarm limit = 75%, ENABLED

// high water mark = disabled

// stack type = high to low

See also

DisableAlarmLimit Shell Commanddisablealarmlimitshell

DisableHighWaterMark Shell Commanddisablehighwatermarkshell

EnableHighWaterMark Shell Commandenablehighwatermarkshell

SetStackAlarm Shell Commandsetstackalarmshell

Shell Syntax Conventionsshellsyntaxconventions

StackInfo Shell Commandstackinfoshell

# # $ K EnableHighWaterMark Shell Command

Enable high-water markhighwatermarkterm.

The Stack Window'sstack Options/Enable High-Water Mark is equivalent.

Syntax

enableHighWaterMark

Parameters

none

Examples

Enable high-water mark.

> enablehighwatermark

Show current setting.

> stackinfo

// stack base = 0x001E1A

// size = 1000

// current stack pointer = 0x001DE6

// alarm limit = 79%, ENABLED

// high water mark = 0x001DE6

// stack type = high to low

See also

DisableAlarmLimit Shell Commanddisablealarmlimitshell

DisableHighWaterMark Shell Commanddisablehighwatermarkshell

EnableAlarmLimit Shell Commandenablealarmlimitshell

FillStackPattern Shell Commandfillstackpatternshell to reset the high-water mark.

Shell Syntax Conventionsshellsyntaxconventions

StackInfo Shell Commandstackinfoshell

# # $ K FillStackPattern Shell Command

Fills the stack with a special pattern in order to detect the high-water markhighwatermarkterm.  The fill is from the current stack pointer to the end of the stack.  Use this command to reset the high-water mark.

Syntax

fillStackPattern

Parameters

none

Examples

Reset the high-water mark.

> fillstackpattern

See also

DisableHighWaterMark Shell Commanddisablehighwatermarkshell

EnableHighWaterMark Shell Commandenablehighwatermarkshell

Shell Syntax Conventionsshellsyntaxconventions

StackInfo Shell Commandstackinfoshell

# # $ K SetStackAlarm Shell Command

Set the percentage marker for the alarm limitalarmlimitterm.

The Stack Window'sstack Options/Alarm Limit... is equivalent.

Syntax

setStackAlarm <% of stack>

Parameters

% of stack	0 to 100% of the stack.

Examples

Set the stack alarm limit at 55%.

> setstackalarm 55; enablealarmlimit

Show current setting.

> stackinfo

// stack base = 0x001E1A

// size = 1000

// current stack pointer = 0x001DE6

// alarm limit = 54%, ENABLED

// high water mark = 0x001DE6

// stack type = high to low

See also

DisableAlarmLimit Shell Commanddisablealarmlimitshell

EnableAlarmLimit Shell Commandenablealarmlimitshell

Shell Syntax Conventionsshellsyntaxconventions

StackInfo Shell Commandstackinfoshell

# # $ K SetStackBase Shell Command

Set the base of the stack.

The Stack Window'sstack Options/Stack Base... is equivalent.

Syntax

setStackBase <address>

Parameters

address	Upper memory boundary of the stack.

Examples

Set the stack base and size.

> setstackbase 0x1000; setstacksize 0x500

Show current settings.

> stackinfo

// stack base = 0x001000

// size = 500

// current stack pointer = 0x001DE6

// alarm limit = 54%, ENABLED

// high water mark = 0x000B00

// stack type = high to low

See also

SetStackBase Shell Commandsetstackbaseshell

Setting Stack Areasettingstackarea

Shell Syntax Conventionsshellsyntaxconventions

StackInfo Shell Commandstackinfoshell

# # $ K SetStackSize Shell Command

Set the number of bytes in the stack.

The Stack Window'sstack Options/Stack Size... is equivalent.

Syntax

setStackSize <stack size>

Parameters

stack size	Number of bytes in stack.

Examples

Set the stack base and size.

> setstackbase 0x1000; setstacksize 0x500

Show current settings.

> stackinfo

// stack base = 0x001000

// size = 500

// current stack pointer = 0x001DE6

// alarm limit = 54%, ENABLED

// high water mark = 0x000B00

// stack type = high to low

See also

SetStackBase Shell Commandsetstackbaseshell

Setting Stack Areasettingstackarea

Shell Syntax Conventionsshellsyntaxconventions

StackInfo Shell Commandstackinfoshell

# # $ K StackInfo Shell Command

Display the following stack information:

-	Stack base.

-	Stack size.

-	Current stack pointer.

-	Alarm limitalarmlimitterm.

-	High-Water markhighwatermarkterm.

-	Stack type.  All supported compiler'stoolchains stacks are high memory to low memory.

The Stack Windowstack is equivalent.

Syntax

stackInfo

Parameters

none

Examples

Display current information.

> stackinfo

// stack base = 0x001E1A

// size = 1000

// current stack pointer = 0x001DE6

// alarm limit = 75%, ENABLED

// high water mark = 0x001DE6

// stack type = high to low

See also

DisableAlarmLimit Shell Commanddisablealarmlimitshell

DisableHighWaterMark Shell Commanddisablehighwatermarkshell

DisplayStack Shell Commanddisplaystackshell

EnableAlarmLimit Shell Commandenablealarmlimitshell

EnableHighWaterMark Shell Commandenablehighwatermarkshell

FillStackPattern Shell Commandfillstackpatternshell

SetStackAlarm Shell Commandsetstackalarmshell

SetStackBase Shell Commandsetstackbaseshell

SetStackSize Shell Commandsetstacksizeshell

Shell Syntax Conventionsshellsyntaxconventions

# # $ K Set Stack Area Shell Command

There are two Shell commands, one to set the base and one to set the size.  Since there is a delay between command executions, invoking the first command to change the value of the stack base or size can inadvertently define an invalid stack area.  To avoid this problem, set both the stack base and size information with one command, SetStackArea.

The Stack Window'sstack Options/Stack Area... is equivalent.

Syntax

SetStackArea [<address> <stack size> [fillArea]]

Parameters

<address>	Upper memory boundary of the stack.

<size>	Number of bytes required for the stack.

fillArea	Specify this flag to initialize the stack area.

Examples

Set the stack base and size and initialize stack area.

> setstackarea 0x1000 0x500 fillarea

Show current settings.

> stackinfo

// stack base = 0x001000

// size = 500

// current stack pointer = 0x001DE6

// alarm limit = 54%, ENABLED

// high water mark = 0x000B00

// stack type = high to low

See also

Setting Stack Areasettingstackarea

Shell Syntax Conventionsshellsyntaxconventions

StackInfo Shell Commandstackinfoshell

# # $ K Update Stack Window During Emulation

You can manually update the Stackstack window if you suspect that the stack information has changed but that change is not reflected in the window.

1.	If you want to be able to manually update memory during emulation then, from the Toolbartoolbar, choose Options/Enable Run Access before starting emulation.

2.	Choose File/Refresh Display.

# $ # The Stack Alarm and the High-Water Mark

The stack alarm limit and high-water mark are saved at the end of a MICEpack/SLD session.  The saved settings are used when you reinvoke MICEpack/SLD, even if a different file is loaded.

# $ # + Stack Frame

A stack frame contains one function in program execution history, including its parameters and local variables.  If the stack frame contains an assembly language function, then succeeding frames may be inaccurate.

# $ # K Inspect Source

Inspect Source scrolls the Source window to the top of the function that is highlighted in the Stack window.

$ Stack Window

# stack

+ powerviews:0080

K stack;window

$ Setting Stack Area

# settingstackarea

+ usingthestack:0010

K stack

$ Navigating the Stack

# navigatingthestack

+ usingthestack:0020

K stack

$ Stack Alarm

# stackalarm

+ usingthestack:0030

K stack

$ High-Water Mark in Stack

# highwatermark

+ usingthestack:0040

K stack

$ Stack Overflow Message

# stackoverflowmessage

+ usingthestack:0050

K stack

$ When is Stack Valid?

# whenisstack

+ usingthestack:0060

K stack

$ Stack Shell Commands

# stackshellcommands

K stack;shell

$ Alarm Limit

# alarmlimitterm

K stack;alarm limit;stack alarm;

# stackalarmterm

$ Stack Alarm Definition

K stack;alarm;high-water mark;

$ High-Water Mark

# highwatermarkterm

K stack;high-water mark; stack alarm

# stackareadialog

$ Stack Area Dialog Box

K stack;dialog box;stack base

# alarmlimitdialog

$ Alarm Limit Dialog Box

K stack;Dialog box;alarm limit;stack alarm;

# disablealarmlimitshell

$ DisableAlarmLimit Shell Command

K stack;alarm limit;disable alarm limit;shell, stack;

# disablehighwatermarkshell

$ DisableHighWaterMark Shell Command

K stack;high-water mark;disable high water mark;shell, stack;

# displaystackshell

$ DisplayStack Shell Command

K stack;display stack;shell, stack;

# enablealarmlimitshell

$ EnableAlarmLimit Shell Command

K stack;stack alarm;enable alarm limit;shell, stack;

# enablehighwatermarkshell

$ EnableHighWaterMark Shell Command

K stack;high-water mark;shell, stack;

# fillstackpatternshell

$ FillStackPattern Shell Command

K stack;fill stack pattern;shell, stack;

# setstackalarmshell

$ SetStackAlarm Shell Command

K stack;alarm limit; set stack alarm;shell, stack;

# setstackbaseshell

$ SetStackBase Shell Command

K stack;stack base;set stack base;shell, stack;

# setstacksizeshell

$ SetStackSize Shell Command

K stack;stack size;set stack size;shell, stack;

# stackinfoshell

$ StackInfo Shell Command

K stack;alarm limit;high-water mark;stack type;shell, stack;

# setstackareashell

$ Set Stack Area Shell Command

K stack;setStackArea;shell, stack;

# updatestack

$ Update Stack Window During Emulation

K stack;refresh stack;update stack;

$ The Stack Alarm and High-Water Mark

# defaultstackalarm

$ Stack Frame

# stackframeterm

+ stack;stack frame;

$ Inspect Source

# inspectsource

K stack;source;