# $ # + K Status Window

The Status Window shows emulationemulationterm, tracetrace, and break causeemulationtraceand information.

Screen Description

The Status Window displays:

-	The emulation and trace status in the title bar.

-	The reason for the last break is displayed in the window area.

The minimized Status Window shows the emulation and trace status, and updates whenever status changes.

Menu Selections

By default, the Status window appears on top of all other MICEpack/SLD windows.  Use the Control menu (open by clicking the upper-left corner bar) to toggle off Always on Top.

Using the Status Window

Emulation, Trace, and Break Cause Valuesemulationtraceand

See also

Status Shell Commandsstatusshellcommands

# $ # K Status Shell Commands

Status related commands are:

bkptbkptshell [enable | disable] [temporary | permanent][<symbol> | <address> [sp | up]] [@<id> ]

bkptClearbkptclearshell @<id> | all | <address> [sp | up]

emuStatusemustatusshell

gogoshell

goIntogointoshell [call | return] [line | statement]

goUntilgountilshell [call | return] [line | statement]

halthaltshell

isEmuHaltedisemuhaltedshell (returns 0 if running, non-zero if halted)

stepstepshell [into | over] [<count> ]

stepSrcstepsrcshell [into | over] [line | statement] [<count> ]

See also

Shell Command List (alphabetic)shellcommandlist

Shell Command List (topical)shellcommandlisttopical

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Emulation, Trace, and Break Cause Values

The emulationemulationterm status is:

-	Halted.  The processor is not running code.

-	Hung.

-	Running.  The processor is running code.

-	Stepping.  Stepping one or more instructions.

The trace status is:

-	Tracing: <active buffer>.  Trace is active and the buffer collecting current data is shown.

Tracing is shown when emulation is halted to indicate it is enabled for the next emulation session.

-	Not Tracing.  Trace viewing while emulating is allowed.

The break cause is:

-	None.  The initial break cause before emulating.

-	Reset.  The processor is in its reset stateresettingtheprocessor.

-	User Halted.  A request for halt from the ToolBar Window'stoolbar halt button, the Source Window'ssource halt button, the Source Window's Run/Halt menu option, or the halt Shell commandhaltshell stopped emulation

-	Trigger Breakpoint.  The Trigger Windowtrigger brk action occurred.

-	Step.  An assembly line step executed.

-	Execution Breakpoint.  A software or hardware breakpointbreakpointterm occurred.

-	Single Step Failure.  An assembly line step executed, but failed.

# # $ K Load Progress Dialog Box

The Load Progress dialog box appears when you load a file from the Tool Bartoolbar or from the File menu of the Source Windowsource, but not when you load a file using the loadloadshell command from the Shell Windowshell.

The following information is continuously updated to show what has been loaded:

-	The loadfileloadfileterm path.

-	The name of the load module (displayed while symbols are being loaded).

-	A progress gauge, showing the approximate percentage of the load completed.

-	The number of bytes of code.

-	The number of modules.

-	The number of symbols.

-	The number of functions.

-	The number of user-defined types.  Note: the compiler normally generates a new type for each type in a source module, even if part of a shared include file.

-	The number of line references.  Note: the load optionsloadoptionsdialog default to on-demand symbol loading; i.e., no line references are loaded until you browse a module and the number of line references displayed here is 0.

You can abort a load at any time by pressing the Cancel button.  The Help button is dimmed during the load because communication between your computer and MICEpack requires 100% of the CPU.  When the load completes, the following information is displayed:

-	The starting PCpcmarkerterm address.  An unknown in this field indicates that the loadfile does not contain the starting PC and you must set it using the CPU Windowcpu, the register Shell commandregistershell, or the Source Window's Run/Go From Cursorsource command.

-	The stack base.  An unknown in this field indicates that the loadfile does not containing the stack base address and you can set it by using the Stack Areasettingstackarea command of the Stack Windowstack, or by using the setstackareasetstackareashell Shell command.  You can also use the CPU Windowcpu or the register Shell commandregistershell to set the stack pointer.

-	The stack size.  An unknown in this field indicates that the loadfile does not contain the stack size and you can set with the Stack Area Shell command.

See also:

Load Informationloadinformationdialog

# # $ K Cancel Load

This dialog box appears when you choose Cancel from the Load Progressloadprogressdialog dialog box.  Choose Yes to immediately stop the load.  Choose No to continue loading.

See also:

Load Informationloadinformationdialog

Load Progressloadprogressdialog

# # $ K Load Information

Choose Load Information from the File menu of the Source Windowsource to display:

-	The loadfileloadfileterm path.

-	The name of the load module (displayed while symbols are being loaded).

-	The number of bytes of code.

-	The number of modules.

-	The number of symbols.

-	The number of functions.

-	The number of user-defined types.  Note: the compiler normally generates a new type for each type in a source module, even if part of a shared include file.

-	The number of line references.  Note: the load optionsloadoptionsdialog default to on-demand symbol loading; i.e., no line references are loaded until you browse a module and the number of line references displayed here is 0.

-	The starting PCpcmarkerterm address.  An unknown in this field indicates that the loadfile does not contain the starting PC and you must set it using the CPU Windowcpu, the register Shell commandregistershell, or the Source Window's Run/Go From Cursorsource command.

-	The stack base.  An unknown in this field indicates that the loadfile does not containing the stack base address and you can set it by using the Stack Areasettingstackarea command of the Stack Windowstack, or by using the setstackareasetstackareashell Shell command.  You can also use the CPU Windowcpu or the register Shell commandregistershell to set the stack pointer.

-	The stack size.  An unknown in this field indicates that the loadfile does not contain the stack size and you can set with the Stack Area Shell command.

See also:

Load Progressloadprogressdialog

$ Status Window

# status

+ powerviews:0090

K status;window; breakpoint

$ Status Shell Commands

# statusshellcommands

K status; shell;

$ Emulation, Trace, and Break Cause Values

# emulationtraceand

K emulation;trace;breakpoint

# loadprogressdialog

$ Load Progress Dialog Box

K load;status;Dialog Box;

# cancelloaddialog

$ Cancel Load

K load;cancel load;Dialog Box

# loadinformationdialog

$ Load Information

K load;stack;pc;status;