# # $ K + Technical Notes

Assembly Language Supportassemblylanguagesupport

C and C++ Supportcandc

Changing Compilerschangingcompilers

Comm Port Switchingcommportswitching

Compiler Supporttoolchains

Supported Compiler OptimizationsSupported_Code_Optimization_Features>main

Function Keysfunctionkeys

Getting Startedgettingstarted

Improving MICEpack/SLD Performanceimprovingpowerviewsperformance

Initialization File (micepack.ini)initializationfile

Initialization Script (include.me)initializationscript

Powering Down the Targetpoweringdownthe

Sample Filessamplefiles

Script Filesscriptfiles

Solutions to Common Problemssolutionstocommon

Stopping Emulation When Invalid Stack Area Accessedstoppingemulationwhen

Trace and Trigger Fundamentalstraceandtriggerfundamentals

Triggering on Access to a C Variabletriggeringonaccess

# $ # + K COMM Port Switching

The "type=" line of [COMM] and "baseport=" line in [MicrotekParallel] sections in MICEPACK.INI, defines the communication port MICEpack/SLD uses the next time you invoke MICEpack/SLD.  For example, the following defines COM2 as the communication:

//MICEPACK.INI - Configuration Information for MICEpack/SLD
	[Comm]
	type=serial

[Serial]
	comport=com2

Check your MP Setup dialog box (by double clicking the “MP/Setup” icon from Program Manager) for correct setup

# $ # + K Improving MICEpack/SLD Performance

This can be divided into two areas: Windows tuning and MICEpack/SLD-specific tuning.

Windows tuning

Look for these items to increase performance:

1.	Have enough memory.  MICEpack/SLD runs faster on an 8 MB machine.

2.	Use SmartDrive or some other disk caching software.

3.	Use a  permanent swap fileswapfileterm.

4.	Use 32-bit access (see Windows Control Panel, 386 Enhanced).

For more information, see the Windows Resource Toolkit, published by Microsoft.

MICEpack/SLD tuning

Each open MICEpack/SLD window takes time to update.

Mixed source/assembly modesourcemixedand slows window updates.

# $ # + K Function Keys

F1	MICEpack/SLD Help, at Index

F2	Halt emulation

F3	Reserved

F4	Reserved

F5	Activate the toolbar

F6	Cycle to next window

F7	Step into a function

F8	Step over a function

F9	Emulate (Go)

F10	Activate the menu bar

# $ # + K Initialization File

When you install MICEpack/SLD, SETUP creates the MICEPACK.INI file in your Windows directory.  The file contains information that MICEpack/SLD uses when it starts and each time a window is opened.

The following sections appear in MICEPACK.INI:

[comm]		Communication type.

[Serial]		COM Port .

[InitScript]		Initial script to run on startup.

[Trace Info]		Trace window options.

[SourceInfo]		Source window Go, Step, and View options.

[StackInfo]		Stack size, base address, and other Stack window options.

[ToolChain]		Compiler information

MICEpack/SLD edits entries in MICEPACK.INI when completing an operation, with the following exception.   You must edit this section manually:

The  [InitScript]initializationscript section defines the initialization script.

The MICEpack/SLD User’s Guide describes the other sections in detail.

# $ # + K Powering Down the Target

Turn off the emulator before turning off the target system.

Power must be applied and removed in the correct sequence.  Failure to follow this sequence may result in severe damage to your target system or to the emulator.  The order of power sequencing is as follows:

1.	Apply power to the MICEpack emulator.

2.	Apply power to the target system.

3.	Remove power from the target system.

4.	Remove power from the MICEpack emulator

# $ # K demo.abs sample Program

The DEMO.ABS program in the \MPMCF\SAMPLE directory is generic to the ColdFire processor core.

For any of the ColdFire processors, execute "demo.inc" from the command pane of the Shell window.  It sets up the compilerUsed flag.

> include "\mpmcf\sample\demo.inc";

After running the demo program, be sure to set the compilerUsed flag back to whichever compiler you are using.  This can be done with the Options Compiler Used... menu command in the Source window or the compilerUsedcompilerusedshell command from the Shell.

MICEpack emulator

To run this program, simply set up the map for 64K of code beginning at address 0x10000 in your target system.  The program begins at 0x1002 and ends about 0x17342.  The stack starts at about 0x17338 and grows down.

# $ # + K Sample Files

\MICEPACK\SAMPLE directory

DEMO.ABS

The DEMO.ABS programdemocpu32 in this directory is written for any processor that has the ColdFire processor core.  See the SAMPLES.TXT file for more information.

See also

Getting Startedgettingstarted

# $ # + K Stopping Emulation When Invalid Stack Area Accessed

1.	Check your stack base and stack size using the Stack Window's Options/Stack Area menu item or the StackInfo Shell Command.

2.	The last location in the stack equals the stack base minus the stack size.

3.	Define an event to trigger on writes to the address just below the last location in the stack.  Enter the address.  Select 0 for the r/w- signal.

# # $ K + Supported Compiler Optimizations

The following compiler optimizations are supported for the MRImri>main and Intermetricsintermetrics>main toolchains:

## General Optimizations:

1.	Algebraic Simplification:

Replace expressions with equivalent but simplified expressions

Example: a*1 becomes a

2.	Constant Folding:

Arithmetic and logical operations on constants are evaluated at compile time.

Example: i = 2 + 3 - x becomes i = 5 - x

3.	Strength Reduction:

Replace operations with equivalent but less expensive ones.

Example: b = a * 128; becomes b = a << 7

4.	Redundant Code Elimination:

Delete code sequences that produce no significant side effects.

Example: a &= a; (No code is generated)

5.	Unreachable Code:

Perform jump optimization for unreachable code

Example: if (a != a) b = 2; (No code is generated)

## Global Optimizations:

1.	Register coloring:

Use one register for multiple variables where each variable has its own lifetime information.

Note:
In the Variable and Stack windows, the value of a dead variable is shown in black and is not editable.

2.	Factorization:

Store frequently used static values and addresses in registers.

Example:	test(1);

test(2);

3.	Dead (unreachable) Code Elimination.

4.	Unused Definitions Elimination:

Delete assignment definitions in which the value is either never used, or used after being reassigned.

Example:	i = k + 1;  -> deleted.

...

i = MAX_VALUE;

5.	Global Constant Propagation:

The variable contains a constant value that may be carried forward to the subsequent uses of the value of that variable until a new value is assigned to that variable.

Example:	x = 2;

if (x) { ... };

func(x, ...);

6.	Global Copy Propagation:

Assign a register variable to a global variable that may be carried forward to subsequent uses of the global variable.

7.	Branch Merging:

Combine common instructions preceding two jump instructions which jump to the same location.

Example:	func1(x);

x++;

if (x)

goto A1;

func2(x);

x++;

if (x)

goto A1;

## Local Optimizations:

All optimizations that are performed in global scope (see Global optimizations).

## Loop Optimizations:

All loop optimization are supported.

## Optimizations that are NOT supported:

1.	Function In-lining

2.	Grouping Stack Adjust Instructions (Combining Stack Pops).

3.	Global and local flow optimization.

# technicalnotes

$ Technical Notes

K answers;help;PowerViews;list

+ POWERVIEWS:0150

$ Comm Port Switching

# commportswitching

+ technical:0060

K communications;initialization file;pwrviews.ini

$ Improving PowerViews Performance

# improvingpowerviewsperformance

+ technical:0110

K PowerViews;performance;window

$ Function Keys

# functionkeys

+ technical:0090

K keys;function keys

$ Initialization File

# initializationfile

+ technical:0120

K initialization file;pwrviews.ini;files

$ Powering Down the Target

# poweringdownthe

+ technical:0155

K target

$ CPU32 Sample Programs

# democpu32

K samples; CPU32

$ Sample Files

# samplefiles

+ technical:0170

K help;files

$ Stopping Emulation When Invalid Stack Area Accessed

# stoppingemulationwhen

+ technical:0235

K emulation;stack;trigger

# Supported_Code_Optimization_Features

$ Supported Compiler Optimizations

K compiler optimizations;

+ TECHNOTE