# # $ K ToolBar Window

The ToolBar contains buttons to access the most commonly used windows and commands.

Button Bar

Click on one of the buttons to access the feature:

Load	Load code and symbols.

Trigger	Set up trigger.trigger.

Source	Show source codesource.

Stack	Display program stackstack.

CPU	Show CPU registers.cpu.

Mem	Display memory regionmemory.

Periph	Show Peripheral Registersperiph.

Go	Start emulationemulationterm.

Halt	Stop emulation.

Show	Show trace collectedtrace.

Shell	Command lineshell.

Menu Selections

File

Exit MICEpack/SLD	Exit MICEpack/SLD applicationexitingpowerviews.

Configure

Trace Off	Disable trace feature.  If the FIFO that holds the data to be output to the DDATA port is full, ColdFire CPU performace may degrade.  This could be avoided by disabling trace feature.

Real-time Trace	Enable or disable real-time trace. When real-time trace is disable, the trace display could interpret  entire trace buffer data.

Run Access	Enable or disable run accessrunaccessterm. The run access option is activated when the real-time trace is enable.

Symbolic Disassembly	Enable or disable symbols in disassembly, which is displayed in the Sourcesource and Memorymemory windows.

Reset	Resetresettool and reinitialize the target processor.

Layout

Save Settings Now	Save the current coordinates for the MICEpack/SLD windows and icons, to be restored the next time you invoke the tool.

Save Settings on Exit	Save the window and icon coordinates upon exiting MICEpack/SLD.

Windows	Move between windowsmovingbetweenwindows.

Help	On-line helponlinehelp

Using the ToolBar Window

ToolBar Shell Commandstoolbarshellcommands

Reading and Writing Memory while Emulating using Run Accessrunaccessterm

Minimizing MICEpack/SLD from the ToolBarminimizingpowerviewsfrom

See also

Getting Startedgettingstarted

# $ # K Minimizing MICEpack/SLD from the ToolBar

Minimizing the ToolBar window minimizes all of the MICEpack/SLD windows and icons into a single icon.  Maximizing the ToolBar icon will restore the ToolBar and access to the windows and other icons.

See also

Moving Between Windowsmovingbetweenwindows

# # $ K Toolbar Shell Commands

Shell commands related to the ToolBar:

gogoshell

halthaltshell

loadloadshell "<filename>" [symbols | nosymbols] [code | nocode] [progress | noprogress] demand | nodemand] [module <module path name>] [reload] [warn | nowarn] mangle | nomangle] [asm | noasm] [supervisor | user

runAccessrunaccessshell [on | off]

versionversionshell

Memory Shell Commandsmemoryshellcommands

Shell Commandsshellcommands

Source Shell Commandssourceshellcommands

Stack Shell Commandsstackshellcommands

Status Shell Commandsstatusshellcommands

Trace Shell Commandstraceshellcommands

Variable Shell Commandsvariableshellcommands

See also

Shell Command List (Alphabetic)shellcommandlist

Shell Command List (Topical)shellcommandlisttopical

# $ # K Moving Between Windows

To move the cursor to a MICEpack/SLD window:

-	Click on the window

-or-

-	Choose it from the Windows menu

-or-

-	Press F6.

To move to a different application's Window:

-	Click on it or press Alt-Tab.

# $ # K On-line Help

To access on-line help:

-	Press F1.  This will give context sensitive help for the current window or dialog box.

-	Choose the Help menu item and pick a topic of interest.

# $ # K Symbol Entry Syntax

Symbols are specified as:

[#<module name>] [#<function name>] [#<variable name>] [#<column number>] [#<line number>]

A fully-qualified symbol contains the module name, function name, and variable name.  Column number specifies a statement breakpoint.  Line number specifies a line number.

If you do not specify a fully-qualified name MICEpack/SLD attempts to:

-	match the symbol in the current function

-	match the symbol in the current module

-	match the symbol to a global

Many symbol specifications are valid, including:

#<line number>

#<module name>

#<function name>

#<variable name>

#<column number>

#<module name>#<function name>

#<module name>#<variable name>

#<module name>#<column number>

#<function name>#<variable name>

#<function name>#<column number>

#<module name>#<function name>#<variable name>

#<module name>#<variable name>#<column number>

#<function name>#<variable name>#<column number>

#<module name>#<function name>#<variable name>#<column number>

# $ # K Version Shell Command

Display hardware and software version information.

Syntax

version

Parameters

none

Examples

Display version information.

>version

// System configuration is MICEpackÒ for MC68306

// Software version is 2.3M

// Firmware version is 0.0(0)

// Motherboard version is 1.0(1)

// Probe version is 0.0(0)

// Trace R version is 1.0(1)

// Trace S version is 1.0(1)

// Trace T version is 1.0(1)

// Dos version is 6.20

// Windows version is 3.10

See also

Reporting Problems to Microtekreportingproblemsto

Shell Syntax Conventionsshellsyntaxconventions

# $ # K Swapfile

A portion of the hard disk is set aside to swap memory images when more memory is needed than actually exists.  To find the amount of swapfile space available:

-	Choose Program Manager's Main program group.

-	Choose the Control Panel.

-	Choose 386 Enhanced.

-	Choose Virtual Memory... button.

There are three settings:

-	Drive points to a hard disk partition (e.g. C:).

-	Size is the number of bytes in the swapfile (e.g. 6,000 KB is 6 Megabytes).

-	Type is permanent or temporary (e.g., type is Permanent if using BIOS).

MICEpack/SLD requires a minimum of 4 MB of permanent swapfile.

# $ # + K Exiting MICEpack/SLD

The ToolBar Window'stoolbar File/Exit closes all windows and terminates the MICEpack/SLD application.

Each individual window has an exit menu option.  This exit closes the window, but does not exit MICEpack/SLD.

# # $ K Peripheral Window

The Peripheral window enables the viewing and modifying of internal peripheral registers.

Screen Description

View the internal registers in compressed or expanded format.  The columns are (from left to right):

-	(-) or (+) indicates expanded or compressed, respectively.  Click to change format.

-	Register address.  Double-click anywhere on the line to edit.

-	Register value.

-	Register mnemonic.

-	Register Description.

Menu Selections

File

Exit	Close the Peripheral Window.

Edit

Registereditperiph...	Edit a peripheral register.

Go To Peripheralperiphgotoperiphdialog...	Scroll to a specified peripheral.

Go To Addressperiphgotoaddressdialog...	Scroll to a specified peripheral address.

View

Expand All	Expandexformperiph the register display.

Compress All	Compresscomformperiph the register display.

Refresh Display	Refreshupdateperiph the register display.

Windows	Move between windowsmovingbetweenwindows.

Help	On-line helponlinehelp.

Using the Peripheral Window

Editing a Peripheral Registereditperiph

Scrolling through the Peripheral Windowscrollperiph

# # $ K Edit Peripheral Register Dialog Box

Depending on the peripheral you are editing, MICEpack/SLD will display one of the following dialog boxes:

-	Edit Peripheral Register Dialog Boxeditperiphregdialog (for most peripherals).

-	Edit Peripheral Data Register Dialog Boxeditperiphregbufferdialog

# # $ K Expanded Format Peripheral Window

Shows the individual bit fields of a peripheral register, as well as the information shown in compressed formatcomformperiph.

# # $ K Compressed Format Peripheral Window

In this format, the columns in the Peripheral windowperiph are (from left to right):

-	Register address.

-	Register mnemonic

-	Register value (in red).

-	Register description.

# # $ K Update Peripheral Registers During Emulation

You can manually update the Peripheralperiph window if you suspect that the register values have changed but that change is not reflected in the window.

1.	If you want to be able to manually update memory during emulation then, from the Toolbartoolbar, choose Options/Enable Run Access before starting emulation.

2.	Choose View/Refresh Display.

# # $ K Scrolling the Peripheral Window

## If the Emulator is Halted:

-	Scrolling or paging updates the registers.

# # $ K Receive and Transmit Buffers

Receive and transmit buffers are displayed in the Peripheralperiph window in their entirety.  Buffers are displayed in byte format.

-	Double click on a buffer line to display an edit dialog box that allows you to edit specific bytes in hex.

# # $ K Reset

Choose Configure/Reset from the Toolbartoolbar to do the following:

-	The processor reset pin is asserted.

-	The program counter and stack pointer values are read from memory.

-	All windows are updated with the following results:

-	The Stack display is invalid because the stack is reset.

-	The Source window points to the start of code.

See also

Resetting the Processorresettingtheprocessor

# # $ K Peripheral Go To Address Dialog Box

The window displays:

-	The peripheral at a valid hexadecimal address (prefixed by 0x).

To reposition the Peripheralperiph window display:

-	Enter an address.

See also

Go To Peripheral Dialog Boxperiphgotoperiphdialog

# # $ K Go To Peripheral Dialog Box

1.	Click the down-arrow on the drop-down box to display the list of peripheral groups.

2.	Enter a peripheral group for repositioning the Peripheralperiph window display.

See also

Peripheral Go To Address Dialog Boxperiphgotoaddressdialog

# # $ K Edit Peripheral Data Register Dialog Box

The dialog box displays:

-	A listbox that shows the data in all memory locations that comprise the buffer.

-	An edit field for the selected memory location.

To edit the buffers:

1.	Highlight the memory location you want to change

2.	Type the new value.

3.	When finished with all changes, write all locations of the buffer by choosing OK.

4.	Choose Cancel to exit the dialog box without making changes.

See also

Edit Peripheral Register Dialog Boxeditperiphregdialog

Receive and Transmit Buffersbufferperiph

# # $ K Edit Peripheral Register Dialog Box

The dialog box displays:

The register name in the title.

A list box with register fields.

The field to the right of the Field Value spin box.

Use the dialog box as follows:

1.	Edit the register value.

-or-

Use the list box to select a register field and edit it with the Field Value spin box.

2.	Choose  Write to write to the register.  This does not close the dialog box so you can make further changes and write again.

3.	When finished, choose Close to close the dialog box.  No writes take place when Close is chosen.

See also

Edit Peripheral Data Register Dialog Boxeditperiphregbufferdialog

# toolbar

$ ToolBar Window

K toolbar;window;emulation

$ Minimizing PowerViews from the ToolBar

# minimizingpowerviewsfrom

K icons;minimizing;window

# toolbarshellcommands

$ Toolbar Shell Commands

K toolbar;shell

$ Moving Between Windows

# movingbetweenwindows

K window

$ On-line Help

# onlinehelp

K help;window

$ Symbol Entry Syntax

# symbolentrysyntax

K symbols

$ Version Shell Command

# versionshell

K version;PowerViews;revision

$ Swapfile

# swapfileterm

K swapfile

$ Exiting PowerViews

# exitingpowerviews

+ gettingstarted:0130

K exit;PowerViews

# periph

$ Peripheral Window

K peripherals;registers

# editperiph

$ Edit Peripheral Register Dialog Box

K peripherals;registers;edit peripherals;

# exformperiph

$ Expanded Format Peripheral Window

K peripherals;registers;expanded format

# comformperiph

$ Compressed Format Peripheral Window

K peripherals;registers;compressed format

# updateperiph

$ Update Peripheral Registers During Emulation

K peripherals;registers;run access;update peripherals;

# scrollperiph

$ Scrolling the Peripheral Window

K peripherals;

# bufferperiph

$ Receive and Transmit Buffers

K peripherals;receive buffers;transmit buffers;edit buffers;

# resettool

$ Reset

K reset;processor reset

# periphgotoaddressdialog

$ Peripheral Go To Address Dialog Box

K peripherals;go to address;dialog box;

# periphgotoperiphdialog

$ Go To Peripheral Dialog Box

K peripherals;go to peripheral;dialog box;

# editperiphregbufferdialog

$ Edit Peripheral Data Register Dialog Box

K peripherals;edit peripheral;dialog box;

# editperiphregdialog

$ Edit Peripheral Register Dialog Box

K peripherals;edit peripheral;dialog box;