# $ # + K Trace Window

The Trace Window displays a real-time collection of execution instructions.

Screen Description

The frame number, address and disassembled instruction are displayed.

Menu Selections

File

Save As...	Save some or all frames to a file in a readable format.

Exit	Close the Trace window.

Edit

Search...	Search an address in trace buffer from current frame.

Search Next	Search the next occurrence of the address.

View

Linked Cursor	Activate linked cursorlinkedcursor with Source Windowsource.

Goto

Start Frame	View the beginning of the current buffer.

End Frame	View the end of the current buffer.

Frame...	Moving viewing area to a specific frame.

Windows	Move between windowsmovingbetweenwindows

Help	On-line helponlinehelp

Using the Trace Window

Trace and Trigger Fundamentalstraceandtriggerfundamentals

Saving Trace to a Filesavingtraceto

Linked Cursorlinkedcursor

See also

Trace Shell Commandstraceshellcommands

# $ # + K Saving Trace to a File

The Trace Window'strace File/Save As... option creates a text file with disassembled instruction trace data.

This file can be printed.  A file comparison program can pinpoint differences between execution snapshots.

See also

Trace Save As Dialog Boxtracesaveasdialog

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # + K Linked Cursor

The Trace Window'strace View/Linked Cursor links the Trace and Source Windowssource.  This feature enables you to correlate trace cycles back to the original source statement.

The linked cursor appears as a yellow highlight in both windows.  As the Trace Window is scrolled, the Source Window uses the address information to highlight the corresponding source line.

Scrolling the Trace Window repositions the Source Window but scrolling the Source Window has no effect on the Trace Window.

See also

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # K Trace Shell Commands

There are no Shell commands for trace.  Use the Trace Windowtrace to display trace and set trace options.

# $ # K Trace Save As Dialog Box

Enter the directory and the name of the file to save the trace information.  The file format is readable and printable.  A second dialog box appears.  The fields are:

-	Start is the frame within the buffer to save.

-	End is the last frame to save.

See also

Saving Trace to a Filesavingtraceto

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # K Frame Number Dialog Box

Specify a frame number (in the given range of valid frames) to position the top of the Trace window to that frame.  A frame represents one processor instruction.

Note

-	A negative number represents frames prior to the trigger

-	Frame 0 is the trigger point

See also

Trace and Trigger Fundamentalstraceandtriggerfundamentals

$ Trace Window

# trace

+ powerviews:0110

K trace;window

$ Saving Trace to a File

# savingtraceto

+ usingthetrace:0050

K trace;files

$ Linked Cursor

# linkedcursor

+ usingthetrace:0070

K trace;source

$ Trace Shell Commands

# traceshellcommands

K trace;shell

$ Trace Save As Dialog Box

# tracesaveasdialog

K trace;dialog box;trace save;save

$ Frame Number Dialog Box

# framedialog

K trace;dialog box;frame;goto frame