# # $ K XLT Shell Command

Translates any address to its equivalent forms.

If <address> is virtual, xlt displays both the linear and physical equivalents.

If <address> is linear or physical, only the physical equivalent address is displayed.

Syntax:

xlt<address>

Parameters:

<address>:  specifies a numeric or symbolic address.

Examples (case is not significant):

xlt 1000p

xlt 2000L

xlt 1:2:400

xlt 2:400

xlt 400

# # $ K Addresses

Addresses can have the following formats:

<offset>L	Linear Address.

<offset>P	Physical Address.

(#module)#symbol	Symbolic segment:offset interpreted as a Virtual address.

<ldt>:<segment>:<offset>	Virtual address using specified LDT.

<segment>:<offset>	Virtual address using current LDT.

<offset>	Virtual address using current LDT and DS.

# # $ K Pmode

The 386 processors operate in different pmodes.  These are real, virtual-86 (V86), protected, and System Management Mode (SMM).

Protected mode is further divided into 16-bit protected mode and 32-bit protected mode.

The 386DX and 386SX processors do not have System Management Mode.

Both the 386CX and 386EX have SMM.

The 386 processors have a segmented architecture; that is, addresses consist of a segment and an offset.  The segment determines the base address of an addressable region, and the offset is added to that base to arrive at the final linear address.  In some modes, the linear address may be further processed by the paging unit to construct the physical address which is seen on the processor pins.

The segment registers consist of a 16-bit user-visible register (CS, DS, ES, FS, GS, or SS) and 3 hidden components--the segment base, limit, and access rights.  The processor mode (pmode) affects how the processor loads the hidden portion of the segment registers.

When the 16-bit visible segment register is loaded by the user program, the processor automatically loads the hidden portion based on rules determined by the pmode.  In real and virtual-86 mode, the base is the segment multiplied by 16, the limit is always 64k bytes, and the access rights allow execution, read, and write.  In protected mode, the base, limit, and access rights are extracted from the segment descriptor indicated by the segment register value.  The descriptor is an 8-byte data structure in one of two arrays called the global descriptor table (GDT) and local descriptor table (LDT).  Bit 2 of the segment register selects which table is used.  In SMM, the base and access rights are as in real mode, but the limit is always 4 gigabytes.

Pmode also affects whether the paging unit may be used or not.  In real and SMM modes, the paging unit is not used, so the physical address is always the same as the linear address.  In V86 and protected mode, paging is active if the PG bit in the CR0 register is set.

Finally, pmode affects the instruction set of the processor.  The 386 has two sets of addressing modes--16 bit and 32 bit and two default data sizes--16 bit and 32 bit.  The default address and data sizes are determined by the pmode and the D bit in the code segment descriptor.

In real, V86, and SMM modes, 16 bits is the default.  In protected mode, the D bit determines the default address size (this is, the difference between 16- and 32-bit protected modes).  An address size override prefix byte may be added to any instruction to switch to the opposite (non-default) address size; therefore, even in real mode, the 32-bit addressing modes may be used if the instruction includes the override byte.  Similarly, a data size override may be used to select the opposite data size.  Thus, even in real mode, a program can directly use 32-bit data quantities.

For example, the instruction 89 00 is:

addr size    data size      instruction

16           16          mov [bx+si],ax

32           16          mov [eax],ax

16           32          mov [bx+si],eax

32           32          mov [eax],eax

# * # $ K + Pmode Shell Command

Returns the processor modePMODE.

Syntax

pmode

Parameters

none.

Examples

pmode

# * # $ K + SaveMap Shell Command

This command saves a Map configuration.

Syntax:

SaveMap "<filename>"

Parameters:

"<filename>":  The file recieving the Map configuration

Example:

saveMap mapfile.sav

# * # $ K + RestoreMap Shell Command

This command restores a saved Map configuration.

Syntax:

RestoreMap "<filename>"

Parameters:

"<filename>":  The file containing the Map configuration

Example:

restoreMap mapfile.sav

# * # $ K + Dt Shell Command

Displays the descriptor table entries for a single selector or range of selectors.  Which selectors are displayed is determined by <selector range>, <register>, or base <address>, one of which must be specified.  Specifying <register> uses that value for the selector.  Dt enables you to not specify which dt table.  The dt table is determined by bit 2 (TI) of the selector.

Syntax:

Dt (<selector_range> | <register> | base <address> [limit <bytes>]) [all]

Parameters:

<selector_range>:  specifies either a single value (e.g., 0 or 0x08) or two values to specify a range (e.g., 4 14 or 0x10 0x400).

<register>:  specifies a register.

base <address> [limit <bytes>]:  specifies the descriptor table base address.  If a base address is specified, then <selector_range> or limit<bytes> must be specified to define the range to be displayed.

all:  displays all entries, including invalid or reserved entries.  Note that all is NOT the default.

Examples:

The first example displays all of the descriptor entries from selector 0x08 to 0x48. The second example displays just the current DS descriptor entry.

dt 0x08 0x48 all

dt ds

See Also:

gdtgdtshell, idtidtshell, ldtldtshell, tsstssshell

# * # $ K + Gdt Shell Command

Displays the global descriptor table entries for a single selector or range of selectors.  Which selectors are displayed is determined by <selector_range>, <register>, base <address>, or the current dgt_base and get_limit.  Specifying <register> uses that value for the selector.

The display stops at the end of the range or when the gdt_limit is reached.

Syntax:

Gdt [<selector_range> | <register> | base <address> [limit <bytes>]] [all]

Parameters:

No parameters:  all valid entries in the range gdt_base to gdt_base+gdt_limit are shown.

<selector_range>:  specifies either a single value (e.g., 0 or 0x08) or two values to specify a range (e.g., 4 14 or 0x10 0x400).

<register>:  specifies a register.

base <address> [limit <bytes>]:  specifies the descriptor table base address.  If a base address is specified, then <selector_range> or limit<bytes> must be specified to define the range to be displayed.

all:  displays all entries, including invalid or reserved entries.  Note that all is NOT the default.

Examples:

The first example displays global descriptor table entries.  The table base is 501010L.  This command displays global descriptor table entries from 501018L (selector 0x08) to 501028L (selector 0x18).

gdt 0x00 0x18 base 501010L

gdt all

See Also:

dtdtshell, idtidtshell, ldtldtshell, tsstssshell

# * # $ K + Idt Shell Command

Displays the interrupt descriptor table entries for a single index or range of indices.  Which entries are displayed is determined by <index_range>, <register>, base <address>, oor the current idt_base and idt_limit.  If you specify <register>, the selector for that register is used.

The display stops at the end of the range or when the idt_limit is reached.

Syntax:

Idt [<index_range> | <register> | base <address> [limit <bytes>]] [all]

Parameters:

No parameters:  then all valid entries in the range idt_base to idt_base+idt_limit are shown.

<index_range>:  specifies either a single value (e.g., 0 or 0x08) or two values to specify a range (e.g., 4 14 or 0x10 0x400).

<register>:  specifies a register.

base <address> [limit <bytes>]:  specifies the descriptor table base address.  If a base address is specified, then <selector_range> or limit<bytes> must be specified to define the range to be displayed.

all:  displays all entries, including invalid or reserved entries.  Note that all is NOT the default.

Examples:

The first example displays interrupt descriptor tables.  The table base is 501010L.  This command displays interrupt descriptor tables from 501018L (selector 0x08) to 501028L (selector 0x18).

idt 0x00 0x18 base 501010L

idt all

See Also:

dtdtshell, gdtgdtshell, idtidtshell, tsstssshell

# * # $ K + Ldt Shell Command

Displays the local descriptor table entries for a single selector or range of selectors.  Which selectors are displayed is determined by <selector_range>, <register>, base <address>, or the current ldt_base and ldt_limit.  Specifying <register> uses that value for the selector.

The display stops at the end of the range or when the ldt_limit is reached.

Syntax:

Ldt [<selector_range> | <register> | base <address> [limit <bytes>]] [all]

Parameters:

No parameters:  the ldt_base and ldt_limit are used to locate and display the LDT.

<selector_range>:  specifies either a single value (e.g., 0 or 0x08) or two values to specify a range (e.g., 4 14 or 0x10 0x400).   If <selector> is given then we use this selector from the GDT table to specify the ldt base and limit.

<register>:  specifies a register.

base <address> [limit <bytes>]:  specifies the descriptor table base address.  If a base address is specified, then <selector_range> or limit<bytes> must be specified to define the range to be displayed.

all:  displays all entries, including invalid or reserved entries.  Note that all is NOT the default.

Examples:

The first example displays local descriptor tables.  The table base is 501010L.  This command local displays descriptor tables from 501018L (selector 0x08) to 501028L (selector 0x18).

ldt 0x00 0x18 base 501010L

ldt all

See Also:

dtdtshell, gdtgdtshell, idtidtshell, tsstssshell

# * # $ K + Tss Shell Command

Displays the task state segments for any selector or base address.  If you specify <register>, the selector for that register is used.

Syntax:

Tss [<selector> | <register> | base <address> [limit <bytes>]] [all]

Parameters:

<selector>:  specifies a single value (e.g., 0 or 0x08).  This selector is referenced from the GDT.  If no selector given then the tss_base and tss_limit is used.

<register>:  specifies a register.

base <address> [limit <bytes>]:  specifies the descriptor table base address.

all:  displays all task state segments plus the I/O bit map.  Displays all entries, including invalid or reserved entries.  Note that all is NOT the default.

Examples:

The first example displays descriptor table at 0x00.

tss 0x00

tss all

See Also:

dtdtshell, gdtgdtshell, idtidtshell, ldtldtshell

# * # $ K + Config Shell Command

The 386EX enables the user to program the HLDA pin to be either the HLDA function or an I/O bit.  The PowerPack hardware needs to know when the bus has been granted to an external master so that overlay RAM cycles are disabled to prevent corruption.  If the HLDA pin is visible, it is used for this purpose.  If not visible, it is assumed that no external masters exist.

If you use the Intel Evaluation Board, which programs the HLDA pin to be an I/O bit, set config ignoreHlda on.

Syntax:

Config ignoreHlda [on | off]

Parameters:

on:  causes PowerPack to ignore the HLDA pin state.  Set config ignoreHlda on when the HLDA pin is programmed as an I/O bit.

off:  causes PowerPack to examine the HLDA pin state before it generates overlay RAM or trace/trigger strobe (initial default).

Examples:

config ignoreHlda on

# xltshell

$ XLT Shell Command

K xlt;address;shell, address;

# addressesintel

$ Addresses

K addresses;

# PMODE

$ Pmode

K Pmode;SMM;Real;V86;Prot32;Prot16;

* PPINTEL

# pmodeshell

$ Pmode Shell Command

K shell, pmode;pmode

+ INTEL

* PPINTEL

# savemapshell

$ SaveMap Shell Command

K shell, map;map;saveMap

+ INTEL

* PPINTEL

# restoremapshell

$ RestoreMap Shell Command

K shell, map;map;restoreMap

+ INTEL

* PPINTEL

# dtshell

$ Dt Shell Command

K shell, descriptor;descriptor;dt

+ INTEL

* PPINTEL

# gdtshell

$ Gdt Shell Command

K shell, descriptor;descriptor;gdt

+ INTEL

* PPINTEL

# idtshell

$ Idt Shell Command

K shell, descriptor;descriptor;idt

+ INTEL

* PPINTEL

# ldtshell

$ Ldt Shell Command

K shell, descriptor;descriptor;ldt

+ INTEL

* PPINTEL

# tssshell

$ Tss Shell Command

K shell, descriptor;descriptor;tss

+ INTEL

* PPINTEL

# configshell

$ Config Shell Command

K config;shell, HLDA;ignoreHlda;evaluation board, Intel;Intel evaluation board;

+ INTEL