# # $ K + Technical Notes

Assembly Language Supportassemblylanguagesupport

High-level Language Supportcandc

Comm Port Switchingcommportswitching

Function Keysfunctionkeys

Getting Startedgettingstarted

Improving PowerViews Performanceimprovingpowerviewsperformance

Initialization File (pwrviews.ini)initializationfile

Initialization Script (include.me)initializationscript

PmodePMODE

Powering Down the Targetpoweringdownthe

Sample Filessamplefiles

Script Filesscriptfiles

Solutions to Common Problemssolutionstocommon

Stopping Emulation When Invalid Stack Area Accessedstoppingemulationwhen

Trace and Trigger Fundamentalstraceandtriggerfundamentals

Triggering on Access to a C Variabletriggeringonaccess

# $ # + K COM Port Switching

The "comport=" line in PWRVIEWS.INI defines the serial port PowerViews uses the next time you invoke PowerViews.  For example, the following defines COM2 as the port:

[Serial]

comport=com2

Use a text editor such as Notepad to change the comport.  COM1 and COM2 are supported.  Delete the line "comport=com2" line, reinvoke PowerViews, and PowerViews will prompt for a new COM port.

# $ # + K Improving PowerViews Performance

This can be divided into two areas: Windows tuning and PowerViews-specific tuning.

Windows tuning

Look for these items to increase performance:

1.	Have enough memory.  PowerViews runs faster on an 8 MB machine.

2.	Use SmartDrive or some other disk caching software.

3.	Use a  permanent swap fileswapfileterm.

4.	Use 32-bit access (see Windows Control Panel, 386 Enhanced).

For more information, see the Windows Resource Toolkit, published by Microsoft.

PowerViews tuning

Each open PowerViews window takes time to update.

Mixed source/assembly modesourcemixedand slows window updates.

# $ # + K Function Keys

F1	PowerViews Help, at Index

F2	Halt emulation

F3	Start Trace

F4	Stop Trace

F5	Activate the toolbar

F6	Cycle to next window

F7	Step into a function

F8	Step over a function

F9	Emulate (Go)

F10	Activate the menu bar

# $ # + K Initialization File

When you install PowerViews, SETUP creates the PWRVIEWS.INI file in your Windows directory. The file contains information that PowerViews uses when it starts and each time  a window is opened.

The following sections appear in PWRVIEWS.INI:

[Serial]	COM port number.

[InitScript]	Initial script to run on startup.

[Trace Info]	Trace Control and Trigger window options.

[TrigInfo]	Trigger Control and Trigger window options.

[SourceInfo]	Source window Go, Step, and View options.

[StackInfo]	Stack size, base address, and other Stack window options.

PowerViews edits entries in PWRVIEWS.INI when completing an operation, with the following exceptions.   You must edit these sections manually:

1.	The  [InitScript]initializationscript section defines the initialization script.

2.	The   [Serial]commportswitching section defines the COM port used with PowerViews.

The manual describes the other sections in detail.

# $ # + K Powering Down the Target

Turn off the target system before turning off the emulator.

Power must be applied and removed in the correct sequence.   Failure to follow this sequence may result in severe damage to your target system or to the emulator.  The order of power sequencing is as follows:

1.	Apply power to the emulator.

2.	Apply power to the target system

3.	Remove power from the target system

4.	Remove power from the emulator.

# $ # + K Sample Files

TO BE EDITED

\PWRVIEWS\INTEL directory

DEMO.ABS

See also

Getting Startedgettingstarted

# $ # + K Stopping Emulation When Invalid Stack Area Accessed

1.	Check your stack base and stack size using the Stack Window's Options/Stack Area menu item or the StackInfo Shell Commandstackinfoshell.

2.	The last location in the stack equals the stack base minus the stack size.

3.	Define an event to trigger on writes to the address just below the last location in the stack.  Enter the address.  Select 0 for the r/w- signal.

4	Define a trigger on this event, selecting BRK to stop emulation.

# technicalnotes

$ Technical Notes

K answers;help;PowerViews;list

+ POWERVIEWS:0150

$ Comm Port Switching

# commportswitching

+ technical:0060

K communications;initialization file;pwrviews.ini

$ Improving PowerViews Performance

# improvingpowerviewsperformance

+ technical:0110

K PowerViews;performance;window

$ Function Keys

# functionkeys

+ technical:0090

K keys;function keys

$ Initialization File

# initializationfile

+ technical:0120

K initialization file;pwrviews.ini;files

$ Powering Down the Target

# poweringdownthe

+ technical:0155

K target

$ Sample Files

# samplefiles

+ technical:0170

K help;files

$ Stopping Emulation When Invalid Stack Area Accessed

# stoppingemulationwhen

+ technical:0235

K emulation;stack;trigger