# $ # + K Trigger Window

The Trigger Window allows bus eventseventterm to cause triggertriggerterm actions.  Triggers allow the detection of real-time bus activity and the making of a real-time decision based on the activity.

Screen Description

The left side sets the conditionselectingtriggerconditions to trigger on.  The right side is the trigger actionselectingtriggeractions to perform when the condition is true.

Menu Selections

File

Exit	Close the Trigger window.

Edit

Events...	Go to the Event Windowevent.

Options

Trace Control...	Configure trace parameterstracecontroldialog.

Timer 0 Unit

clock	Set timer 0 unit to be ep clock.

s	Set timer 0 unit to be second.

ms	Set timer 0 unit to be millisecond.

us	Set timer 0 unit to be microsecond.

Timer 1 Unit

clock	Set timer 1 unit to be ep clock.

s	Set timer 1 unit to be second.

ms	Set timer 1 unit to be millisecond.

us 	Set timer 1 unit to be microsecond.

Counter - Decimal Value

Set counter 0 and counter 1 unit to be decimal.

Counter - Hexadecimal Value

Set counter 0 and counter 1 unit to be  hexadecimal.

External Out - High 	Set external out to be high mode.

External Out - Low	Set external out to be low mode.

External  High-Z            Set external out to be high-Z mode.

Level

Show Level 0	Display actions for sequence level 0.  The conditions are the same for all levels.  The actions are different.

Show Level 1	Display actions for sequence level 1.

Show Level 2	Display actions for sequence level 2.

Show Level 3	Display actions for sequence level 3.

Windows	Move between windowsmovingbetweenwindows

Help	On-line helponlinehelp

Using the Trigger Window

Trace and Trigger Fundamentalstraceandtriggerfundamentals

Creating Eventscreatingevents

Selecting Trigger Conditionsselectingtriggerconditions

Selecting Trigger Actionsselectingtriggeractions

Using Multiple Trigger Levelsusingmultipletrigger

External Trigger In and Outexternaltriggerin

Trigger Examplestriggerexamples

# $ # + K Trace and Trigger Fundamentals

The trace features are:

-	128k trace frames.

-	Display trace as bus cycles and disassembled instructions.

-	Synchronize trace frames with source using the linked cursorlinkedcursor.

-	Trigger trace collection within buffers as:

-	Pre to collect cycles before the triggertriggerterm.

-	Center to collect an equal amount before and after the trigger.

-	Post to collect the trigger point and cycles after the trigger.

-	Trace search allows eventeventterm pattern recognition.

The trigger features are:

-	8 eventseventterm or combinations of address range, data range, external in signals and processor status.

-	4 levels of sequencingselectingtriggeractions.  Each level has unique actionsselectingtriggeractions.  The 8 events are the same for all levels.

-	2 counters or 2 timers combine with the events to cause actions when a user-specified count or time is reached.

Use trace to:

-	Record the real-time execution of code.

-	Linklinkedcursor the trace execution to the source.

-	View processor status pins.

Use triggers to:

-	Stop emulation on an eventevent.

-	Control collection of trace.

-	Detect complex sequencesselectingtriggeractions of events.

-	Count the number of times a subroutine is called before performing an action.

See also

Event Windowevent

Trace Windowtrace

Trigger Windowtrigger

# # $ K + Triggering on Access to a C Variable

To trigger on a variable:

1.	In the Event Windowevent, create an eventcreatingevents:

a.	Enter the symbolic name in the address start field.

b.	Enter a specific data value or leave the data fields blank to trigger on any value.

c.	Select one or multiple status code.

2.	In the Trigger Windowtrigger, set the trigger:

a.	Choose the event name from the event name list.

b.	Click on enable to activate the event.

c.	Click on brk to halt emulation when the event occurs.  Choose other actions as appropriate.

3.	Start emulationemulationterm:

a.	Emulation stops when the trigger is detected.

See also

Trace and Trigger Fundamentalstraceandtriggerfundamentals

Symbol Entry Syntaxsymbolentrysyntax

# $ # + K Selecting Trigger Conditions

The Trigger Windowtrigger conditions are:

-	0 to 7 events.  Use the Edit/Events... menu option to create events.

-     Cnt0/Tm0.   Toggle the button to switch them.

-     Cnt1/Tm1.   Toggle the button to switch them.

-	Ext in.           The bottom left is the external trigger inexternaltriggerin.

-	Select enable to activate an event, counter, timer or the external trigger.

Event conditions are verified in parallel at bus cycle.  All qualified actions are executed, even if multiple events are satisfied.

See also

Selecting Trigger Actionsselectingtriggeractions

# $ # + K Selecting Trigger Actions

The Trigger Windowtrigger actions are:

-	Seq to sequence to the specified level.  The specified level can contain the same or different conditions and actions.

-	Brk to break or stop emulationemulationterm.

-      Ton to start trace collection when the event matched.

-	Toff to stop trace collection when the event matched.

-	Tq to collect only the single frame when the event matched.

-	Tdis to turn trace machine off.  This occurs:

-	In pre-trigger mode, trace is turned off immediately.

-	In center trigger mode, trace is turned off after enough data has been collected such that the trigger is close to the center of the trace buffertracebufterm.

-	In post trigger mode, trace is turned off after enough data has been collected such that the trigger is near the beginning of the trace buffer.

-	In all cases (pre, post, or center) the Toff action is ignored if a trigger has already occurred in this trace buffer.

-	If Counters are chosen:

-	Inc0 increments the first counter by 1.

-	Rst0 sets the first counter to 0.

-	Inc1 and Rst1 are for the second counter.

-	If Timers are chosen:

-	Start0 increments the first timer by 1 clock.

-	Rst0 sets the first timer to 0.

-      Stop0 stops the first timer.

-	Start1, Stop1 and Rst1 are for the second timer.

- 	Ext out sets the external trigger outputexternaltriggerin logic level to be high, low or high-Z mode.

See also

Selecting Trigger Conditionsselectingtriggerconditions

# $ # + K Using Multiple Trigger Levels

Four trigger levels allow the real-time detection of complex eventseventterm

An example of multiple levels is:

-	There are two functions that contain calls to the same subroutine.

-	Halt emulation if the subroutine is called from the one function, but not the other.

-	The subroutine may or may not be called each time the functions are executed since the subroutine is in conditional code.

To program the Trigger Windowtrigger to accomplish this:

-	Create an eventcreatingevents called breakfunction for the subroutine being called.

-	Create an event called goodcaller with the address of the caller that causes the break.

-	Create a third event called badcaller with its address.

-	Create a fourth event called goodcallerreturn with the return address of the goodcaller function.

-	On level 0:

-	Breakfunction is not enabled.

-	Goodcaller uses seq to set next level(1).

-	Badcaller is not enabled.

-	Goodcallerreturn is not enabled.

-	On level 1:

-	Breakfunction has brk set to stop emulation.

-	Goodcaller is not enabled.

-	Badcaller uses seq to set level 0 to return to level 0 if it is called before Breakfunction can break.

-	Goodcallerreturn uses seq to set level 0 to return to level one if Breakfunction not called.

Use the Trigger Window'strigger Level menu to change the level, which is displayed in the window title bar.

See also

Selecting Trigger Actionsselectingtriggeractions

Selecting Trigger Conditionsselectingtriggerconditions

# # $ K + Using Counters

Counters are incremented and reset one of the trigger actionsselectingtriggeractions.  Use the counters to perform an action after a condition occurs a certain number of times.  For example:

-	Stop emulation on the 7th iteration of a for loop.

-	Program the trigger:

-	Create an eventcreatingevents with the address of a line within the for loop that is fetched each time.

-	Select the event in the Trigger Window's event name conditionselectingtriggerconditions, and select enable and inc0.

-	Set the cnt0 condition to 7, and select the enable and the brk boxes.

See also

Selecting Trigger Actionsselectingtriggeractions

Selecting Trigger Conditions

# # $ K + Using Timers

Timers are start, stop and rst one of the trigger actionsselectingtriggeractions.  Use the timer to perform an action after a condition occurs a period of time.  For example:

-	Stop emulation on the 2 ms of a for loop.

-	Program the trigger:

-	Create an eventcreatingevents with the address of a line within the for loop that is fetched each time.

-	Select the event in the Trigger Window's event name conditionselectingtriggerconditions, and select enable and start0.

-	Set the tm0 condition to 2 ms, and select the enable and the brk boxes.

See also

Selecting Trigger Actionsselectingtriggeractions

Selecting Trigger Conditionsselectingtriggerconditions

selectingtriggerconditions

# $ # + K External Trigger-In and Out

The Trigger Windowtrigger allows the Trigger-In BNC connection to be combined with eventseventterm and counterusingcountersand conditions selectingtriggerconditions and timerusingcountersand conditionsselectingtriggerconditions to perform trigger actionsselectingtriggeractions.

External Trigger-In signals is TTL  level compatible.

The external Trigger-Out connector level is set with low,  high or high-Z as one of the trigger actionsselectingtriggeractions.  The output state is choosen in the Options menu of the trigger window.  The external Trigger-In condition, when enabled, is recognized when the level on the Trigger-In is matched.

# $ # + K Trigger Examples

Many of the trigger subjects have examples.  Some additional ones are:

Stopping Emulation When Invalid Stack Area Accessedstoppingemulationwhen

Triggering on Access to a C Variabletriggeringonaccess

See also

Trigger Windowtrigger  for all the trigger subjects.

# $ # K Trigger

A trigger causes an action or actions to occur when one or more eventseventterm are matched on specified sequence levels.  Triggers allow the detection of real-time bus activity and the making of a real-time decision.

For example, "halt emulation when a variable is written" is a trigger.

The trigger point in the trace is controlled by the Trigger Windows' Options/Trace Control...menu item.  The modes are:

-	Pre to collect trace before the trigger.  The trigger is near the end of the buffer.

896 to 768 frames are collected before the trigger.

128 to 256 frames are collected after the trigger.

-	Post to collect trace after the trigger.  The trigger is near the beginning of the buffer.

128 to 256 frames are collected before the trigger.

896 to 768 frames are collected after the trigger.

-	Center to collect an equal amount before and after the trigger.  The trigger is near the center of the buffer.

About 512 frames are collected before and after the trigger.

# $ # K Trace Control Dialog Box

The fields are:

-	The triggertriggerterm position within each trace buffer.

-      Emulation Break on Full checkbox. Use it to break emulation when trace buffer is full.

-      Trace off on Full checkbox. Use it to stop trace collection when trace buffer is full.

See also

Pre, Post, and Centered Trace Collectionprepostand

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # K Trace Buffer

A trace buffer is a collection of program execution information.

$ Trigger Window

# trigger

+ powerviews:0120

K trigger;window

$ Trace and Trigger Fundamentals

# traceandtriggerfundamentals

+ technical:0250

K trace;trigger;event

# triggeringonaccess

$ Triggering on Access to a C Variable

K trigger;C;variable

+ TECHNICAL:0260

$ Selecting Trigger Conditions

# selectingtriggerconditions

+ usingthetrigger:0020

K trigger

$ Selecting Actions

# selectingtriggeractions

+ usingthetrigger:0030

K trigger;trigger actions;actions

$ Using Multiple Trigger Levels

# usingmultipletrigger

+ usingthetrigger:0040

K trigger

# usingcountersand

$ Using Counters and Timers

K trace;timer;counter;trigger;

+ USINGTHETRIGGER:0050

# usingcountersand

$ Using Counters and Timers

K trace;timer;counter;trigger;

+ USINGTHETRIGGER:0050

$ External Trigger In and Out

# externaltriggerin

+ usingthetrigger:0060

K trigger

$ Trigger Examples

# triggerexamples

+ usingthetrigger:0080

K trigger

$ Trigger

# triggerterm

K trigger;event trigger

$ Trace Control Dialog Box

# tracecontroldialog

K trace;trace control;dialog box

$ Trace Buffer

# tracebufterm

K trace;