# # $ K Stack Size is 0.  It will be reset to 1K.

Cause:	The stack size is set to 0, which is invalid.

Solution:	No action is necessary unless you want the stack to be larger than 1 KB.

See also

setStackArea Shell Commandsetstackareashell

# # $ K Stack area will be filled with a pattern to enable HighWaterMark.

Cause:	You have enabled the high water mark and MICEpack/SLD must fill the stack area in order to determine where the HighWaterMark is.

Solution:	This is information only.

See also

HighWaterMarkhighwatermark

# # $ K No stack frames available.

Cause:	The stack pointer may not be within the stack area.

Solution:	Check that the stack frame pointer and stack pointer are within the current stack area.

See also

setStackArea Shell Commandsetstackareashell

# # $ K Stack session name not found.

Cause:	The specified name is invalid.

Solution:	Specify a valid stack session name.

# # $ K SWAT operation aborted to change mode.

Cause:

Solution:	.

# # $ K SWAT hardware overflowed, information is missing.

Cause:

Solution:	.

# # $ K SWAT not responding to commands.

Cause:

Solution:	.

# # $ K The number of hardware breakpoints exceeds capability.

Cause:	The number of hardware breakpoints is already at the maximum.

Solution:	Remove one or more of the breakpoints.

# # $ K Cannot configure the probe hardware.

Cause:	Internal error

Solution:	Contact Microtekcontactingmicrotek.

# # $ K Ethernet is not yet available.

Cause:	Internal error

Solution:	Contact Microtekcontactingmicrotek.

# # $ K Cannot add line number symbols.

Cause:	Line number records in this loadfile are not in a standard format.

Solution:	Verify that you are compiling your program with debug symbols

# # $ K Specified line number is out of range.

Cause:	(Internal error) Line number records have been corrupted.

Solution:	Try reloading symbols.  If this does not correct the problem contact Microtekcontactingmicrotek.

# # $ K Cannot create symbol type.

Cause:	Can't create a symbol type during load.  Possibly a module from a non-supported language is being linked in - either from your code, or libraries you've linked in (Pascal, Ada, and FORTRAN debug record types are not currently supported).

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K Symbol not located in table - base index too large.

Mismatched error message.  Contact Microtekcontactingmicrotek

# # $ K C++ duplicate name detected.

Mismatched error message.  Contact Microtekcontactingmicrotek

# # $ K C++ type not handled: mangled names may result.

Mismatched error message.  Contact Microtekcontactingmicrotek

# # $ K Loadfile compiled using <compiler> tool is incompatible with current Compiler Used.

Cause:	The current loadfile was compiled using a different compiler than the one selected.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

Also, make sure that you used the correct compiler and linker switches.

# # $ K Invalid register ID.

Cause:	A register ID in the current loadfile is invalid.

Solution:	Check for a corrupted file, or a file not in OMF86/OMF386 format.

# # $ K The NOT feature is not supported for this field.

Cause:	The NOT feature (BOOLEAN negation) is not allowed for a field.

Solution:	You must negate the value before entering it.

# # $ K No active trigger found.

Cause:	There is no active trigger.

Solution:	.

# # $ K Invalid tracing mode.

Cause:	The tracing mode selected is invalid.

Solution:	.

# # $ K The variable was not found in Shell.

Cause:	The variable was not found.

Solution:	Check the spelling carefully, and remember to completely specify the variable.

# # $ K Cannot find address in symbol table.

Cause:	Attempted lookup of an address in the symbol table failed to find a matching symbol at this address.  Symbols may not have been loaded, or this address is out of the range of symbol addresses.

Solution:	Verify your program has been compiled and linked with debug symbols.

# # $ K The attempt to edit the value has failed.

Cause:	The variable name, module name, function name, or value were specified incorrectly, so the variable value edit failed.

Solution:	Check the spelling carefully, and remember to completely specify the variable.

# # $ K Public symbol updated.

Cause:	Debug information for variables can occur several times in a loadfile.  When the first occurrence is an 'external' reference, the symbol table creates an initial entry for the symbol.  The later processing of debug records the loadfile enhances the symbol record with additional information.

Solution:	No action required;  informational message only.

# # $ K Cannot set end address smaller than start address.

Cause:	Obsolete error.

Solution:	Please report this problem to Microtekcontactingmicrotek.

# # $ K No symbols are loaded.

Cause:	Full symbolic information has not been loaded (enabling source-level debugging).

Solution:	If you have an assembly language program, only public symbols are loaded, resulting in this message.  If you have high-level language modules in your program (C source files), be sure you have specified debug symbols in your compilation switches.

# # $ K Invalid trigger event index.

Cause:	Internal error.  No user error.

Solution:	Please report this problem to Microtekcontactingmicrotek.

# # $ K Cannot initialize stack area for HighWaterMark.

Cause:	Internal error.  No user error

Solution:	Please report this problem to Microtekcontactingmicrotek.

# # $ K Cannot set section base in symbol table.

Cause:	Section bases may already be defined, or, your application may be defining more than the maximum number of sections supported (in which  case the remainder are discarded).

Solution:	To explicitly clear out sections defined by previous loadfiles (which should automatically happen when you load a new program), you can use the removesymbols Shell command.  If you have more than 32 sections defined in your application, you can merge sections by the mergeSection Shell command (which is adequate for source-level debugging, but may not be adequate for section relocation).

# # $ K No module to view since no file is loaded.

Cause:	The source and the symbol table are out of synchronization.

Solution:	Reload the symbols, then close/reopen the Source window.

# # $ K No module to view since no file is loaded.

Cause:	No loadfile has been loaded, so there is no source module available to browse.

Solution:	Load a file, then try again.

# # $ K Module does not have a valid address range.

Cause:	Usually, a module doesn't have a valid address range if it is a data module (i.e. if the module contains only data definitions).

Solution:	Add a dummy function to the data module, and recompile it, to force the compiler to generate an address range for the module and allow you to view the module in the Source window.

# # $ K Invalid symbol address type.

Cause:	The text you entered does not correspond to any recognized form of symbolic address.  The input conventions for the types of address are:

1.	Hex values, which must begin with 0x -- such as 0x11.

2.	Decimal values, using only the digits 0..9.

3.	Symbolic addresses, which must begin with a '#'.

Solution:	Try re-entering the address using these conventions.

See also

Symbol Entry Syntaxsymbolentrysyntax

# # $ K No source module(s) loaded.

Cause:	No source modules have been loaded.

Solution:	Try reloading, and if you have high-level language modules in your program (C source files), be sure you have specified debug symbols in your compilation switches.

# # $ K Peripheral configuration file not found.

Cause:	The peripheral configuration file is not in the specified directory.

Solution:	Be sure you specified the correct filename and path.  (If you did not specify a path, the MICEpack/SLD directory was searched for this file.)

# # $ K Address addition overflowed.

Cause:	One situation that can cause this error to be generated is dereferencing a structure pointer that already points near the end of memory.

Solution:	Be sure data values for your local variables are properly initialized before dereferencing their values.

# # $ K Peripheral register is unknown.

Cause:	The register name does not conform to the Intel mnemonics.

Solution:	Check the register name.

# # $ K Corrupted peripheral configuration file found.

Cause:	File is corrupted.

Solution:	Edit the file or re-create the file.

# # $ K Unable to close peripheral configuration file.

Cause:	The file for saving or restoring peripherals cannot be closed.

Solution:	A DOS error has occurred.  Check the file and try again.

# # $ K Unexpected end of peripheral configuration file found.

Cause:	The peripheral configuration file is corrupted in the MICEpack/SLD default directory (e.g. c:\mp186).

Solution:	The peripheral configuration file has been truncated.  Re-install the MICEpack/SLD software.

# # $ K Register found with unknown peripheral group.

Cause:

Solution:

# # $ K Attempted to edit register with an invalid value.

Cause:	You entered an invalid register value.

Solution:	Re-edit the register with the correct value.

# # $ K Encountered peripheral configuration EOF.

Cause:

Solution:

# # $ K TRegister object failed to construct during initialization.

Cause:

Solution:

# # $ K TField object failed to construct during initialization.

Cause:

Solution:

# # $ K TGroup object failed to construct during initialization.

Cause:

Solution:

# # $ K Peripheral window creation failed.

Cause:

Solution:

# # $ K Unable to create peripheral display cache.

Cause:

Solution:

# # $ K Attempted to add a display cache entry out of sequence.

Cause:

Solution:

# # $ K Display cache entry found has no match.

Cause:

Solution:

# # $ K Reached end of available cache.

Cause:

Solution:

# # $ K + Expected line number block.

Cause:	Module contains only data definitions, which causes the compiler not to generate a line number block for that module.  Usually, you cannot browse the module in the Source window.

Solution:	Add a dummy function to the module and recompile it to force the compiler to generate a line number block for the module.  For example, add the following dummy function: void dummy Function() {/* do nothing */}

# # $ K Unable to open peripheral configuration file.

Cause:	A peripheral configuration file is missing or invalid.

Solution:	If you have moved MICEpack/SLD after your initial installation, make sure to copy the peri????.cfg and *.pcf files to the new location.

# # $ K Location of peripheral block is indeterminate.

Mismatched error message.  Contact Microtekcontactingmicrotek

# # $ K + Module %s does not contain line number information.

Cause:	The module was compiled without debug information.  You cannot browse the module in the Source window.

Solution:	Recompile with the debug information switches turned on.

# # $ K + No CompilerUsed defined in MP186.INI file.

Mismatched error message.  Contact Microtekcontactingmicrotek

# # $ K + Peripheral configuration file version is newer than one this version of MICEpack/SLD can read.

Cause:	This version of the configuration file data format is higher than one which this version of MICEpack/SLD can read.

Solution:	Either use the older peripheral configuration files that came with this version of MICEpack/SLD, or upgrade the version of MICEpack/SLD to the latest version.

# # $ K + Unexpected type: %s.

Cause:	The loader found a bad data type.

Solution:	  Contact Microtekcontactingmicrotek.

# # $ K + Memory Window Count Exceeded.

Cause:	Since the maximum number of Memory windows are already open, you cannot open another new Memory window.

Solution:	Use one of the open Memory windows.

# # $ K + Unable to open Memory Window.

Cause:	Opening another Memory window failed because there are insufficient system resources.

Solution:	Close other MICEpack/SLD windows or other Windows applications to free system resources, and then try again to open a new Memory window.

# # $ K Space designator is incompatible with current processor.

Cause:  Space designators are processor specific.

Solution:	Space designators are UP, UD, SP, SD, and CPU.

# # $ K Address mode does not conform to processor standards.

Cause:	Address mode is processor specific.

Solution:

# # $ K Address offset exceeds processor's addressing limit.

Cause:	Address is out of bounds.

Solution:	Use the Limit Checking Tablelimitcheck to verify the address.

# # $ K Limit Checking Table

| Type | Pmode | Processor | Limits |
| --- | --- | --- | --- |
| Virtual | SMM | all | 0:0 - FFFF:FFFFFFFF |
|  | Real | all | 0:0 - FFFF:FFFF |
|  | Virtual-86 | all | 0:0 - FFFF:FFFF |
|  | Protect16, 32 | all | selector £ table limit; offset within segment limit |
| Linear | all | all | 0 - FFFFFFFF |
| Physical | all | 386DX | 0 - FFFFFFFF |
|  | all | 386SX | 0 - FFFFFF |
|  | all | 386CX | 0 - 3FFFFFF |
|  | all | 386EX | 0 - 3FFFFFF |

# # $ K Address offset exceeds processor's segment limit.

Cause:	Address offset is out of bounds for the given segment.

Solution:	Use the Limit Checking Tablelimitcheck to verify the address.

# # $ K LDT specified does not exist.

Cause:	LDT register contained an invalid value.

Solution:	Initialize the LDT register or specify a valid LDT.

# # $ K Unable to translate physical address to linear address.

The address server does not support physical to linear address translation.

Internal error.  Contact Microtekcontactingmicrotek.

# # $ K Unable to translate linear address to virtual address.

Cause:	There is no information available for the address translation.

Solution:	Update symbolic information.

# # $ K Unable to compare addresses of differing segments.

Cause:	Addresses compared do not have the same segment.

Solution:	Specify addresses of the same segment for comparison purposes.

# # $ K Unable to compare addresses of differing types.

Cause:	Addresses compared do not have the same type.

Solution:	Specify addresses of the same type for comparison purposes.

# # $ K Unable to compare addresses of differing spaces.

Cause:	Addresses compared do not have the same space.

Solution:	Specify addresses of the same space for comparison purposes.

# # $ K Requested operation is incompatible with processor.

Cause:	A processor-specific routine was accessed while operating with the wrong processor family.

Solution:	Internal error.  Contact Microtekcontactingmicrotek.

# # $ K The GDT null descriptor is invalid for use in an address.

Cause:	Indexing into the GDT resulted in the null descriptor, which always generates an exception.

Solution:	The upper 13 bits of the segment selector contains the GDT index.  An index of 0 references the GDT null descriptor which is invalid in the context of an address.  Make sure that the upper 13 bits do not result in an index of 0.

# # $ K Requested offset in the descriptor table exceeds the physical limits.

Cause:	Indexing into a descriptor table with the specified segment selector resulted in an offset exceeding the physical limits of the table.

Solution:	The upper 13 bits of the segment selector contains the descriptor table index.  Specify an index inside the descriptor table limit.  Specify an index into the descriptor table which is less than the limit.

# # $ K Specified segment is no longer valid or present in memory.

Cause:	Specified segment contains a cleared P-bit.

Solution:	The system deallocated this segment and it needs to be loaded into memory before use.  A segment not present exception will be generated if the descriptor selector is loaded into a segment register.  Do not use the segment.

# # $ K Segment specified is not a valid type for requested use.

Cause:	The segment specified may be a system segment and not a valid code or data segment.

Solution:	Use valid code and data segments for addresses.  When the S bit is set in the segment descriptor it is a valid code or data segment.

# # $ K The page table entry is not present during page translation.

Cause:	The present bit in the page table entry is 0 and the page is not in memory.  During address translation, the accessed page table entry had a cleared present bit.  The page table entry cannot be used for physical address translation.

Solution:	Update the page tables.

# # $ K The pmode is uninitialized and prevents address translation.

Cause:	Address translation depends on current processor mode.  The mode is uninitialized and the address can not be translated.

Solution:	Initialize processor mode prior to address translation requirements.  To initialize the processor, operate it in one of the following legal modes:  Real, V86, Prot16, Prot32, SMM.

# # $ K Shell command specified is not supported for the current processor.

Cause:	The shell command is processor-specific and is not supported on the current processor.

Solution:	Search Help index for "shell" for legal commands for the current processor.

# # $ K MBAR is invalid; no other SIM registers will be processed.

Cause:	The valid bit in the MBAR register is reset, which prevents access to the remaining registers.

Solution:	This for your information only.

# # $ K Invalid pod version.

Cause:	The hardware configuration EEPROM contents are not recognized.

Solution:	This error occurs if the EEPROM contents are altered or erased.  Contact Microtekcontactingmicrotek for how to recover the data.

# # $ K Timeout waiting for processor to enter ice mode.

Cause:	The emulation processor should have entered ICE mode.

Solution:	Check the RESET, READY#, HOLD, and CLK2 signals.

# # $ K Timeout waiting for processor to respond to commands.

Cause:	The emulation processor did not complete a command.

Solution:

-	Check the RESET, READY#, HOLD, and CLK2 signals.

-	There may be a bus contention on an address, status, or data bus.

# # $ K Timeout waiting for processor to process breakpoint.

Cause:	The emulation processor entered ICE mode, but did not complete the breakpoint-processing sequence.

Solution:

-	Check the RESET, READY#, HOLD, and CLK2 signals.

-	There may be a bus contention on an address, status, or data bus.

# # $ K Insufficient overlay memory.

Cause:	You tried to map more overlay memory than you have.

Solution:	Try one or both of the following solutions:

-	Reduce the size of memory being mapped.

-	Set another region of memory to Target.

# # $ K Emulation adapter is not connected properly to target.

Cause:  The adapter is not properly connected, or the adapter is damaged.

Solution:	Try the following solutions:

-	Remove and re-seat the adapter.

-	Verify that pin 1 of the emulator aligns with pin 1 of the target.

-	Check for broken or bent pins on the adapter.

# # $ K Cannot drive target processor FLT# pin to low State.

Cause:	Target FLT# pin is improperly driven on the user target.

Solution:

-	The target must not actively drive FLT# or connect FLT# to Vcc.  A register pullup is recommended.

-	Check for the proper orientation and seating of the adapter.

# # $ K Target power is off.

Cause:	Your request cannot be serviced because the target power is off.

Solution:

-	Turn on the target power.

-	Check for proper orientation and seating of the adapter.

# # $ K Cannot drive emulation processor FLT# pin to high state.

Cause:	Emulator FLT# pin is stuck in the low state.

Solution:	Contact Microtekcontactingmicrotek.

# # $ K No clock input from target.

Cause:	CLK2 is off or is very slow.

Solution:	The emulator requires a minimum of 262 KHz CLK2 input.

-	Increase CLK2 to at least 262 KHz.

-	Check for proper orientation and seating of target adapter.

# # $ K Processor reset during operation.

Cause:	Reset was asserted by the target during the last operation.

Solution:	Try the following actions:

-	Turn off watchdog timers or other sources of reset.

-	Disable reset from the Signals menu of the CPU window.

-	Check for the proper orientation and seating of the adapter.

# # $ K Hold is continuously asserted.

Cause:	Hold input was asserted continuously, preventing access to the processor bus,

Solution:	Try the following actions:

-	Disable target sources of hold.

-	Disable hold from the Signals menu of the CPU window.

-	Check for the proper orientation and seating of the target adapter.

# # $ K Read operation aborted prior to completion.

Cause:	Abort was requested during a read operation.

Solution:	 No action required;  informational message only.

# # $ K Test operation aborted prior to completion.

Cause:	Abort was requested during a ramtst operation.

Solution:	No action required;  informational message only.

# # $ K Unknown test command received by emulator.

Cause:	Internal error

Solution:	Contact Microtekcontactingmicrotek.

# # $ K Unsupported event recognizer type.

Cause:	Internal error

Solution:	Contact Microtekcontactingmicrotek.

# # $ K Invalid loader server.

Cause:	The Loader server is not initialized correctly.  This problem can have several causes:

1.	There is not enough memory.

2.	LOADERS.INI is missing from the MICEpack/SLD directory.

3.	MICEpack/SLD cannot open LOADERS.INI.

Solution:	Use the corresponding solution:

1.	Install more memory or close other applications.

2.	Reinstall MICEpack/SLD.

3.	Increase the number of files (files=<number> in CONFIG.SYS), or close other open files.

# # $ K Invalid File Handle %s.

Cause:	Error occurred when MICEpack/SLD attemped to read the file.

Solution:	 This could be caused by a corrupted file.

# # $ K Cannot get file %s timestamp.

Cause:	Corrupted load file.

Solution:	Report problem to Microtekcontactingmicrotek.

# # $ K Invalid input timestamps.

Cause:	File timestamps are corrupted.

Solution:	Report problem to Microtekcontactingmicrotek.

# # $ K Loader is loading.  Unable to perform the requested operation.

Cause:	Loading operation is in progress.

Solution:	Wait for the operation to complete, or cancel the current loading operation.

# # $ K Loader %s is not found.

Cause:	The loader DLL is missing, or the Loader server is corrupted.

Solution:	Use the resetLoaders shell command to reset and try again.  You may need to re-install MICEpack/SLD.

# # $ K Cannot create function name %s for loader %s.

Cause:	The loader DLL is not compatible with the current MICEpack/SLD system.

Solution:	Report problem to Microtekcontactingmicrotek.

# # $ K Cannot get loader function %s.

Cause:	The loader DLL is corrupted.

Solution:	Report problem to Microtekcontactingmicrotek.

# # $ K Loader %s is not loaded.

Cause:	The loader DLL is missing.

Solution:	Use the resetLoaders Shell command to re-initialize the loader.  If this does not solve the problem, try close other applications to free-up memory, or install more memory.

# # $ K Loadfile %s is not found.

Cause:	The input loadfile is not found.

Solution:	Make sure that the input loadfile path and filename is specified correctly.

# # $ K Bad loadfile.  Unknown load format signature.

Cause:	The input loadfile is not a supported load format (OMF86, OMF386).

Solution:	Check the input loadfile and retry.

# # $ K Cannot locate the LOADERS.INI file.

Cause:	The LOADERS.INI file is missing

Solution:	Check for the LOADERS.INI file in the current MICEpack/SLD directory, or re-install  MICEpack/SLD to get a valid LOADERS.INI file and use the ResetLoaders Shell Command to reset.

# # $ K Invalid number of loaders specified in the LOADERS.INI file.

Cause:	The LOADERS.INI file is corrupted.

Solution:	Re-install MICEpack/SLD to get a valid LOADERS.INI file and use the ResetLoaders Shell Command to reset.

# # $ K Missing loader initialization information.

Cause:	The LOADERS.INI file is corrupted.

Solution:	Re-install MICEpack/SLD to get a valid LOADERS.INI file and use the ResetLoaders Shell Command to reset.

# # $ K Missing loader library name.

Cause:	The LOADERS.INI file is corrupted.

Solution:	Re-install MICEpack/SLD to get a valid LOADERS.INI file and use the ResetLoaders Shell Command to reset.

# # $ K Missing loader format signature number.

Cause:	The LOADERS.INI file is corrupted.

Solution:	Re-install MICEpack/SLD to get a valid LOADERS.INI file and use the ResetLoaders Shell Command to reset.

# # $ K Too many load format signatures.

Cause:	The LOADERS.INI file is corrupted.

Solution:	Re-install MICEpack/SLD to  get a valid LOADERS.INI file and use the ResetLoaders Shell Command to reset.

# # $ K Missing loader format signature.

Cause:	The LOADERS.INI file is corrupted.

Solution:	Re-install MICEpack/SLD in order to reload the LOADERS.INI file and use the ResetLoaders Shell Command to reset.

# # $ K Unknown load format.

Cause:	The input loadfile is not a supported load format.

Solution:	Check the input loadfile and retry.

# # $ K Assertion error.

Cause:	Internal error

Solution:	Report problem to Microtekcontactingmicrotek.

# # $ K Cannot open the requested dialog box.

Cause:	Windows sytem resources are low.

Solution:	Free-up Windows resources and try again.

# # $ K Path to %s file is not available.

Cause:	Internal error

Solution:	Report problem to Microtekcontactingmicrotek.

# # $ K Unable to read %s file.

Cause:	Error occurred while trying to read the file.

Solution:		Check the path and file name.  On a network, check file and directory permissions.

# # $ K Unable to write %s INI file.

Cause:	Error occurred while trying to write to the file.

Solution:	Check the path and file name.  On a network, check file and directory permissions.

# # $ K Exceeded the Maximum type (32767) allowed by the OMF86 spec.

Cause:	The OMF86 type index must be between 0 and 32767.  Types which exceed this limit cause symbols to resolve incorrectly.

Solution:	Warning.  No action required.

# # $ K Invalid symbol type signature

Cause:	An invalid symbol type record was encountered.

Solution:	Regenerate the load file.

# # $ K Logical address is not handled by OMF86 loader.

Cause:	Logical address was encountered; load file may not be usable.

Solution:	The OMF86 file must be in absolute format.

# # $ K Multiple segments were found in the same module.

Cause:	OMF86 loader does not support multiple segments in the same object module.

Solution:	Report problem to Microtekcontactingmicrotek.

# # $ K OMF file encountered; not compatible with current XXXXX processor in use.

Cause:	Attempting to load an OMF file but the target processor is not an X86 processor.

Solution:	Use a different load file.

# # $ K Error Opening A File

If you get an error attempting to open a file, check the following:

1.	File name and pathname are correctly specified.

2.	Number of open files.  Increase the number of files (files=<number> in CONFIG.SYS), or close other open files.

3.	Hard drive space.  If your drive is full, you will not be able to open a file.

4.	RAM Memory.  Close other applications or install more memory.

# HE_STACK_SIZE_0

$ Stack Size is 0.  It will be reset to 1K.

K error, o-t

# HE_STK_FILL_STACK_AREA

$ Stack area will be filled with a pattern to enable HighWaterMark.

K error, o-t

# HE_STK_NO_FRAMES

$ No stack frames available.

K error, h-n

# HE_STK_SESSION_NAME_NOT_FOUND

$ Stack session name not found.

K error, o-t

# HE_SWAT_ABORT

$ SWAT operation aborted to change mode.

K error, o-t

# HE_SWAT_PA_OVERFLOW

$ SWAT hardware overflowed, information is missing.

K error, o-t

# HE_SWAT_TIMEOUT

$ SWAT not responding to commands.

K error, o-t

# HE_TOO_MANY_XBRKS

$ The number of hardware breakpoints exceeds capability.

K error, o-t

# HE_XILINX_PROGRAMMING

$ Cannot configure the probe hardware.

K error, a-g

# HE_ETHERNET_NOT_SUPPORTED

$ Ethernet is not yet available.

K error, a-g

# HE_LINENUM_BAD_STATE

$ Cannot add line number symbols.

K error, a-g

# HE_VS_RANGE

$ Specified line number is out of range.

K error, o-t

# HE_SYM_TYPE

$ Cannot create symbol type.

K error, a-g

# HE_TABLE_SMALLER_THAN_INDEX

$ Symbol not located in table - base index too large.

K error, o-t

# HE_CPP_DUPLICATE

$ C++ duplicate name detected.

K error, a-g

# HE_CPP_NOT_SUPPORTED

$ C++ type not handled: mangled names may result.

K error, a-g

# HE_LOADFILE_HAD_INCOMPATIBLE_TOOLCHAIN

$ Loadfile compiled using <compiler> tool is incompatible with current Compiler Used.

K error, h-n

# HE_INVALID_REGID

$ Invalid register ID.

K error, h-n

# HE_NOT_NOT_ALLOWED

$ The NOT feature is not supported for this field.

K error, o-t

# HE_NO_TRIG_DEFINED

$ No active trigger found.

K error, h-n

# HE_INVALID_MODE

$ Invalid tracing mode.

K error, h-n

# HE_VS_CLI_VAR_NOT_FOUND

$ The variable was not found in Shell.

K error, o-t

# HE_ADDRESS_NOT_FOUND

$ Cannot find address in symbol table.

K error, a-g

# HE_VS_VALU_EDIT_FAIL

$ The attempt to edit the value has failed.

K error, o-t

# HE_ADDRESS_UPDATED

$ Public symbol updated.

K error, o-t

# HE_ADR_END_ADDR_TOO_SMALL

$ Cannot set end address smaller than start address.

K error, a-g

# HE_SYMBOL_NOT_A_MODULE

$ No symbols are loaded.

K error, h-n

# HE_BAD_EVENT

$ Invalid trigger event index.

K error, h-n

# HE_STK_INIT_STK_FAIL

$ Cannot initialize stack area for HighWaterMark.

K error, a-g

# HE_SYMBASE

$ Cannot set section base in symbol table.

K error, a-g

# HE_SRC_FIND_ADDRESS

$ No module to view since no file is loaded.

K error, h-n

# HE_SRC_NO_LOADFILE

$ No module to view since no file is loaded.

K error, h-n

# HE_SRC_MODULE_HAS_NO_ADDR_RANGE

$ Module does not have a valid address range.

K error, h-n

# HE_SYM_SYM_START_NOT_SHARP

$ Invalid symbol address type.

K error, h-n

# HE_SYM_NO_MODULES_LOADED

$ No source module(s) loaded.

K error, h-n

# PP_NO_CFG_STR

$ Peripheral configuration file not found.

K error, o-t

# HE_ADR_RESULT_OVERFLOW

$ Address addition overflowed.

K error, a-g

# PP_UNK_REG_STR

$ Peripheral register is unknown.

K error, o-t

# PP_CORR_CFG_STR

$ Corrupted peripheral configuration file found.

K error, a-g

# PP_CLOSE_CFG_STR

$ Unable to close peripheral configuration file.

K error, u-z

# PP_UNEX_CFG_STR

$ Unexpected end of peripheral configuration file found.

K error, u-z

# PP_UNK_GRP_STR

$ Register found with unknown peripheral group.

K error, o-t

# PP_INV_VALUE_STR

$ Attempted to edit register with an invalid value.

K error, a-g

# PP_CFG_EOF_STR

$ Encountered peripheral configuration EOF.

K error, a-g

# PP_TREG_FAIL_STR

$ TRegister object failed to construct during initialization.

K error, o-t

# PP_TFLD_FAIL_STR

$ TField object failed to construct during initialization.

K error, o-t

# PP_TGRP_FAIL_STR

$ TGroup object failed to construct during initialization.

K error, o-t

# PP_WIN_FAIL_STR

$ Peripheral window creation failed.

K error, o-t

# PP_NO_CACHE_STR

$ Unable to create peripheral display cache.

K error, u-z

# PP_INV_CACHE_ENT_STR

$ Attempted to add a display cache entry out of sequence.

K error, a-g

# PP_UNK_CACHE_ENT_STR

$ Display cache entry found has no match.

K error, a-g

# PP_END_CACHE_STR

$ Reached end of available cache.

K error, o-t

# HE_LINE_EXPT

$ Expected line number block.

K error, a-g;

+ ERRORS4

# HE_PP_OPEN_CFG_FAIL

$ Unable to open peripheral configuration file.

K error, u-z;

# HE_PP_INVALID_BASE

$ Location of peripheral block is indeterminate.

K error, h-n

# HE_NO_LINEBLK_IN_MODULE

$ Module %s does not contain line number information.

K error, h-n;

+ ERRORS4

# HE_NO_TOOLUSE_DEFINED

$ No CompilerUsed defined in MP186.INI file.

K error, h-n;

+ ERRORS4

# HE_PP_OLD_VERSION

$ Peripheral configuration file version is newer than one this version of MICEpack/SLD can read.

K error, o-t;

+ ERRORS4

# HE_UNEXPECTED_TYPE

$ Unexpected type: %s.

K error, u-z;

+ ERRORS4

# HE_MEM_COUNT_EXCEEDED

$ Memory Window Count Exceeded.

K error, h-n

+ ERRORS4

# HE_CANT_OPEN_MEM_WINDOW

$ Unable to open Memory Window.

K error, u-z

+ ERRORS4

# HE_ADR_SPACE_INVALID

$ Space designator is incompatible with current processor.

K error, o-t;

# HE_ADR_MODE_INVALID

$ Address mode does not conform to processor standards.

K error, a-g;

# HE_ADR_EXCEEDS_ADR_LIMIT

$ Address offset exceeds processor's addressing limit.

K error, a-g;

# limitcheck

$ Limit Checking Table

K addresses;limit checking;

# HE_ADR_EXCEEDS_SEG_LIMIT

$ Address offset exceeds processor's segment limit.

K error, a-g;

# HE_ADR_LDT_INVALID

$ LDT specified does not exist.

K error, h-n;

# HE_ADR_PHYS_TO_LINEAR_NOT_SUPPORTED

$ Unable to translate physical address to linear address.

K error, u-z;

# HE_ADR_LINEAR_TO_VIRTUAL_INVALID

$ Unable to translate linear address to virtual address.

K error, u-z;

# HE_ADR_DIFFER_SEGMENTS

$ Unable to compare addresses of differing segments.

K error, u-z;

# HE_ADR_DIFFER_TYPES

$ Unable to compare addresses of differing types.

K error, u-z;

# HE_ADR_DIFFER_SPACES

$ Unable to compare addresses of differing spaces.

K error, u-z;

# HE_PRC_OPER_INCOMPAT

$ Requested operation is incompatible with processor.

K error, o-t;

# HE_ADR_NULL_GDT_DESCRIPTOR

$ The GDT null descriptor is invalid for use in an address.

K error, o-t;

# HE_ADR_DESCTBL_LIMIT_EXCEEDED

$ Requested offset in the descriptor table exceeds the physical limits.

K error, o-t;

# HE_ADR_SEGMENT_INVALID

$ Specified segment is no longer valid or present in memory.

K error, o-t;

# HE_ADR_SEGMENT_USE_INVALID

$ Segment specified is not a valid type for requested use.

K error, o-t;

# HE_ADR_PAGE_UNAVAILABLE

$ The page table entry is not present during page translation.

K error, o-t;

# HE_ADR_PMODE_UNAVAILABLE

$ The pmode is uninitialized and prevents address translation.

K error, o-t;

# HE_ADR_SHELL_NOT_SUPPORTED

$ Shell command specified is not supported for the current processor.

K error, o-t;

# HE_SAVECS_MBAR_INVALID

$ MBAR is invalid; no other SIM registers will be processed.

K error, h-n;

# HE_POD_VERSION

$ Invalid pod version.

K error, h-n;

# HE_ICE_MODE_TIMEOUT

$ Timeout waiting for processor to enter ice mode.

K error, o-t;

# HE_ICE_CMD_TIMEOUT

$ Timeout waiting for processor to respond to commands.

K error, o-t;

# HE_ICE_BKPT_TIMEOUT

$ Timeout waiting for processor to process breakpoint.

K error, o-t;

# HE_INSUF_MAP_BLOCKS

$ Insufficient overlay memory.

K error, h-n;

# HE_TARGET_CONNECT

$ Emulation adapter is not connected properly to target.

K error, a-g;

# HE_TARGET_FLT

$ Cannot drive target processor FLT# pin to low State.

K error, a-g;

# HE_TARGET_VCC

$ Target power is off.

K error, o-t;

# HE_BONDOUT_FLT

$ Cannot drive emulation processor FLT# pin to high state.

K error, a-g;

# HE_NO_CLOCK

$ No clock input from target.

K error, h-n;

# HE_RESET_OCCURRED

$ Processor reset during operation.

K error, o-t;

# HE_HOLD_STUCK

$ Hold is continuously asserted.

K error, h-n;

# HE_READ_ABORT

$ Read operation aborted prior to completion.

K error, o-t;

# HE_TEST_ABORT

$ Test operation aborted prior to completion.

K error, o-t;

# HE_UNKNOWN_TEST_CMD

$ Unknown test command received by emulator.

K error, u-z;

# HE_EVTREC_TRACE

$ Unsupported event recognizer type.

K error, u-z;

# HE_LDR_INVALID_SERVER

$ Invalid loader server.

K error, h-n

# HE_LDR_INVALID_FILE_HANDLE

$ Invalid File Handle %s.

K error, h-n

# HE_LDR_CANNOT_GET_FILE_TIME

$ Cannot get file timestamp.

K error, a-g

# HE_LDR_INVALID_INPUT_TIMESTAMPS

$ Invalid input timestamps.

K error, h-n

# HE_LDR_IN_PROGRESS

$ Loader is loading.  Unable to perform the requested operation.

K error, h-n

# HE_LDR_LOADER_NOT_FOUND

$ Loader %s is not found.

K error, h-n

# HE_LDR_CANNOT_CREATE_FUNC_NAME

$ Cannot create function name %s for loader %s.

K error, a-g

# HE_LDR_CANNOT_GET_LIB_FUNC

$ Cannot get loader function %s.

K error, a-g

# HE_LDR_LOADER_IS_NOT_LOADED

$ Loader %s is not loaded.

K error, h-n

# HE_LDR_LOADFILE_NOT_FOUND

$ Loadfile %s is not found.

K error, h-n

# HE_LDR_BAD_LOADFILE

$ Bad loadfile.  Unknown load format signature.

K error, a-g

# HE_LDR_CANNOT_LOCATE_LOADERS_INI

$ Cannot locate the LOADERS.INI file.

K error, a-g

# HE_LDR_INVALID_NUM_LOADERS

$ Invalid number of loaders specified in the LOADERS.INI file.

K error, h-n

# HE_LDR_MISSING_LOADER_INFO

$ Missing loader initialization information.

K error, h-n

# HE_LDR_MISSING_LOADER_LIB_NAME

$ Missing loader library name.

K error, h-n

# HE_LDR_MISSING_LOADER_FORMAT_SIGNATURE_NUMBER

$ Missing loader format signature number.

K error, h-n

# HE_LDR_TOO_MANY_SIGNATURES

$ Too many load format signatures.

K error, o-t

# HE_LDR_MISSING_LOADER_NUM_SIG

$ Missing loader format signature.

K error, h-n

# HE_LDR_UNKNOWN_LOAD_FORMAT

$ Unknown load format.

K error, u-z

# HE_ASSERTION

$ Assertion error.

K error, a-g

# HE_CANNOT_OPEN_DIALOG

$ Cannot open the requested dialog box.

K error, a-g

# HE_PATH_NOT_AVAILABLE

$ Path to %s file is not available.

K error, o-t

# HE_CLIULIB_UNABLE_TO_READ_INIFILE

$ Unable to read %s file.

K error, u-z

# HE_CLIULIB_UNABLE_TO_WRITE_INIFILE

$ Unable to write INI file.

K error, u-z

# HE_OMF86_TYPES_TOO_LARGE

$ Exceeded the Maximum type (32767) allowed by the OMF86 spec.

K error, a-g

# HE_OMF86_BAD_TYPE_SIGNATURE

$ Invalid symbol type signature

K error, h-n

# HE_OMF86_LOGICAL_ADDRESS_NOT_HANDLED

$ Logical address is not handled by OMF86 loader.

K error, h-n

# HE_OMF86_MULTIPLE_SEGMENT

$ Multiple segments were found in the same module.

K error, h-n

# HE_OMF86_PROC_NOT_INTEL

$ OMF file encountered; not compatible with current XXXXX processor in use.

K error, o-t

# Errors_Opening_A_File

$ Error Opening A File

K file, error opening;error, opening a file;