# # $ K + Getting Started

The Up and Running manual contains hardware and software installation information.  Also, a PowerViews tutorial discusses product usage and features.

See also

Contacting Microtekcontactingmicrotek

# # $ K LoadSym option is needed for the input argument.

Contact Microtekcontactingmicrotek

# # $ K Cannot set break on a non-opcode location.

Cause:	You attempted to set a breakpoint on a comment line or other line without an opcode.

Solution:	Scroll the Source window until the next opcode location.

# # $ K Probe head configuration jumper is not installed.

Cause:	Installation not complete.

Solution:	See the Up and Running for installation instructions.

# # $ K OMF file was encountered; not compatible with current %s processor in use.

Contact Microtekcontactingmicrotek

# # $ K The target is holding RESET asserted; operation cannot be completed.

Contact Microtekcontactingmicrotek

# # $ K + Timeout waiting for emulation to start.

Contact Microtekcontactingmicrotek

# # $ K Physical address format is not supported for this operation.

Cause:  This operation only supports virtual or linear address format.

Solution: Use a virtual or linear address.  For example:

Virtual address = 40:100E

Linear address  = 1000L

# # $ K The beginning and ending addresses must be in the same segment.

The requested operation does not span segments, so the beginning and ending addresses must be in the same segment.

# # $ K Segment selector references a descriptor marked as not present.

Cause:	You tried to set one of the segment registers (CS, DS, ES, FS, GS, SS, TR or LDTR) to a selector that points to a descriptor which has its present bit cleared (0).

Solution:	Set the segment register to a selector that points to a descriptor that is present.

# # $ K CS selector does not reference a code segment descriptor.

Cause:	You tried to set the CS segment register to a selector that points to a descriptor that is not a code segment descriptor.  You chose either a data segment desciptor or system segment descriptor.

Solution:	Set the CS segment register to a selector that points to a code segment descriptor.

# # $ K SS selector does not reference a data segment descriptor.

Cause:	You tried to set the SS segment register to a selector that points to a descriptor that is not a data segment descriptor.  You chose either a code segment desciptor or system segment descriptor.

Solution:	Set the SS segment register to a selector that points to a data segment descriptor.

# # $ K SS selector references a data segment descriptor that is read only.

Cause:	You tried to set the SS segment register to a selector that points to a descriptor that is not read/write.  Stack segments are data segments which must be read/write.

Solution:	Set the SS segment register to a selector that points to a data segment descriptor that is read/write.

# # $ K Selector must reference a data or readable code descriptor.

Cause:	You tried to set the DS, ES, FS or GS segment register to a selector that points to a descriptor that is neither a data segment descriptor nor a readable code descriptor.

Solution:	Set the segment register to a selector that points to a data segment descriptor or a readable code descriptor.

# # $ K Selector RPL must be less than or equal to DPL.

Cause:	You tried to set the DS, ES, FS or GS segment register to a selector whose Requested Privilege Level (RPL) is greater than the priviledge level (DPL) of the descriptor that it points to. The RPL must be less than or equal to the DPL. The RPL is the least significant two bits of the selector.

Solution:	Either choose a different descriptor or enter a smaller value for the selector's RPL value.

# # $ K Cannot set the CS selector to the null selector

Cause:	You tried to set the CS segment register to the null selector (GDT entry 0).  The CS segment register cannot be set to the null selector.

Solution:	Set the CS segment register to a non-null selector.

# # $ K Cannot set SS selector to null selector

Cause:	You tried to set the SS segment register to the null selector (GDT entry 0).  The SS segment register cannot be set to the null selector.

Solution:	Set the SS segment register to a non-null selector.

# # $ K LDTR selector table indicator bit must be 0 (GDT).

Cause:	The LDTR register must be set to a selector value that points to a descriptor in the GDT.  The table indicator bit is bit 2 and must be set to 0 (the GDT table).  Bits 0 and 1 contain the RPL (requested privelge level).

Solution:	Set the LDTR register to a selector value with bit 2 set to 0.

# # $ K TR selector table indicator bit must be 0 (GDT).

Cause:	The TR register must be set to a selector value that points to a descriptor in the GDT.  The table indicator bit is bit 2 and must be set to 0 (the GDT table).  Bits 0 and 1 contain the RPL (requested privelge level).

Solution:	Set the TR register to a selector value with bit 2 set to 0.

# # $ K Cannot set LDTR selector value while in Real mode.

Cause:	You tried to set the LDTR register while the processor was in Real mode. The processor has to be in Protected mode to set this register.

Solution:	This register is only available for editing in Protected mode.

# # $ K Cannot set TR selector value while in REAL mode.

Cause:	You tried to set the TR register while the processor was in Real mode. The processor has to be in Protected mode to set this register.

Solution:	This register is only available for editing in Protected mode.

# # $ K TR selector does not reference a TSS descriptor.

Cause:	You tried to set the TR register to a selector that does not point to a TSS descriptor.

Solution:	Set the TR register to a selector that points to a TSS descriptor.

# # $ K TR selector references a busy TSS descriptor.

Cause:	You tried to set the TR register to a selector that points to a TSS descriptor marked as busy.  If a TSS is marked as busy, you cannot use that descriptor.

Solution:	Set the TR register to a selector that points to a different TSS descriptor.

# # $ K LDTR selector does not reference an LDT descriptor.

Cause:	You tried to set the LDT register to a selector that does not point to an LDT descriptor.

Solution:	Set the LDTR register to a selector that points to an LDT descriptor.

# gettingstarted

$ Getting Started

K answers;help;examples;initialization;PowerViews

+ POWERVIEWS:0020;TOOLBAR:0020;SAMPLEFILES:0010;SOURCELANGUAGEDEBUGGING:0050;TECHNICAL:0100

# HE_ARGS_NEED_SYM

$ LoadSym option is needed for the input argument.

K LoadSym option is needed for the input argument.

# HE_DASM_CANT_SET_BKPT

$ Cannot set break on a non-opcode location.

K error, a-g;

# HE_NO_JUMPER

$ Probe head configuration jumper is not installed.

K error, o-t

# HE_OMF86_LOADFILE_PROC_NOT_COMPATIBLE

$ OMF file was encountered; not compatible with current %s procssor in use.

K error, o-t;

# HE_RESET_STUCK

$ The target is holding RESET asserted; operation cannot be completed.

K error, o-t;

# HE_WAIT_EMON_TIMEOUT

$ Timeout waiting for emulation to start.

K error, o-t

+ GETSTART

# HE_ADDR_TYPE_NOT_SUPPORTED

$ Physical address format is not supported for this operation.

K error, o-t

# HE_SEGMENT_MISMATCH

$ The beginning and ending addresses must be in the same segment.

K error, o-t

# HE_SEGMENT_NOT_PRESENT

$ Segment selector references a descriptor marked as not present.

K error, o-t

# HE_DESC_NOT_CODE

$ CS selector does not reference a code segment descriptor.

K error, a-g

# HE_DESC_NOT_DATA

$ SS selector does not reference a data segment descriptor.

K error, o-t

# HE_DESC_NOT_WRITABLE

$ SS selector references a data segment descriptor that is read only.

K error, o-t

# HE_NOT_DATA_OR_CODE_READ

$ Selector must reference a data or readable code descriptor.

K error, o-t

# HE_RPL_GREATER_THEN_DPL

$ Selector RPL must be less than or equal to DPL.

K error, o-t

# HE_CS_HAS_NULL_SELECTOR

$ Cannot set CS selector to null selector

K error, a-g

# HE_SS_HAS_NULL_SELECTOR

$ Cannot set SS selector to null selector

K error, a-g

# HE_LDTR_DESC_NOT_IN_GDT

$ LDTR selector table indicator bit must be 0 (GDT).

K error, h-n

# HE_TR_DESC_NOT_IN_GDT

$ TR selector table indicator bit must be 0 (GDT).

K error, o-t

# HE_LDTR_REAL_MODE_ERR

$ Cannot set LDTR selector value while in REAL mode.

K error, a-g

# HE_TR_REAL_MODE_ERR

$ Cannot set TR selector value while in REAL mode.

K error, a-g

# HE_NOT_A_TSS

$ TR selector does not reference a TSS descriptor.

K error, o-t

# HE_TSS_BUSY

$ TR selector references a busy TSS descriptor.

K error, o-t

# HE_NOT_A_LDT

$ LDTR selector does not reference an LDT descriptor.

K error, o-t