# $ # + K Trace Window

The Trace Window displays a real-time collection of processor bus activity.

Screen Description

The screen displays several modestraceviewmodes: bus, clock, and instruction.  In bus and clock mode, the address, data, and processor status lines are displayed across the top with the frame numbers on the left side.  In instruction mode, the frame number, address and disassembled instruction are displayed.  If enabled from the View menu, Timestamp can be displayed relative to the beginning of trace or relative to the previous frame's timestamp.

Menu Selections

File

Save As...	Save some or all frames to a file in a readable format.

Exit	Close the Trace window.

Edit

Events...	View and edit eventseventterm with the Event Windowevent.

Search...	Position the trace buffer on an event.

Clear Trace	Clear the trace display.

View

Clock	Choose clock view modetraceviewmodes.

Bus	Choose bus view mode.

Instruction	Choose instruction view mode.

Linked Cursor	Activate linked cursorlinkedcursor with Source Windowsource.

Timestamp	Display timestamp.

BTM Cycles	Turn on or off BTM cyclesBTM_Cycles

Trace

Start	Start trace collection.

Stop	Stop trace collection.

Trace Control...	Configure trace parameterstracecontroldialog.

Timestamp

Relative To Frame	Timestamp is displayed relative to the beginning of trace.

Delta	Timestamp is displayed relative to the previous frame's timestamp.

Zero at Frame	Set the base frametimestampdialog for calculating Relative To Frame timestamp.  The zero frame is marked with dashes (--).

Setup...	Set the system clock frequencycrystalfrequencydialog.

Goto

Start Frame	View the beginning of the current buffer.

Trigger Frame	View the trigger frame, which is always clock frame 0.

End Frame	View the end of the current buffer.

Frame...	Moving viewing area to a specific frame.

Next Buffer	Display the next buffermultipletracebuffer.  The title bar displays the current buffer number

Previous Buffer	Display the previous buffer.

Buffer...	Display a specific buffer.

Windows	Move between windowsmovingbetweenwindows

Help	On-line helponlinehelp

Using the Trace Window

Trace and Trigger Fundamentalstraceandtriggerfundamentals

Trace View Modestraceviewmodes

Pre, Post, and Centered Trace Collectionprepostand

Multiple Trace Buffer Supportmultipletracebuffer

Searching the Tracesearchingthetrace

Saving Trace to a Filesavingtraceto

Halt When Last Trace Buffer Fullhaltwhenlast

Linked Cursorlinkedcursor

# $ # + K Trace View Modes

The Trace Window's View menu has three trace viewing options:

-	Clock cycleclockcycleterm displays every cycle of bus activity.

-	Bus cyclebuscycleterm displays aligned address, data, and processor status.

-	Instruction displays disassembled instructions.  The trace buffer is decoded into executed instructions and instructions which were pre-fetched but did not execute.  Only the executed instructions are displayed.

PowerViews looks for discontinuities in the trace due to a branch, call, etc.  In the rare case where there are no discontinuities in the entire buffer, instructions cannot be displayed.

The beginning of the buffer does not show any disassembled instructions until it finds a discontinuity.

In the three viewing modes, the frame numbers reflect the clock cycle numbering.  For bus and instruction view modes, gaps appear since multiple clock frames make up a single bus or instruction frame.

See also

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # + K Pre, Post, and Centered Trace Collection

The Trace Window'strace Options/Trace Control... and the Trigger Window'strigger Options/Trace Control... menu options set the trace collection mode:

-	Pre collects clock cyclesclockcycleterm up to a triggertriggerterm.  Use this mode to view what has occurred before a specified eventeventterm.  The trigger occurs near the end of the buffer.

-	Post collects clock cycles after a trigger.  The trigger occurs near the beginning of the buffer.

-	Center collects clock cycles before and after a trigger.  The trigger occurs near the center of the buffer.

The trigger point is frame 0.  Cycles before the trigger are negative and after the trigger are positive.  For example, with 128k trace frames and 256 buffers, there are 512 frames in each buffer (128k /256=512).  For each view mode, the numbering would be:

-	Pre from -511 to 0.

-	Post from 0 to +511.

-	Centered from -255 to +256.

See also

Multiple Trace Buffer Supportmultipletracebuffer

Trace Control Dialog Boxtracecontroldialog

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # + K Multiple Trace Buffer Support

The Trace Window'strace Options/Trace Control... and the Trigger Window'strigger Options/Trace Control... menu options set the number of trace buffers.  There can be 1, 2, 4, 8, 16, 32, 64, 128, or 256 buffers.

The greater the number of buffers, the fewer the number of clock cyclesclockcycleterm per buffer.  With 128k trace buffer, 1 buffer contains 128k frames.  With 256 buffers, only 512 frames are in each buffer.   The Trace Control dialog boxtracecontroldialog shows the size of each buffer in parentheses.

Multiple buffers are useful for:

-	Collecting multiple iterations of the same eventeventterm.  Comparing the buffers reveals differences.  Use the Trace Window's File/Save As... to save a buffer to a file for comparison or use the Trace Window to view different buffers.

-	Collect trace around different events.  Use the Trigger Windowtrigger to sequence to a buffer.  With different actions selectedselectingtriggeractions, different collection criteria is applied.

See also

Trace Control Dialog Boxtracecontroldialog

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # + K Searching the Trace

The Trace Window's Edit/Search menu option searches the current trace buffer.

The eventeventterm search is done at the clock cycleclockcycleterm level. In View/Clock mode, the exact frame is displayed.  In View/Bus and View/Instruction, the matched frame may not be on a bus or instruction boundary.  In this case, the next boundary is displayed.

See also

Search Dialog Boxsearchdialog

Creating Eventscreatingevents

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # + K Saving Trace to a File

The Trace Window'strace File/Save As... option creates a text file with bus cyclebuscycleterm, clock cycleclockcycleterm, or disassembled instruction trace data.  This file can be printed.  A file comparison program can pinpoint differences between execution snapshots.

See also

Trace Save As Dialog Boxtracesaveasdialog

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# # $ K + Halt When Last Trace Buffer Full

This feature is supported when more than one trace buffer is defined.  When the last trace buffer is filled, trace is collected in the first trace buffer, then emulationemulationterm halts.  The contents of the first trace buffer are overwritten.  To prevent the loss of this buffer, do one of the following:

-	Skip the first buffer.  Trigger on a don't-care eventeventterm to immediately sequence to the second buffer.  A don't-care event is the default event.

-	Do not use this feature.  Instead, use a counterusingcountersand to break emulation when the last buffer event occurs.  Select inc0 (or inc1).  Set the counter terminal count to be the number of trace buffers.  Select next.

See also

Multiple Trace Buffer Supportmultipletracebuffer

Pre, Post, and Centered Trace Collectionprepostand

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # + K Linked Cursor

The Trace Window'strace View/Linked Cursor links the Trace and Source Windowssource.  This feature enables you to correlate trace cycles back to the original source statement.

The linked cursor appears as a yellow highlight in both windows.  As the Trace Window is scrolled, the Source Window uses the address information to highlight the corresponding source line.

Scrolling the Trace Window repositions the Source Window but scrolling the Source Window has no effect on the Trace Window.

See also

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # K Trace Save As Dialog Box

Enter the directory and the name of the file to save the trace information.  The file format is readable and printable.  A second dialog box appears.  The fields are:

-	Save Format saves bus cyclesbuscycleterm, clock cyclesclockcycleterm, or disassembled instructions.

-	Start is the beginning buffer and the frame within the buffer to save.

-	End is the last buffer and frame to save.

See also

Saving Trace to a Filesavingtraceto

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # K Trace Search Dialog Box

The trace is searched by matching an eventeventterm.  The Trace Window's display moves to the matched frame or a no match message box appears.

Click on the Search Event down arrow to list the defined events.  If none are defined, the Event Windowevent is displayed to create one.

The Start Frame defaults to the first frame in the buffer or the last match frame +1.  The search always goes until a match is found or the end of the buffer is reached.

See also

Searching the Tracesearchingthetrace

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # K Frame Number Dialog Box

Specify a frame number (in the given range of valid frames) to position the top of the Trace window to that frame.  A frame represents one processor clock cycle.

The frame numbering depends on the pre, post, or centered collection modeprepostand.

Note

-	A negative number represents frames prior to the trigger

-	Frame 0 is the trigger point

-	Positive numbers represent frames captured after the trigger.

See also

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # K Buffer Dialog Box

Enter the buffer number or use the arrows to scroll.  Buffers are numbered from 0.  The number of buffers is set in the Trace Window'strace Options/Trace Control... or the Trigger Window'strigger Options/Trace Control... menu options.

See also

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # K Zero Timestamp At Frame Dialog Box

Set the base frame for timestamp.  The fields are:

-	Frame: the frame for the timestamp.

See also

Saving Trace to a Filesavingtraceto

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# $ # K System Clock Dialog Box

Set the processor system frequency.  The fields are:

-	Frequency: the processor system frequency.

A number between 0.01 Hz and 40 MHz is legal.  The number can be entered in floating point format, and can be entered in Hz, KHz, or MHz.

See also

Trace and Trigger Fundamentalstraceandtriggerfundamentals

# # $ K + BTM Cycles

A BTM cycle is a special bus cycle executed by the bondout processor when execution is discontinuous (e.g., at a jump, call, interrupt, return).

A check mark indicates BTM cycles do occur.  Their occurrence impacts real-time execution slightly.  You can display trace in instruction mode.

No check mark indicates BTM cycles do not occur, emulation runs in real-time, and you cannot display trace as instructions.

#### NOTE:  Enabling or disabling BTM Cycles clears the trace buffer.

$ Trace Window

# trace

+ powerviews:0110

K trace;window

$ Trace View Modes

# traceviewmodes

+ usingthetrace:0010

K trace

$ Pre, Post, and Centered Trace Collection

# prepostand

+ usingthetrace:0020

K trace

$ Multiple Trace Buffer Support

# multipletracebuffer

+ usingthetrace:0030

K trace

$ Searching the Trace

# searchingthetrace

+ usingthetrace:0040

K trace

$ Saving Trace to a File

# savingtraceto

+ usingthetrace:0050

K trace;files

# haltwhenlast

$ Halt When Last Trace Buffer Full

K trace;halt

+ USINGTHETRACE:0060

$ Linked Cursor

# linkedcursor

+ usingthetrace:0070

K trace;source

$ Trace Save As Dialog Box

# tracesaveasdialog

K trace;dialog box;trace save;save

$ Trace Search Dialog Box

# searchdialog

K trace;dialog box;search;

$ Frame Number Dialog Box

# framedialog

K trace;dialog box;frame;goto frame

$ Buffer Dialog Box

# bufferdialog

K trace;dialog box;buffer;goto buffer

$ Zero Timestamp At Frame Dialog Box

# timestampdialog

K trace;dialog box;timestamp;

$ System Clock Dialog Box

# crystalfrequencydialog

K trace;system clock;clock;timestamp;crystal frequency

# BTM_Cycles

$ BTM Cycles

K BTM cycles;trace;

+ TRACE