# Software Performance Analysis (SPA) Window

## Introduction

The Software Performance Analysis can significantly assis in improving performance development of complicated software that requires minium CPU execution time for each program module. Choose SPA window from the Windows menu in the ToolBar.

This chapter describes how to do the following:

·	 Add, delete and edit ranges.

·	Select ranges, execute module analysis and read the result.

·	Select a range, execute time analysis and read the result.

·	Set ranges, execute code coverage and read the result.

## Sample Module Anaylsis Window

The following is a sample module analysis.

### Start Analysis

1.  Select  ranges you want to analyze in combo list of the module   analysis window. It can analyze up to four ranges.

2.  Set an execution breakpoint if you need.

3.  Press the Start button.

### Analysis Results

Time:  indicates the overall duration of execution for each specified module during emulation.

Count:  indicates the number of times of execution for each specified module during emulation.

Time Percentage(Bar graph) : displays the  percentage of  the spended time for each specified module during emulation.

Count Percentage: displays the  percentage of  the number of times for each specified module during emulation.

Total Time:  indicates the overall lenght of execution time for the emulation processor.

Total Count:  indicates total counts of execution for the emulation processor.

< figure >

## Sample Time Anaylsis Window

The following is a sample time analysis.

### Start Analysis

1.  Select a range you want to analyze in combo list of the time analysis window.

2. Set an execution breakpoint if you need.

3.  Press the Start button.

### Analysis Results

Count:  indicates the number of times of execution for each specified module during emulation.

Count Percentage: displays the  percentage of  the number of times for each specified module during emulation.

Average: indicates the average duration of execution for the emulation processor.

Longest: indicates the longest duration of execution for the emulation processor.

Shortest: indicates the shortest duration of execution for the emulation processor.

<figure>

## Sample Code Coverage Window

The following is a sample code coverage.

### Start Analysis

1.  Select  ranges you want to analyze in combo list of the module   analysis window. It can analyze up to four ranges.

2.  Set an execution breakpoint if you need.

2.  Press the Start button.

### Analysis Results

When the execution breakpoint matched or the stop button pressed down, you can double click a plus mark in the front of function name on the left side window. On the right side window, the source/assembly code will be display and the executed lines will be highlighted.

<figure>

## Ranges Dialog

Ranges dialog can add, delete, edit, save and restore ranges.

<figure>

### Add a Range

1.  Press  Ranges button to open the ranges dialog.

2.  Press Add button to open an add-range dialog.

3.  Enter range name, start and end address of the range.

<figure>

### Edit a Range

1.  Press  Ranges button to open the ranges dialog.

2.  Select a range you want to edit in list box of the range dialog.

3.  Press Edit button to open an edit-range dialog.

4.  Modify start or end address of the range.

### Delete a Range

1.  Press  Ranges button to open the ranges dialog.

2.  Select a range you want to delete in list box of the range dialog.

3.  Press Delete button to delete the range.

### Save  Ranges

1.  Press  Ranges button to open the  ranges dialog.

2.  Press Save button to save all ranges to a file.

### Restore Ranges

1.  Press  Ranges button to open the ranges dialog.

2.  Press Restore button to restore all ranges in a file.

## Set a BreakPoint

The following is a sample to set a breakpoint.

### Set a breakpoint

1.  Press  BreakPoint button to open a breakpoint dialog.

2.  Select #module1 in combo list of module name.

3.  Select #func1 in combo list of function name.

4.  Set the address of breakpoint directly if you need.

< figure >

# Software Performance Analysis (SPA) Window Reference

## Introduction

This chapter describes the menus and commands in the SPA window.

## SPA Window Menus

| Menu | Use To: |
| --- | --- |
| File | Exit the SPA window. |
| SPA | Toggle module analysis, time analysis and code coverage. |

### File Menu

Exit the SPA window from the File menu.

#### Exit

Choose Exit to close the SPA window.

### SPA Menu

Switch among module analysis, time analysis and code coverage.

#### Module Analysis

To unveil the overall duration of activity for a group (maximum of 4) of selected program modules. Calculatioin begins each time execution enters the start address of a module and terminates only when the module end address is reached.

#### Time Analysis

To unveil the activity distribution for a specified program module range. The frequency of execution within different time frames is displayed.  It also includes the shortest duration, the longest duration and the average duration.

#### Code Coverage

To unveil the code areas which have been executed. It highlighted the executed code in source or assembly level.

file that you want to load in the File dialog box which pops-up.  The dialog box saves the last path name specified.

2.	Choose the Options button in the dialog box to open the Load Options dialog box. The Load Options dialog box includes the following check boxes.

·	Load code.  For example, you may want to load only code if symbols are already loaded.

·	Load symbols. For example, you may want to load only symbols if you are debugging ROM code.  There are three subheadings under Load Symbols:

Toggle on demand loading.  By default, MP/SLD loads symbols on demand.  On demand loading means that

MP/SLD defers loading local symbol and line-number information for an individual module until that module is either displayed in the Source window or a breakpoint is set in it.  On demand loading can save time on the initial load, especially if you are debugging only a few of your modules.

Toggle on the Demangle C++ Names to demangle C++ names.

·	Toggle the loading-status report by choosing Report Status.

·	Toggle Report Warnings.  If a load file contains irregular information, you may choose to suppress warnings during loading (no check-mark next to Report Warnings).

-or-

1.	Choose the Load button on the MP/SLD toolbar.  (Press F5 to activate the toolbar.)

CAUTION

Files can be loaded while the emulator is running.  Be sure to load files at a location other than the location of the program you are running.  If you load a file at an already used location, the emulator can behave unpredictably.

## Debugging from the Source Window

You can view source, set or clear breakpoints, start or stop emulation, step through source, or inspect variables.

### Viewing Source

You can view modules from the Source window.  If the Source file has been modified since the absolute object file was generated, a warning message is displayed and an asterisk marks the source file name in the Source window title.

Split Box

Breakpoint

Line Numbers

CS:IP

To view the source file

| The CS:IP is marked by >>. | 1. Load the executable file. The Source window initially displays code starting at the current CS:IP (called the PC). If source is not available, disassembly is displayed. |
| --- | --- |

NOTE

When you open the Source window after loading code, assembly-level startup code is displayed.  You or a compiler vendor may write initialization code which sets up the processor environment prior to the start of main().  Many programs start in assembly code and not in main().

2.	To view a module (a C or assembly source file), choose Browse Modules from the File menu in the Source window.  All modules are listed in the Browse Modules dialog box.  You can view Source Only for modules with full symbolic information, including line numbers.  Other modules can be viewed as disassembly.

3.	Toggle between source only and mixed source and disassembly displays from the View menu.

4.	From the MP/SLD Toolbar, you can also toggle Symbolic Disassembly from the Configure menu.  If toggled on, code symbols appear in the disassembly display.

### Viewing Source in Two Panes

Sometimes you need to view different parts of the source code at the same time.  Instead of scrolling back and forth in the source code, you can split the Source window into two panes.  One pane is active; click in the inactive pane to make it active.

Two panes allows you to:

·	Scroll each one independently.

·	Browse two different modules (one per pane).

·	View or set breakpoints in different modules.

#### Splitting or Unsplitting the Window

|  | Before the Source window is split, the split box is at the top of the vertical scroll bar. After a window is split, the split box is located at the split position. When the mouse is pointing to the split box, MP/SLD displays a split-box cursor. |
| --- | --- |

1.	Select the split box on the top of the vertical scroll bar.

2.	When the mouse pointer changes shape, drag the split box to the position you want.

#### Moving Between Panes

·	Click in the inactive pane to make it active.

### Setting Breakpoints in Source

You can set breakpoints from the Source or Breakpoint windows.  See also "Setting a Breakpoint."

|  | You can set a breakpoint in the Source window with a mouse. If you point the mouse to the left of a source line, MP/SLD displays a breakpoint cursor. |
| --- | --- |

To set a breakpoint:

1.	Move the mouse pointer to the left of the source line where you want to set a breakpoint.

2.	When the mouse pointer changes shape, click on the line to set a breakpoint.  Clicking on the primary (usually the left) button sets a permanent breakpoint.  Clicking on the secondary (usually the right) button sets a temporary breakpoint.

#### Setting a Breakpoint on a Non-executable Statement

If you try to set a breakpoint on a non-executable statement, a breakpoint is set on the first subsequent executable statement.

#### Software Breakpoints

The MICEpack emulator provides:

·	256 software breakpoints.

### Starting or Stopping Emulation in the Source Window

You can emulate, halt emulation, step and step continuously in the Source window.

#### Starting Emulation Using the Source Buttons

·	Start emulation by choosing the Go button.

-or-

·	Start emulation by choosing the Until/Into Call or the Until/Into Return button.  Toggle Until/Into with the Set Go Buttons command from the Options menu.

-or-

·	Start emulation by pressing the F9 key.

#### Stopping Emulation Using the Source Buttons

·	Stop emulation by choosing the Halt button.

-or-

·	Stop emulation by pressing the F2 key.

#### Stepping Through Source

1.	Step by Source Line or Source Statement.  Set the level of step granularity by setting the Step Execution Granularity command from the Options menu.

2.	Set how many steps (1-100) are executed per Step command.  Set the number of steps with the Step Count command from the Options menu.  The default number of steps is one.

3.	Step into calls or step over calls.  From the Run menu, choose Step Into or Step Over.

NOTE:

If you step into a function call, then the display steps into the source for that call and executes the first executable line in the procedure.

If you step over a function call, then the display treats that call as a single statement.

If the Source Only is toggled from the View menu, and a function has no source associated with it, then a step into works the same as a step over.  To step into a function with no associated source, choose View Mixed Source and Asm.

If stepping is slow, close windows to improve performance.  Each window must be updated after each step.  Performance can be affected by these updates.

#### Stepping Continuously Through Source

1.	Set step options as described in "Stepping Through Source" above.

2.	From the Run menu, choose Step Into Continuously or Step Over Continuously.

NOTE

How fast MP/SLD performs continuous stepping depends on the number of windows it must update after each step.

### Linking the Trace and Source Windows

You can link the Source and Trace window displays.  If the windows have been linked, you can scroll through the Trace window and view the corresponding data in the Source window.  To link the Source window to the Trace window, do the following:

·	For best results, choose Instruction from the Trace window View menu.

·	Choose the Linked Cursor command from the View menu in the Execution Trace window.

NOTE

The Linked Cursor feature enables you to display the history of executed source code lines.

## Using the Breakpoint Window

### Opening the Breakpoint Window

Set, view, and clear breakpoints from the Breakpoint window.

To open the Breakpoint window:

·	From the Windows menu, choose Breakpoint.

-or-

·	From the Source window, choose Show All from the Breakpoints menu.

The Breakpoint window display includes the state (enable, disable), type (permanent, temporary), module, function, line number, and column number of each breakpoint.

### Setting a Breakpoint in the Breakpoint Window

1.	From the Breakpoints menu, choose Set Breakpoint to display a dialog box.

2.	Set the breakpoint location:

a.	Select a module from a module list and a function for the module from the function list.

-or-

b.	Type an address in the Breakpoint at edit box.

NOTE

1.	You can enter a numeric or symbolic address.  Numeric addresses are hexadecimal addresses such as 1000.  An example of a symbolic address is #codelabel.

2.	Software breakpoint address must be set at the first address of the instruction.

3.	You may set a software breakpoint at ROM memory where it uses hardware breakpoint to implement breakpoints.

3.	Select the breakpoint type as permanent or temporary.

4.	Enable or disable the breakpoint.

5.	Choose Set to set the breakpoint.

### Specifying the Breakpoint State and Type

| When the breakpoint is reached, emulation halts before the instruction at the breakpoint address is executed. | Breakpoints can be enabled, disabled, or cleared. An enabled breakpoint is defined and active; emulation breaks when the breakpoint is reached. A disabled breakpoint is defined but inactive; emulation does not break when the breakpoint is reached. |
| --- | --- |

The two states, enabled and disabled, allow you to turn breakpoints on or off.  For example, enable a permanent breakpoint and emulate until that breakpoint is reached.  Then, disable the breakpoint and continue emulation.  Later, you can re-enable the breakpoint and continue emulation.

You define breakpoints as permanent or temporary.  A permanent breakpoint remains defined even after the breakpoint is reached.  A temporary breakpoint is cleared after the first time the breakpoint is reached (and emulation stops at that address).

NOTE

Symbols must be loaded before you can define breakpoints on line numbers or functions.

NOTE

You can enter a numeric or symbolic address.  Numeric addresses are hexadecimal addresses such as 1000.  An example of a symbolic address is #codelabel.

3.	Select the breakpoint type as permanent or temporary.

4.	Enable or disable the breakpoint.

5.	Choose Set to set the breakpoint.

#### Clear

To remove a breakpoint, do the following:

1.	Select the breakpoint from the breakpoint list.

2.	Choose Clear (or, choose the Clear button).

#### Enable

If you have set a permanent or a temporary breakpoint, enable it by doing the following:

1.	Select the breakpoint from the breakpoint list.

2.	Choose Enable (or choose the Enable button).

#### Disable

Temporarily disable a breakpoint to execute freely through the code where the breakpoint is located.  Reuse the breakpoint by enabling it.  Disable a breakpoint by doing the following:

1.	Select the breakpoint from the breakpoint list.

2.	Choose Disable (or choose the Disable button).

#### Clear All

Clear all breakpoints in the breakpoint list by choosing Clear All.

#### Enable All

Choose Enable All to enable all breakpoints in the breakpoint list.

#### Disable All

Choose Disable All to disable (without removing) all breakpoints in the breakpoint list.

#### Go to Source

Choose Go to Source to move the cursor to the specified breakpoint in the Source window.

## Breakpoint Window Buttons

Choose the Breakpoint window buttons to do the following:

·	Set a breakpoint.

·	Clear a breakpoint.

·	Go To Source of a breakpoint.

·	Enable a breakpoint.

·	Disable a breakpoint.

·	Enable all breakpoints.

·	Disable all breakpoints.

### Set

Set a breakpoint by doing the following steps:

1.	Choose the Set button to display a dialog box.

2.	Set the breakpoint location:

a.	Select a module from a module list and a function for the module from the function list.

-or-

b.	Type an address in the Breakpoint at edit box.

NOTE

You can enter a numeric or symbolic address.  Numeric addresses are hexadecimal addresses such as 1000.  An example of a symbolic address is #codelabel.

3.	Select the breakpoint type as permanent or temporary.

4.	Enable or disable the breakpoint.

5.	Choose Set to set the breakpoint.

### Clear

Select a breakpoint, then choose the Clear button to delete it.

### Go To Source

Select a breakpoint, then choose the Go To Source button to display the source or assembly at that address.

### Enable

Select a breakpoint, then choose the Enable button to enable it.

### Disable

Select a breakpoint, then choose the Disable button to disable it.

### Enable All

Choose the Enable All button to enable all previously defined breakpoints.

### Disable All

Choose the Disable All button to disable all previously defined breakpoints.

# Source Window Reference

## Introduction

This chapter describes the menus and commands in the Source window.

## Source Window Menus

| Menu | Use To: |
| --- | --- |
| File | Load code, view load file information, browse modules, browse from one module to another, or exit the Source window. |
| Edit | Navigate through source. |
| View | Toggle the source display between source only and mixed source and assembly, toggle the display of line numbers. |
| Run | Start and stop emulation, step, or continuously step. Also, reset or reset and start emulation. |
| Breakpoint | Define permanent or temporary breakpoints; clear, enable or disable breakpoints. |
| Options | Add a source path, toggle go until call/return and go into call/return; set step granularity and count, set the source browser history depth, or set the source line delimiter. |
| Windows | Open another window. For more information, see the chapter "Getting Started." |
| Help | Get help on commands. For more information, see the chapter "Getting Started." |

### File Menu

From the File menu, you can load files, browse modules, or page through multiple files.  If you have loaded a file recently, you may be able to reload it by choosing its name from the File menu.  MP/SLD lists the four files you most recently loaded at the bottom of the File menu.  This is the fastest way to load a file and is especially handy when you work on a small group of files over a period of time.

#### Load Code

Choose Load Code from the File menu to load a file into overlay memory or target memory.

#### Load Information

Choose Load Information from the File menu to display loadfile name, information about the loadfile, and the current PC.

#### Browse Modules

| A module is a source file together with its symbol information. | Choose Browse Modules from the File menu. The Browse Modules dialog box lists the loadfile modules. |
| --- | --- |

#### Previous Browsed Module

You may open up many modules during a debugging session, one at a time.  MP/SLD remembers the sequence of modules you have opened in a debugging session and the functions you have browsed by double-clicking the function name in source.  Choose Previous Browsed Module from the File menu to scroll back to the previous module.

#### Next Browsed Module

You may open up many modules while debugging.  MP/SLD remembers the sequence of modules you have opened in a debugging session and the functions you have browsed by double-clicking the function name in source.  Choose Next Browsed Module from the File menu to scroll forward to the next module.

#### Exit

Choose Exit to close the Source window.

### 1,2,3,4

List of the last four files you loaded.  Reload a file by choosing it from the File menu.

### Edit Menu

Scroll the Source window from the Edit menu.  You can search for a string, search for the next occurrence of the string, scroll to a line in source, or scroll to the line specified by CS:IP.

#### Search

If you are looking for a specific name or other string in the current module, go to the top of the module (press the Home key) and choose Search (the search is case sensitive).  If you are looking for repeated instances of the same string, choose Search Next.  The search is one-way (it does not wrap to the beginning of the file) and starts from the cursor.

#### Search Next

If you want to find the next occurrence of a string, choose Search Next.  The search is one-way (it does not wrap to the beginning of the file) and starts from the cursor.

#### Go To Line

Scroll directly to a line in the current module.  Choose Go To Line to move the cursor to a specified line.  If the specified line is greater than the last line, the Source window scrolls to the end of the displayed module.

#### Go To Address

Scroll directly to any address.  Choose Go To Address to bring up a dialog box. If there is no source at that address, the Source window shows disassembled code beginning at that address.

#### Go To PC

Choose Go To PC (where PC is CS:IP) to move the cursor to the line pointed to by CS:IP.

### View Menu

Sometimes it is useful to view the source display together with the assembly language display.  As you debug, MP/SLD enables you to toggle the display between source only and mixed source and assembly language.

You can toggle the display of line numbers.

#### Source Only

Choose Source Only to display source code only.

#### Mixed Source and Asm

Sometimes it is useful to view the program source intermixed with the assembly language compiled for each source line.  Choose Mixed Source and Asm to display both source and assembly.

#### Line Numbers

To display line numbers (of a source file) in the Source window, toggle on Line Numbers.

### Run Menu

Choose commands from the Run menu to start and stop emulation or to step.  Set the size and number of steps from the Options menu.  Toggle Into/until (for the Go and Return commands) from the Options menu.

The Run menu also has the Reset command and the Reset and Go command.

#### Go (F9)

Choose Go to start emulation.  Or, press the F9 key to start emulation.

#### Halt (F2)

Choose Halt to halt emulation.  Or, press the F2 key to halt emulation.

#### Step Into (F7)

Step into or over function calls in the Source window.  Choose Step Into to step into a function call.  Set the size of each step with Source Step Granularity from the Options menu.  Set the number of steps with Step Count from the Options menu.  Or, press the F7 key to step into a function call.

NOTE

If the Source Only is toggled from the View menu, and a function has no source associated with it, then a Step Into functions the same as a Step Over.  To step into a function with no associated source, choose View Mixed Source and Asm.

#### Step Over (F8)

Step into or over function calls.  Choose Step Over to step over a function if one exists in the next line or statement.   Set the size of each step with Source Step Granularity from the Options menu.  Set the number of steps with Step Count from the Options menu.  Or, press the F8 key to Step Over a function.

NOTE

Step over causes the program to execute until it reaches the statement or line after the call.  However, if you set a breakpoint inside of a function in that execution path, then emulation stops at the breakpoint.

#### Go Until Call/Return

Break emulation when a line or statement containing a call instruction is reached.  Choose Go Until Call to execute from the current CS:IP until the beginning of a statement or line (depending on the granularity) containing a function-call or return instruction.

Break emulation before a return instruction is executed.  Choose Go Until Return to execute from the current CS:IP until the beginning of a statement or line (depending on the granularity) containing a return instruction.

NOTE

Set the level of step granularity by toggling the Step Execution Granularity command from the Options menu.

#### Go Into Call/Return

Choose Go Into Call to execute from the current CS:IP until the first executable statement inside a function call or return instruction.

Choose Go Into Return to execute from the current CS:IP until the first executable statement or line (in the calling function) after a return instruction.

#### Go To Cursor

Execute up to a statement in the Source window by doing the following:

1.	Select a statement by clicking on it.

2.	Choose Go To Cursor from the Run menu (or the button bar) to execute up to the selected (highlighted) statement.

#### Go From Cursor

Start emulation at any line in the Source file by doing the following:

1.	Select a statement by clicking on it.

2.	Choose Go From Cursor from the Run menu to start emulation from the selected statement.

#### Step Into Continuously

Choose Step Into Continuously to continuously step by statement or line.  If a function is encountered, the function is stepped into.  To stop stepping, press the Esc or F2 key, or choose Halt.

NOTE

To set the size of each step, choose Source Step Granularity from the Options menu.  To set the number of steps, choose Step Count from the Options menu.

#### Step Over Continuously

Choose Step Over Continuously to continuously step by statement or line.  If a function is encountered, the function is stepped over.  To stop stepping, press the Esc or F2 key, or choose Halt.

#### Reset

To reset the CPU (user target), choose Reset to assert the reset pin of the target processor.  This command causes the CPU to reset the CS:IP as FFFF:0.  The reset pin is then released.

### Breakpoints Menu

View, set, enable, disable, or clear a breakpoint using the Breakpoints menu.

#### Set Permanent Breakpoint

Choose Set Permanent Breakpoint to set a permanent breakpoint at a selected line or at the cursor.

#### Set Temporary Breakpoint

Choose Set Temporary Breakpoint to set a temporary breakpoint at a selected line or at the cursor.

#### Set Breakpoint

Set a breakpoint by doing the following steps:

1.	Choose Set Breakpoint to open the Set Breakpoint dialog box.

2.	Set the breakpoint location:

a.	Select a module from a module list and a function for the module from the function list.

-or-

b.	Type an address in the Breakpoint at edit box.

NOTE

1.	You can enter a numeric or symbolic address.  Numeric addresses are hexadecimal addresses such as 1000.  An example of a symbolic address is #codelabel.

2.	Software breakpoint address must be set at the first address of instruction.

3.	You may set a software breakpoint at ROM memory where it uses hardware breakpoint to implement breakpoints.

3.	Select the breakpoint type as permanent or temporary.

4.	Enable or disable the breakpoint.

5.	Choose Set to set the breakpoint.

6.	Choose Close to exit.

#### Clear

Remove a breakpoint by doing the following steps:

1.	Click on the line containing the breakpoint.

2.	Choose Clear from the Breakpoints menu.

#### Enable

Temporarily disable a breakpoint to execute freely through the code where the breakpoint is located.  You can then reuse the breakpoint by enabling it.  Enable a breakpoint by doing the following steps:

1.	Select the line containing the breakpoint

2.	Choose Enable from the Breakpoints menu.  The highlight changes color to red to indicate that the breakpoint is enabled.

#### Disable

Disable a breakpoint by doing the following steps:

1.	Select the line containing the breakpoint

2.	Choose Disable from the Breakpoints menu.  The highlight changes color to grey to indicate that the breakpoint is disabled.

#### Clear All

To remove all breakpoints from the breakpoint list, choose Clear All from the Breakpoints menu.

#### Enable All

Choose Enable All to enable all breakpoints in the breakpoint list.

#### Disable All

Choose Disable All from the Breakpoints menu to disable, but not remove, all breakpoints in the breakpoint list.  After testing your code by running freely, you can re-enable the breakpoints and continue debugging.

#### Show All

Choose Show All to display all breakpoints by opening the Breakpoint window.

### Options Menu

Add a new source path, toggle go until call/return and go into call/return; set step granularity and count, set the source browser history depth and set the source line delimiter using the Options menu.

#### Source Path

Sometimes you compile a module to a directory other than its current directory.  Choose Source Path from the Options menu to open a dialog box.  Use the dialog box to add, delete, or edit a source path.  Choose the Close button to have the changes accepted.  Choose the Cancel button to cancel your changes.

NOTE

Up to 50 paths can be defined.  The paths are searched in the order they appear (top to bottom).  The paths are saved in MP186.INI and are reloaded when you re-invoke MP/SLD.

#### Tab Width

If your source file contains Tab characters, you can set the Tab Width smaller to display more source in the Source window.  To set the Tab Width from 1 to 32 spaces, choose Tab Width from the Options menu.  The Tab Width is the number of spaces that replace a tab character.  The Tab Width setting defaults to 8 spaces.

#### Source Step Granularity

Choose Source Step Granularity from the Options menu to set the granularity for the Step commands as Line (default) or Statement.

·	Line:  Choose Line to step by source lines.  Line is the default.

·	Statement:  Choose Statement to step by statements.  For example, a line of C language source may contain more than one statement (separated by semicolons).  You can step through a single source line by statements.

#### Step Count

Choose Step Count from the Options menu to set how many steps (1-100) are executed per Step command.

#### Browser History Depth

You may open up many modules while debugging.  MP/SLD remembers the sequence of modules you have opened in a debugging session and the functions you have browsed by double-clicking the function name in source.  Choose Browser History Depth from the Options menu to set the number of modules that can be recalled.  Choose Previous- and Next Browsed Module to cycle through these modules.

#### Source Line Delimiter

Choose Source Line Delimiter from the Options menu to set the ASCII character(s) used by the compiler to delimit a source line.  The default source-line delimiter is Carriage Return/Linefeed.

·	Choose Carriage Return/Linefeed to recognize the end-of-line character as carriage return and linefeed.  This is the default.  If you display a UNIX source file with Carriage Return/Linefeed toggled on, only one line of text is displayed in the Source window.

·	Choose Linefeed Only display a UNIX source file in the Source window.  If you display a DOS source file with Linefeed Only toggled on, a black dot is displayed at the end of each line.

#### Set Go Buttons

Choose Set Go Buttons from the Options menu to toggle the Until Call/Return and Into Call/Return command buttons.  The default setting is Into Call/Return.

·	Choose Until Call/Return to stop emulation on a statement or line containing a call or return instruction.

·	Choose Into Call/Return to stop emulation after executing a call or return instruction.

## Function and Variable Popup Menus

| Popup Menu | Access By: | Use To: |
| --- | --- | --- |
| Function | Double-click function name. | Display the function source, or load address of the function. |
| Variable | Double-click variable name. | Inspect or edit a variable; or set a breakpoint on the highlighted line. |

### Function Popup Menu Commands

| Double-click a function name to open the Function menu. | Go to the function source or display the function load address from the Function popup menu. |
| --- | --- |

#### Go To Source

Choose Go To Source to display the function source code (if available) and position the cursor at the beginning of that source.

#### Show Load Address

Choose Show Load Address to display the function's starting address.

#### Set Perm. Breakpoint

Choose Set Perm. Breakpoint to set a permanent breakpoint at the highlighted line.

#### Set Temp. Breakpoint

Choose Set Temp. Breakpoint to set a temporary breakpoint at the highlighted line.

#### Clear Breakpoint

Choose Clear Breakpoint to clear the breakpoint on the highlighted line.

### Variable Popup Menu Commands

| Double-click a variable name to open the Variable menu. | Inspect a variable or set a breakpoint on the highlighted line from the Variable popup menu in the Source window. |
| --- | --- |

#### Inspect Variable

Choose Inspect Variable to add the variable to the Variable window, which displays the value and lets you edit it.

#### Set Perm. Breakpoint

Choose Set Perm. Breakpoint to set a permanent breakpoint on the highlighted line.

#### Set Temp. Breakpoint

Choose Set Temp. Breakpoint to set a temporary breakpoint at the highlighted line.

## Source Window Buttons

Use the Source buttons to do the following tasks:

·	Start emulation (go).

·	Halt emulation.

·	Step into a function.

·	Step over a function.

·	Go until/into call.

·	Go until/into return.

·	Go To Cursor.

### Go

Choose the Go button to start emulation.

### Halt

Choose the Halt button to stop emulation.

### Step Into

Choose the Step Into button to step into a function call.   Set the size of each step with the Source Step Granularity command from the Options menu.  Set the number of steps with the Step Count command from the Options menu.

NOTE

If you step into a function call, then the display steps into the source for that call and executes the first executable line in the procedure.

If you step over a function call, then the display treats that call as a single statement.

If the Source Only is toggled from the View menu, and a function has no source associated with it, then a step into works the same as a step over.  To step into a function with no associated source, choose View Mixed Source and Asm.

If stepping is slow, close windows to improve performance.  Each window must be updated after each step.  Performance can be affected by these updates.

### Step Over

Choose the Step Over button to step over a function.

### Until/Into Call

Toggle Until/Into with the Set Go Buttons command from the Options Menu.  You can break emulation before or after a Call instruction.

·	The Go Until Call button executes from the current CS:IP until the beginning of a statement or line (depending on the granularity) containing a call or a return instruction.

·	The Go Into Call button executes from the current CS:IP until the first executable statement inside a call.

NOTE

Set the level of step granularity by toggling the Step Execution Granu-larity command from the Options menu.

### Until/Into Return

Toggle Until/Into with the Set Go Buttons command from the Options menu.  You can break emulation before a Return instruction is executed.

·	The Go Until Return button stops emulation at the beginning of a statement or line (depending on the granularity) containing a Return instruction.

·	The Go Into Return command executes from the current CS:IP until the first executable statement or line (in the calling function) after a return instruction.

# Displaying and Editing Variables

## Introduction

This chapter describes how to use the Variable window to view and edit a variable.

## Opening the Variable Window

Open the Variable window by choosing Variable from the Windows menu.  To view a variable, choose Add from the Variable menu, or double-click on a variable name in the Source window.

The following example shows one way to open the Variable window from the Source window.

1.	Load a file.

2.	Choose Set Breakpoints from the Breakpoint menu.

3.	Set a breakpoint at main().

4.	Execute to the breakpoint.

5.	Double click on a variable name.

6.	This action brings up a pop-up menu, enabling you to set a breakpoint at the highlighted line or to inspect the variable value.  Choose Inspect Value to display the variable in the Variable window.

You can also inspect and edit a local variable from the Stack window.  For more information on the Stack window see the chapter "Viewing the Stack."

## Viewing and Modifying Variables

### Variable Window Display

View and edit variables from the Variable window.

To select a variable or its value, click on it.  Yellow is used to indicate that you selected the variable or its value.

Before you click on a variable, the name is displayed in one of the following colors:

·	Blue indicates a pointer variable which you can dereference by double clicking.

·	Magenta indicates standard variables.

### Adding Variables

To add a variable to the display

1.	Double click on its name in the Source window

2.	Choose Inspect Variable from the menu that pops up.

-or-

1.	Choose Add from the Variable menu in the Variable window.

You can add global, static, and local variables but local variables that are outside of the current stack context have the value "unknown" displayed in the Variable Window.

#### Specifying Symbols

You can specify a symbol name in a fully-qualified form or specify the variable name.  A fully qualified variable is in the form [#module][#function]var, where var is in the form
#basename[(®ext) | (.ext)]*.  The following are examples of variables:

#mod1#a.b®c

#mod2#func1#a®d®e

#globalvar

### Deleting Variables

To delete a variable from the display, click on its name and then choose Delete from the Variable menu, or press the Delete key.  Undelete the variable by choosing Undelete from the Variable menu.

### Editing Variables

Edit a highlighted value by pointing to and double-clicking the value, or by clicking the value and choosing Edit from the Variable menu.  Press Enter to terminate an edit. Integer variables can be edited in hexadecimal or decimal, floating point variables in floating point format, and characters in their hexadecimal ASCII equivalent.  The Variable window outputs the enumerated name after the hexadecimal value for variables that are defined as enum types.  For example:

enum color c = 2 = lavender

### Dereferencing Pointers

Blue indicates a pointer.  The following is an example of a displayed pointer:

CELL_TYPE *printall#cellPtr = ds:000e

Where ds:000e is the address of the variable pointed to by the pointer (cellPtr).

To dereference the pointer, double click on its name or choose Show from the View menu.  When you dereference the pointer, a new entry is created (in the Variable window) showing the value (in red) of the variable that was pointed to.  The following is an example of a dereferenced pointer:

CELL_TYPE printall#*cellPtr{

struct LINKS *next = ds:0014;

char *StringPtr = ds:0000;

short int length = 2 = 2;

}

# Variable Window Reference

## Introduction

This chapter describes the menus and commands in the Variable window.

## Variable Menus

| Menu | Use To: |
| --- | --- |
| File | Exit the Variable window. |
| Edit | Search for, search for next, and edit a variable. |
| View | Compress, show, or sort variables by name or by the order they were entered in, or refresh the display. |
| Variable | Add, delete, or undelete Variables from the window display. |
| Windows | Open another window. For more information, see the chapter "Getting Started." |
| Help | Get help on commands. For more information, see the chapter "Getting Started." |

### File Menu

Exit the Variable window from the File menu.

#### Exit

Choose Exit to close the Variable window.

### Edit Menu

Search for a variable or edit a variable's value from the Edit menu.

#### Search

Choose Search and Search Next from the Edit menu to find a variable name.  Search finds a variable by the name specified in the Search dialog box (the search is case sensitive).

#### Search Next

Choose Search and Search Next from the Edit menu to find a variable name.  Search Next finds the next instance of the name specified using the Search dialog box (the search is case sensitive).

#### Edit

Click on a value field of a variable, then choose Edit from the Edit menu to change it.  Integers can be edited in decimal or hexadecimal radix.  Floating-point numbers can be edited in floating point format.  Characters can be edited in hexadecimal or ascii.

### View Menu

Change the Variable window display from the View menu.  You can expand or contract the display, or sort the variables in the display alphabetically or in the order they were added to the display, or refresh the display.

#### Show

Click on a variable, then choose Show to dereference the variable (such as a pointer).

#### Compress

Choose the Compress toggle to show only the first line of each variable.

#### Refresh Display

Choose Refresh Display to update the display.

#### Sort

Choose Sort to sort the variables display alphabetically or by ascending history.

·	Choose By History to sort the variables in the order they were added to the variables display.

·	Choose By Variable Name to sort the display alphabetically.

### Variable Menu

Add variables to, or delete variables from, the display using the Variable menu.

#### Add

Choose Add to add a variable to the window.

#### Delete

Click on a variable in the Variable window, then choose Delete to delete it from the window.

#### Undelete

Choose Undelete to undelete the last variable deleted from the display in the Variable window.