# # $ K Register cannot be set to an odd value.

This error message is mismatched.  Contact Microtekcontactingmicrotek

# # $ K Value out of range.

Cause:	The register value is too large for the size of the register.

Solution:	Use a value within range.

# # $ K CPU configuration file missing.

Cause:	The CPU configuration file is not in MICEpack/SLD default directory (e.g. c:\mp186).

Solution:	Reinstall MICEpack/SLD.

# # $ K Processor configuration file format error.

Cause:	The CPU configuration file is corrupted.

Solution:	Reinstall MICEpack/SLD

# # $ K Invalid command line arguments.

Cause:	Syntax error.

Solution:	Look for the syntax description for the command you are using.

See also

CPU Windowcpu

# # $ K Invalid address text string.

Cause:	The numeric address is invalid.

Solution:	Enter a number (0xdddd0000) or a symbol (#main).

See also

Symbol Entry Syntaxsymbolentrysyntax

Addressesaddressesintel

# # $ K Address is larger than the target processor's maximum address.

Cause:	The maximum input address specified is too large for the processor.

Solution:	See the Limit Checking Tablelimitcheck

# # $ K All available breakpoints are assigned.

Cause:	Please contact Microtek to report it.

Solution:	Please report this problem to Microtekcontactingmicrotek.

See also

Breakpoint Windowbreakpoint

# # $ K No breakpoint matched the specified ID.

Cause:	The identification number is not active.

Solution:	Display the current breakpoints with the bkpt Shell commandbkptshell for a list of active IDs.

See also

Breakpoint Windowbreakpoint

# # $ K Exceeded the maximum breakpoint ID value.

Cause:	The breakpoint identification number is too big.

Solution:	The value is 0 to 0xfffe.

See also

Breakpoint Windowbreakpoint

# # $ K The emulator is halted.

Cause:	Informational message indicating emulator is halted.

Solution:	Informational message only.

# # $ K Breakpoint address not on an instruction boundary; breakpoint not set.

Cause:	The address specified does not line up with an instruction boundary.

Solution:	Use the dasm Shell commanddasmshell to disassembly memory and find the correct boundary.

See also

Breakpoint Windowbreakpoint

# # $ K Step count cannot be zero.

Cause:	Step count of zero has no meaning.

Solution:	Specify a step count of 0 to 0xffffffff.

See also

Breakpoint Windowbreakpoint

# # $ K No breakpoint matches the specified address.

Cause:	The address does not match a breakpoint.

Solution:	The breakpoint is not deleted.  Use the bkpt Shell commandbkptshell to display breakpoints with their addresses.

See also

Breakpoint Windowbreakpoint

# # $ K The emulator must be halted before performing this operation.

Cause:	Access to this feature requires the emulator to be halted.

Solution:	Halt emulationemulationterm or read and write memory while emulating using run accessrunaccessterm.

# # $ K Out of memory.

Cause:	No more memory available from Windows.

Solution:	Close other applications or MICEpack/SLD windows to free memory.

# # $ K Invalid instruction.

Cause:	Instruction mnemonic is not valid.

Solution:	Check assembly syntax.

See also

Recommended Readingrecommendedreading

# # $ K Incorrect number of operands for instruction.

Cause:	Number of operands does not match instruction mnemonic.

Solution:	Check assembly syntax.

See also

Recommended Readingrecommendedreading

# # $ K Invalid mode for instruction.

Cause:	The operand syntax is correct, but the mode is not valid for this instruction.

Solution:	Check assembly syntax.

See also

Recommended Readingrecommendedreading

# # $ K Immediate value incorrectly given.

Cause:	Immediate data operands start with a #.

Solution:	Add a # for immediate data (#1234, #0x4444).

See also

Recommended Readingrecommendedreading

# # $ K Symbol name given but symbols disabled.

Cause:	Symbol parsing not enabled.  Please report it to Microtek.

Solution:	Use dasmSym Shell commanddasmsymshell to enable symbols.

See also

Please report this problem to Microtekcontactingmicrotek.

Recommended Readingrecommendedreading

# # $ K Illegal bit for pshm/pulm instructions.

This error message is mismatched.  Contact Microtekcontactingmicrotek

# # $ K Masked instruction format wrong.

This error message is mismatched.  Contact Microtekcontactingmicrotek

# # $ K Illegal index register.

This error message is mismatched.  Contact Microtekcontactingmicrotek

# # $ K Bad displacement.

This error message is mismatched.  Contact Microtekcontactingmicrotek

# # $ K Invalid opcode.

Cause:	Instruction mnemonic is invalid.

Solution:	Check assembly syntax.

See also

Recommended Readingrecommendedreading

# # $ K At top of memory.

Cause:	Cannot disassembly past end of memory.

Solution:	Set the address for disassembly.

# # $ K Invalid instruction address.

Cause:	The address is greater than the maximum for the processor.

Solution:	See Addressesaddressesintel and Limit Checking Tablelimitcheck

# # $ K Instruction mnemonic missing or invalid.

Cause:	The instruction mnemonic is wrong.

Solution:	Check assembly syntax.

See also

Recommended Readingrecommendedreading

# # $ K No queue flushes found in trace buffer.  Cannot view instructions.

This error message is mismatched.  Contact Microtekcontactingmicrotek

# # $ K Assembly syntax error.

Cause:	The syntax is incorrect.

Solution:	Check assembly syntax.

See also

Recommended Readingrecommendedreading

# # $ K Invalid displacement.

Cause:	The displacement is out of range.

Solution:	Check the negative or positive byte or word displacement.

See also

Recommended Readingrecommendedreading

# # $ K Invalid operand.

Cause:	The operand is invalid.

Solution:	Check the assembly syntax.

See also

Recommended Readingrecommendedreading

# # $ K Invalid register.

Cause:	The register name is incorrect.

Solution:	Use a valid register name, such as AX, BX, CX, DX, SI, DI, IP, CS, DS, ES, SS.

See also

Recommended Readingrecommendedreading

# # $ K Invalid effective address.

Cause:	The operand or operand combination is invalid.

Solution:	Check the operands for validity with instruction.

See also

Recommended Readingrecommendedreading

# # $ K Map block too large.

Cause:	The map block specified is too large.

Solution:	Check on the availability of target or overlay ram.

See also

Mapping Memorymappingmemory

# # $ K Address range already mapped.

Cause:	The map block specified is already covered by mapped memory.

Solution:	Use the ToolBar'stoolbar map button or the map Shell commandmapshell to get the current map.  Change request so it does not overlap existing mapped memory.

See also

Mapping Memorymappingmemory

# # $ K There are no map blocks available.

Cause:	Not enough mapped memory blocks exist to satisfy request.

Solution:	Use the ToolBar'stoolbar map button or the map Shell commandmapshell to get the current map.  There are two map blocks, each half of the total mapped memory size.

See also

Mapping Memorymappingmemory

# # $ K No map ram installed on probe.

Cause:	No mapped memory is installed, so map request is not possible.

Solution:	Contact Microtekcontactingmicrotek to discuss mapped memory options.

See also

Mapping Memorymappingmemory

# # $ K Cannot open file.

Cause:	Cannot open file.

Solution:	For saveCS, the file cannot be written.  You must specify the .cs file extension.  For example, saveCS config1.cs.  For restoreCS, the file does not exist.  Check the name and path.

# # $ K Too many input lines.

Cause:	The chip select file has too many lines in it.

Solution:	No chip select registers restored.  The file was incorrectly edited or is corrupted.  Edit file or re-create file.

# # $ K Register value too large.

This error message is mismatched.  Contact Microtekcontactingmicrotek

# # $ K Invalid register name.

This error message is mismatched.  Contact Microtekcontactingmicrotek

# # $ K Invalid register value.

This error message is mismatched.  Contact Microtekcontactingmicrotek

# # $ K Chip select value not found.

This error message is mismatched.  Contact Microtekcontactingmicrotek

# # $ K Map block cannot be mapped.

Cause:	All memory map blocks currently in use, or the map block is not unique.

Solution:	Use the ToolBar'stoolbar map button or the map Shell commandmapshell to get the current map.

See also

Mapping Memorymappingmemory

# # $ K Data pattern too long.

Cause:	The number of data entries is too great.

Solution:	The fill, search, and write pattern is 1 to 256 entries.

See also

Fill Shell Commandfillshell

Search Shell Commandsearchshell

Write Shell Commandwriteshell

# # $ K The end address is less than the start address.

Cause:	The end address cannot be less than the start address.

Solution:	Check the addresses.  Memory is filled in ascending memory order.

See also

Fill Shell Commandfillshell

# # $ K Data values must be less than 256.

Cause:	Data must be a byte stream.

Solution:	Use byte data, or specify a different access size (word, long, or dword).

See also

Fill Shell Commandfillshell

Search Shell Commandsearchshell

Write Shell Commandwriteshell

# # $ K Stack base unknown

Cause:	The stack base was never set.

Solution:	Use the Stack Window'sstack Options/Stack Area or the setStackArea Shell commandsetstackareashell to set the base.

# # $ K Stack area too large.

Cause:	The stack base plus the stack size exceeds the processor's memory space.

Solution:	Use the Stack Window'sstack Options/Stack Area or the setStackArea Shell commandsetstackareashell to set the base.

# # $ K Stack area too small.

Cause:	The stack base plus the stack size exceeds the processor's memory space.

Solution:	Use the Stack Window'sstack Options/Stack Area or the setStackArea Shell commandsetstackareashell to set the base.

# # $ K Invalid high water mark.

Cause:	The high water mark is not inside the stack area.

Solution:	Use Stack Window'sstack Options/Stack Area or the setStackArea Shell commandsetstackareashell to view the stack area.

# # $ K Invalid stack size.

Cause:	The stack size contains non-numeric data.

Solution:	Check the setStackArea Shell commandsetstackareashell syntax.

# # $ K Invalid alarm limit.

Cause:	The alarm limit is not inside the stack area.

Solution:	Use the Stack Window'sstack Options/Stack Area or the setStackArea Shell commandsetstackareashell to view the stack area.

# # $ K Out of memory.

Cause:	No more memory available from Windows.

Solution:	Close other applications or MICEpack/SLD windows to free memory.

# # $ K Cannot find symbol.

Cause:	The symbol is not in the table.

Solution:	Check that the ToolBar Window'stoolbar Load button Options.../Load Symbols or Source Window'ssource File/Load Code... Options.../Load Symbols is checked.  Use the displaySymbols Shell commanddisplaysymbolsshell to check the name of symbols.

# # $ K Cannot find line number.

Cause:	The line number given is greater than the number of lines in the file.

Solution:	Use the Source Window'ssource View/Line Numbers to check line numbers in source.

# # $ K Symbols already loaded.

Cause:	Symbols from a load file are currently loaded.

Solution:	Symbols from one load file supported.  Delete symbols when queried or use the removeSymbols Shell commandremovesymbolsshell.

See also

ToolBar Windowtoolbar

Source Windowsource

# # $ K The load file time stamp does not match previously loaded file's time stamp.

Cause:	Modifications to the load file were made.

Solution:	A source file changed after the symbol table was generated.

See also

Source Windowsource

# # $ K The current execution context does not match any known module.

Cause:	The program counter is outside of any module with symbolic information.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell displays module information.

See also

Source Windowsource

# # $ K Symbol is a local variable, thus it does not have a static address.

Cause:	The symbol is stack resident and has a different address each time its function is invoked.

Solution:	Only static symbols have permanent addresses.

See also

Source Windowsource

# # $ K No symbol information available about source modules.

Cause:	The line number information does not correspond to a source module.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell displays module and line number information.

See also

Source Windowsource

# # $ K Variable not found.

Cause:	The variable is not loaded.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell displays variable information.

See also

Variable Windowvariable

# # $ K Module name not found.

Cause:	There is no module by the name given.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell displays module information.

See also

Variable Windowvariable

# # $ K Function name not found.

Cause:	There is no function by the name given.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell displays module and function information.

See also

Variable Windowvariable

# # $ K Variable name not found.

Cause:	There is no variable by the name given.

Solution:	The displaySymbols Shell commanddisplaysymbolsshell displays module, function, and variable information.

See also

Variable Windowvariable

# # $ K Invalid value: unsigned short int value must be 0 to 65535.

Cause:	The value is outside the range of the variable.

Solution:	The variable is an unsigned short integer and the value must be from 0 to 0xffff (65535).

See also

Variable Windowvariable

# # $ K Invalid value: unsigned long int value must be 0 to 4294967295.

Cause:	The value is outside the range of the variable.

Solution:	The variable is an unsigned integer and the value must be from 0 to 0xffffffff (4,294,967,295).

See also

Variable Windowvariable

# # $ K Invalid value: signed short int value must be -32768 to 32767.

Cause:	The value is outside the range of the variable.

Solution:	The variable is a short integer and the value must be from -32768 to 32767 (0x7fff).

See also

Variable Windowvariable

# # $ K Invalid value: signed long int value must be -2147483648 to 2147483647.

Cause:	The value is outside the range of the variable.

Solution:	The variable is a long integer and the value must be from -2,147,483,648 to 2,147,483,647 (0xffffffff).

See also

Variable Windowvariable

# # $ K Comm port hardware not present.

Cause:	The port specified is not there.

Solution:	The wrong port number was specified or a hardware problem exists with the serial board.

See also

Can't Establish Communicationscantestablishcommunications

# # $ K Device already open.

Cause:	The communications port is already open.

Solution:	Another application is using the comm port.  Close the other application.  If this is not the problem, exit Windows and re-boot the PC to re-initialize the comm port hardware.

# # $ K No active event has been selected.

Cause:	An event must be selected before this action.

Solution:	Set the Event Window'sevent Active Event and then execute action again.

# # $ K Invalid number.

Cause:	The syntax of the number is incorrect.

Solution:	Enter a number (1234, 0x4444, 0177).

See also

Event Windowevent

# # $ K The new event name is too long.

Cause:	The name is more than 63 characters.

Solution:	Event names of 1 to 63 characters are supported.

See also

Event Windowevent

# # $ K Cannot create any more events.

Cause:	64 events can be active at one time.

Solution:	Use the Event Window'sevent File/Save Events As... and File/Restore Events... to save various configurations of events to expand the effective number of events.

# # $ K Cannot open file.

Cause:	The file for saving or restoring events cannot be opened.

Solution:	If saving, the number of open files limit is exceeded.  Close other Windows applications using files or increase BUFFERS= in config.sys.

If restoring, the file name or path is incorrect.

See also

Event Windowevent

# # $ K Cannot write file.

Cause:	A write failed while saving the events.

Solution:	The hard disk drive is full.

See also

Event Windowevent

# # $ K Cannot read file.

Cause:	A read failed when attempting to restore an event file with the wrong format.

Solution:	Check the filename.  If the filename is correct, the format may be bad, or the file may have been corrupted or truncated.

See also

Event Windowevent

# # $ K File name required.

Cause:	The file name is blank.

Solution:	Enter a DOS file name.

See also

Event Windowevent

# # $ K Cannot close file.

Cause:	The file for saving or restoring events cannot be closed.

Solution:	A DOS error has occurred.  Check the file and try again.

See also

Event Windowevent

# # $ K Cannot change an event used by trigger while trace is on.

Cause:	Events currently being used cannot be modified.

Solution:	Turn tracing off or stop emulation using the ToolBartoolbar.

See also

Event Windowevent

# # $ K Cannot change an event used by trigger while emulating.

Cause:	Events currently being used cannot be modified.

Solution:	Stop emulation using the ToolBartoolbar Halt button, the Source Window'ssource Halt button or Run/Halt menu option, or the halt Shell commandhaltshell.

See also

Event Windowevent

# # $ K Cannot calculate end offset without a start.

Cause:	The start address must be defined before the length.

Solution:	Fill in the start address, then the length.

See also

Event Windowevent

# # $ K Cannot convert text to a value.

Cause:	The address or number is invalid.

Solution:	Enter a symbol or a number (1234, 0x4444, 0177).

See also

Symbol Entry Syntaxsymbolentrysyntax

Event Windowevent

# # $ K Cannot open configuration file.

Cause:	The trace configuration file is not in MICEpack/SLD default directory (e.g. c:\mp186).

Solution:	An event configuration file (evXXX.cfg) is missing.  Re-install the MICEpack/SLD software into a new directory and copy these files to the existing directory.  Or, use the new MICEpack/SLD installation.

See also

Event Windowevent

# # $ K Unexpected end of file.

Cause:	The trace configuration file is corrupted in MICEpack/SLD default directory (e.g. c:\mp186).

Solution:	An event configuration file (evXXX.cfg) has been truncated.  Re-install the MICEpack/SLD software into a new directory and copy these files to the existing directory.  Or, use the new MICEpack/SLD installation.

See also

Event Windowevent

# # $ K Cannot open event configuration file

Cause:	The trace configuration file is not in MICEpack/SLD default directory (e.g. c:\mp186).

Solution:	An event configuration file (evXXX.cfg) is missing.  Re-install the MICEpack/SLD software into a new directory and copy these files to the existing directory.  Or, use the new MICEpack/SLD installation.

See also

Event Windowevent

# # $ K Counter value too large; setting to maximum value.

Cause:	The counter value range is 1 to 1023 (0x3ff).

Solution:	The counter was set to the maximum value of 1023.

See also

Using Counters and Timersusingcountersand

Trigger Windowtrigger

# # $ K Invalid event name.

Cause:	The event specified in the trigger is not defined.

Solution:	Check the events that are active in the Trigger Windowtrigger for one that is not defined.

See also

Event Windowevent

# # $ K Cannot change trigger while tracing and/or sequencer is enabled.

Cause:	Triggers cannot be modified while tracing or the sequencer is enabled.

Solution:	Turn tracing off or stop emulation then change the trigger setting.

See also

Trigger Windowtrigger

# # $ K A NEXT action requires multiple trace buffer configuration.

Cause:	The NEXT action starts collection in the next trace buffer.  Only 1 trace buffer is configured.

Solution:	Use the Trigger Window'strigger Options/Trace Control... to set the number of buffers.

See also

Selecting Trigger Actionsselectingtriggeractions

# # $ K Counter value too small; setting to minimum value.

Cause:	The counter value range is 1 to 1023 (0x3ff).

Solution:	The counter was set to the minimum value of 1.

See also

Using Counters and Timersusingcountersand

Trigger Windowtrigger

# # $ K Cannot open load file.

Cause:	The file does not exist.

Solution:	Check file name and path.

# # $ K Cannot add symbol to symbol table: <name>.

This error message is mismatched.  Contact Microtekcontactingmicrotek

.

# # $ K Cannot add function symbol to symbol table; local symbols will be lost: <function name>.

Cause:	The symbol table cannot be expanded.

Solution:	Window's has run out of memory.  Close other applications or other MICEpack/SLD windows.

# # $ K Cannot add module to symbol table: <module name>.

Cause:	The symbol table cannot be expanded.

Solution:	Windows has run out of memory.  Close other applications or other MICEpack/SLD windows.

# # $ K You must specify a start value in order to have a non-zero mask value.

Cause:	You entered a non-zero address mask value and you have not entered an address start value.

-or-

You entered a non-zero data mask value and you have not entered a data start value.

Solution:	Enter a start value before entering a mask value.

# # $ K The address plus the length exceeds the address limit.

Cause:	This error occurs when the start address value plus the length is greater than the maximum address allowed.  For example, if the current processor has 26 bits of address, then the maximum address allowed is 0x3FFFFFF.  If the user enters a start address of 0x3FFFFF0 and a length of 0x10, this is an error (0x3FFFFF0 + 0x10 = 0x4000000 which is greater than 0x3FFFFFF).

Solution:	Enter a smaller start value or a smaller length value.

# # $ K The start data value entered exceeds the data value limit.

Cause:	This error occurs when the start data value is greater than the maximum data value allowed.  For example, if the current processor has 16 bits of data, then the maximum data value allowed is 0xFFFF hex or 65535 decimal.  If the user enters a start data value greater than of 0xFFFF (65535), this is an error.

Solution:	Enter a smaller start data value.

# # $ K The end data value entered exceeds the data value limit.

Cause:	This error occurs when the end data value is greater than the maximum data value allowed.  For example, if the current processor has 16 bits of data, then the maximum data value allowed is 0xFFFF hex or 65535 decimal.  If the user enters an end data value greater than 0xFFFF (65535), this is an error.

Solution:	Enter a smaller end data value.

# # $ K Invalid object for the requested operation.

Cause:	Internal error.

Solution:  Contact Microtekcontactingmicrotek

# # $ K Object list is empty.

Cause:	Internal error.

Solution:  Contact Microtekcontactingmicrotek

# # $ K Cannot get file filter string.

Cause:	Internal error.

Solution:  Contact Microtekcontactingmicrotek

# # $ K Cannot find the owner of the current dialog box.

Cause:	Internal error.

Solution:  Contact Microtekcontactingmicrotek

# # $ K Missing help context ID.

Cause:	Internal error.

Solution:  Contact Microtekcontactingmicrotek

# # $ K Windows COMDLG File dialog error: %s.

Cause:	Internal error.

Solution:  Contact Microtekcontactingmicrotek

# # $ K + Illegal Pointer Edit

Cause:	Attempt to edit a pointer in linear or physical address format.

Solution:	Use virtual address format.

# HE_CPU_CANT_BE_ODD

$ Register cannot be set to an odd value.

K error, o-t

# HE_REG_OUT_OF_RANGE

$ Value out of range.

K error, u-z

# HE_NO_CPU_CONFIG_FILE

$ CPU configuration file missing.

K error, a-g

# HE_CPU_CONFIG_FILE_FORMAT

$ Processor configuration file format error.

K error, o-t

# HE_INVALID_ARG

$ Invalid command line arguments.

K error, h-n

# HE_ADR_INVALID_ADDRESS_STR

$ Invalid address text string.

K error, h-n

# HE_ADR_ADDRESS_TOO_LARGE

$ Address is larger than the target processor's maximum address.

K error, a-g

# HE_BX_NO_BKPT_AVAILABLE

$ All available breakpoints are assigned.

K error, a-g

# HE_BX_INACTIVE_BKPT

$ No breakpoint matched the specified ID.

K error, h-n

# HE_BX_ID_TOO_BIG

$ Exceeded the maximum breakpoint ID value.

K error, a-g

# HE_BX_EMU_IS_HALTED

$ The emulator is halted.

K error, o-t

# HE_BX_NOT_ON_INST_BOUNDARY

$ Breakpoint address not on an instruction boundary; breakpoint not set.

K error, a-g

# HE_BX_INVALID_COUNT

$ Step count cannot be zero.

K error, o-t

# HE_BX_NO_BKPT_MATCH_ADDR

$ No breakpoint matches the specified address.

K error, h-n

# HE_BK_EMULATOR_MUST_BE_HALTED

$ The emulator must be halted before performing this operation.

K error, o-t

# HE_HEAP_OUT_OF_MEMORY

$ Out of memory.

K error, o-t;out of memory;memory

# HE_DAD_NO_SUCH_INST

$ Invalid instruction.

K error, h-n

# HE_DAD_INVALID_NUM_OPERANDS

$ Incorrect number of operands for instruction.

K error, h-n

# HE_DAD_INVALID_MODE_FOR_INST

$ Invalid mode for instruction.

K error, h-n

# HE_DAD_IMMED_FIELD_INCORRECT

$ Immediate value incorrectly given.

K error, h-n

# HE_DAD_SYMBOL_USE_BUT_DISABLED

$ Symbol name given but symbols disabled.

K error, o-t

# HE_DAD_INVALID_REGLIST_BIT

$ Illegal bit for pshm/pulm instructions.

K error, h-n

# HE_DAD_BAD_MASKED_INST_FORMAT

$ Masked instruction format wrong.

K error, h-n

# HE_DAD_INVALID_INDEX_REG

$ Illegal index register.

K error, h-n

# HE_DAD_INCORRECT_DISP

$ Bad displacement.

K error, a-g

# HE_DAD_INVALID_OPCODE

$ Invalid opcode.

K error, h-n

# HE_DAD_AT_TOP_OF_MEMORY

$ At top of memory.

K error, a-g

# HE_DAD_INVALID_INST_ADDR

$ Invalid instruction address.

K error, h-n

# HE_DAD_INST_NOT_FOUND

$ Instruction mnemonic missing or invalid.

K error, h-n

# HE_DAD_NO_FLUSH_FRAMES_FOUND

$ No queue flushes found in trace buffer.  Cannot view instructions.

K error, h-n

# HE_DAD_ASM_ERROR

$ Assembly syntax error.

K error, a-g

# HE_DAD_DISPLACEMENT_INVALID

$ Invalid displacement.

K error, h-n

# HE_DAD_OPERAND_INVALID

$ Invalid operand.

K error, h-n

# HE_DAD_REG_INVALID

$ Invalid register.

K error, h-n

# HE_DAD_EA_INVALID

$ Invalid effective address.

K error, h-n

# HE_BLOCK_TOO_LARGE

$ Map block too large.

K error, h-n

# HE_BLOCK_ALREADY_MAPPED

$ Address range already mapped.

K error, a-g

# HE_NO_FREE_MAP_BLOCKS

$ There are no map blocks available.

K error, o-t

# HE_NO_MEMORY_PRESENT

$ No map ram installed on probe.

K error, h-n

# HE_MAP_CANNOT_OPEN_FILE

$ Cannot open file.

K error, a-g

# HE_MAP_TOO_MANY_INPUT_LINES

$ Too many input lines.

K error, o-t

# HE_MAP_VALUE_TOO_BIG

$ Register value too large.

K error, o-t

# HE_MAP_INVALID_REGNAME

$ Invalid register name.

K error, h-n

# HE_MAP_INVALID_VALUE_INPUT

$ Invalid register value.

K error, h-n

# HE_MAP_VALUE_MISSING

$ Chip select value not found.

K error, a-g

# HE_BLOCK_UNMAPPABLE

$ Map block cannot be mapped.

K error, h-n

# HE_MEM_PAT_LEN

$ Data pattern too long.

K error, a-g

# HE_MEM_END_LESS_THAN_START

$ The end address is less than the start address.

K error, o-t

# HE_MEM_DATA_GREATER_THAN_BYTE

$ Data values must be less than 256.

K error, a-g

# HE_STK_STK_BASE_INVALID

$ Stack base unknown

K error, o-t

# HE_STK_RANGE_TOO_LARGE

$ Stack area too large.

K error, o-t

# HE_STK_RANGE_TOO_SMALL

$ Stack area too small.

K error, o-t

# HE_STK_INVALID_HWM

$ Invalid high water mark.

K error, h-n

# HE_STK_INVALID_STK_SIZE

$ Invalid stack size.

K error, h-n

# HE_STK_INVALID_ALARM_LIMIT

$ Invalid alarm limit.

K error, h-n

# HE_SYM_OUT_OF_MEMORY

$ Out of memory.

K error, o-t

# HE_SYMBOL_NOT_FOUND

$ Cannot find symbol.

K error, a-g

# HE_LINENUM_NOT_FOUND

$ Cannot find line number.

K error, a-g

# HE_SYMBOLS_LOADED

$ Symbols already loaded.

K error, o-t

# HE_TIMESTAMPS_DONT_MATCH

$ The load file time stamp does not match previously loaded file's time stamp.

K error, o-t

# HE_SYM_NO_CURRENT_CONTEXT

$ The current execution context does not match any known module.

K error, o-t

# HE_SYM_LOCAL_VAR_ADDRESS

$ Symbol is a local variable, thus it does not have a static address.

K error, o-t

# HE_SYM_INVALID_LN_END_ADDR

$ No symbol information available about source modules.

K error, h-n

# HE_VS_VAR_NOT_FOUND

$ Variable not found.

K error, u-z

# HE_VS_MODULENAME_NOT_FOUND

$ Module name not found.

K error, h-n

# HE_VS_FUNCNAME_NOT_FOUND

$ Function name not found.

K error, a-g

# HE_VS_VARNAME_NOT_FOUND

$ Variable name not found.

K error, u-z

# HE_VS_USHORT_OVERFLOW

$ Invalid value: unsigned short int value must be 0 to 65535.

K error, h-n

# HE_VS_ULONG_OVERFLOW

$ Invalid value: unsigned long int value must be 0 to 4294967295.

K error, h-n

# HE_VS_SHORT_OVERFLOW

$ Invalid value: signed short int value must be -32768 to 32767.

K error, h-n

# HE_VS_LONG_OVERFLOW

$ Invalid value: signed long int value must be -2147483648 to 2147483647.

K error, h-n

# HE_HARDWARE

$ Comm port hardware not present.

K error, a-g

# HE_OPEN

$ Device already open.

K error, a-g

# HE_NO_ACTIVE_EVENT

$ No active event has been selected.

K error, h-n

# HE_INVALID_DIGIT

$ Invalid number.

K error, h-n

# HE_NAME_TOO_LONG

$ The new event name is too long.

K error, o-t

# HE_TOO_MANY_EVENTS

$ Cannot create any more events.

K error, a-g

# HE_CANT_OPEN_FILE

$ Cannot open file.

K error, a-g

# HE_WRITING_FILE

$ Cannot write file.

K error, a-g

# HE_READING_FILE

$ Cannot read file.

K error, a-g

# HE_NO_FILE_GIVEN

$ File name required.

K error, a-g

# HE_CANT_CLOSE_FILE

$ Cannot close file.

K error, a-g

# HE_TRACING_ON

$ Cannot change an event used by trigger while trace is on.

K error, a-g

# HE_TRIGGER_USING_EVENT

$ Cannot change an event used by trigger while emulating.

K error, a-g

# HE_DEFINE_START_FIRST

$ Cannot calculate end offset without a start.

K error, a-g

# HE_CANT_CONV_TEXT

$ Cannot convert text to a value.

K error, a-g

# HE_CANT_OPEN_TEMPLATE_FILE

$ Cannot open configuration file.

K error, a-g

# HE_UNEXPECTED_END

$ Unexpected end of file.

K error, u-z

# HE_NO_EVFILE_ENTRY

$ Cannot open event configuration file

K error, a-g

# HE_MAX_COUNTER_VALUE

$ Counter value too large; setting to maximum value.

K error, a-g

# HE_INVALID_EVENT_NAME

$ Invalid event name.

K error, h-n

# HE_TRACE_TRIG_ON

$ Cannot change trigger while tracing and/or sequencer is enabled.

K error, a-g

# HE_MULTI_BUFS_NEEDED

$ A NEXT action requires multiple trace buffer configuration.

K error, a-g

# HE_MIN_COUNTER_VALUE

$ Counter value too small; setting to minimum value.

K error, a-g

# HE_CANNOT_OPEN

$ Cannot open load file.

K error, a-g

# HE_CANNOT_ADD_SYM

$ Cannot add symbol to symbol table: <name>.

K error, a-g

# HE_CANNOT_ADD_FUNC

$ Cannot add function symbol to symbol table; local symbols will be lost: <function name>.

K error, a-g

# HE_CANNOT_ADD_MOD

$ Cannot add module to symbol table: <module name>.

K error, a-g

# HE_NO_START_VALUE_DEFINED

$ You must specify a start value in order to have a non-zero mask value.

K error, u-z

# HE_END_LENGTH_VALUE_TOO_BIG

$ The address plus the length exceeds the address limit.

K error, o-t

# HE_START_DATA_VALUE_TOO_BIG

$ The start data value entered exceeds the data value limit.

K error, o-t

# HE_END_DATA_VALUE_TOO_BIG

$ The end data value entered exceeds the data value limit.

K error, o-t

# HE_COMMDLG_INVALID_OBJECT

$ Invalid object for the requested operation.

K error, h-n

# HE_COMMDLG_OBJECT_LIST_EMPTY

$ Object list is empty.

K error, o-t

# HE_COMMDLG_GET_FILTER_STRING

$ Cannot get file filter string.

K error, a-g

# HE_COMMDLG_CANNOT_FIND_OWNER

$ Cannot find the owner of the current dialog box.

K error, a-g

# HE_COMMDLG_MISSING_HELP_ID

$ Missing help context ID.

K error, h-n

# HE_COMMDLG_FILE_DLG

$ Windows COMDLG File dialog error;%s.

K error, u-z

# Illegal_Pointer_Edit

$ Illegal Pointer Edit

K Illegal Pointer Edit

+ ERRORS2