# # $ K + Technical Notes

Assembly Language Supportassemblylanguagesupport

High-level Language Supportcandc

Comm Port Switchingcommportswitching

Function Keysfunctionkeys

Getting Startedgettingstarted

Improving MICEpack/SLD Performanceimprovingpowerviewsperformance

Initialization File (mp186.ini)initializationfile

Initialization Script (include.me)initializationscript

Powering Down the Targetpoweringdownthe

Sample Filessamplefiles

Script Filesscriptfiles

Solutions to Common Problemssolutionstocommon

Stopping Emulation When Invalid Stack Area Accessedstoppingemulationwhen

Trace and Trigger Fundamentalstraceandtriggerfundamentals

Triggering on Access to a C Variabletriggeringonaccess

# $ # + K COM Port Switching

The "comport=" line in MP186.INI defines the serial port MICEpack/SLD uses the next time you invoke MICEpack/SLD.  For example, the following defines COM2 as the port:

[Serial]

comport=com2

Use a text editor such as Notepad to change the comport.  COM1 and COM2 are supported.  Delete the line "comport=com2" line, reinvoke MICEpack/SLD, and MICEpack/SLD will prompt for a new COM port.

Note: Current MICEpack/SLD doesn’t COM port communication with MICEpack-186.

# $ # + K Improving MICEpack/SLD Performance

This can be divided into two areas: Windows tuning and MICEpack/SLD-specific tuning.

Windows tuning

Look for these items to increase performance:

1.	Have enough memory.  MICEpack/SLD runs faster on an 8 MB machine.

2.	Use SmartDrive or some other disk caching software.

3.	Use a  permanent swap fileswapfileterm.

4.	Use 32-bit access (see Windows Control Panel, 386 Enhanced).

For more information, see the Windows Resource Toolkit, published by Microsoft.

MICEpack/SLD tuning

Each open MICEpack/SLD window takes time to update.

Mixed source/assembly modesourcemixedand slows window updates.

# $ # + K Function Keys

F1	MICEpack/SLD Help, at Index

F2	Halt emulation

F5	Activate the toolbar

F6	Cycle to next window

F7	Step into a function

F8	Step over a function

F9	Emulate (Go)

F10	Activate the menu bar

# $ # + K Initialization File

When you install MICEpack/SLD, SETUP creates the MP186.INI file in your Windows directory. The file contains information that MICEpack/SLD uses when it starts and each time  a window is opened.

The following sections appear in MP186.INI:

[Serial]	COM port number.

[MicrotekParallel] MS-PCE card port base address.

[SystemInfo]    Target CPU model.

[InitScript]	Initial script to run on startup.

[Trace Info]	Trace Control and Trigger window options.

[TrigInfo]	Trigger Control and Trigger window options.

[SourceInfo]	Source window Go, Step, and View options.

[StackInfo]	Stack size, base address, and other Stack window options.

MICEpack/SLD edits entries in MP186.INI when completing an operation, with the following exceptions.   You must edit these sections manually:

1.	The  [InitScript]initializationscript section defines the initialization script.

2.	The   [Serial]commportswitching section defines the COM port used with MICEpack/SLD.

The manual describes the other sections in detail.

# $ # + K Powering Down the Target

Turn off the target system before turning off the emulator.

Power must be applied and removed in the correct sequence.   Failure to follow this sequence may result in severe damage to your target system or to the emulator.  The order of power sequencing is as follows:

1.	Apply power to the emulator.

2.	Apply power to the target system

3.	Remove power from the target system

4.	Remove power from the emulator.

# $ # + K Sample Files

\MP186\SAMPLE directory

DEMO.OMF

See also

Getting Startedgettingstarted

# $ # + K Stopping Emulation When Invalid Stack Area Accessed

1.	Check your stack base and stack size using the Stack Window's Options/Stack Area menu item or the StackInfo Shell Commandstackinfoshell.

2.	The last location in the stack equals the stack base minus the stack size.

3.	Define an event to trigger on writes to the address just below the last location in the stack.  Enter the address.  Select memory write status code.

4	Define a trigger on this event, selecting BRK to stop emulation.

# technicalnotes

$ Technical Notes

K answers;help;MICEpack/SLD;list

+ POWERVIEWS:0150

$ Comm Port Switching

# commportswitching

+ technical:0060

K communications;initialization file;mp186.ini

$ Improving MICEpack/SLD Performance

# improvingpowerviewsperformance

+ technical:0110

K MICEpack/SLD;performance;window

$ Function Keys

# functionkeys

+ technical:0090

K keys;function keys

$ Initialization File

# initializationfile

+ technical:0120

K initialization file;mp186.ini;files

$ Powering Down the Target

# poweringdownthe

+ technical:0155

K target

$ Sample Files

# samplefiles

+ technical:0170

K help;files

$ Stopping Emulation When Invalid Stack Area Accessed

# stoppingemulationwhen

+ technical:0235

K emulation;stack;trigger